| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Co-Located Coexistence Text Changes to MAC Control Messages |  |
| Date Submitted | 2026-07-31 |  |
| Source(s) | Jing Zhu Intel Corporation | +1-503-2647073 jing.z.zhu@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D3 for IEEE 802.16 WG Letter Ballot #30b |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Co-Located Coexistence Text Changes to MAC Control Messages

Jing Zhu

Intel Corporation

- Introduction

The contribution proposes the text changes to D3 [1]

- References

[1] IEEE P802.16m/D3

-------------------------------------------------- Begin Proposed Text Change 1-----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 53, page 40. New texts are underlined in blue font.>

Table 673 – MAC Control Messages (continued)

| Message Name | Message Description | Security | Connection |
| --- | --- | --- | --- |
| … | … | … | … |
| AAI_MultiBS_PMI_COM | Multi-BS PMI Combination |  | Unicast |
| AAI_CLC-REQ | Co-located coexistence request | Encrypted/ICV | Basic |
| AAI_CLC-RSP | Co-located coexistence response | Encrypted/ICV | Basic |

-------------------------------------------------- End Proposed Text Change 1-----------------------------------------------