| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Co-Located Coexistence Text Changes to AAI_RNG-REQ, AAI_SBC-REQ, AAI_SBC-RSP, and AAI_REG-RSP |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Jing Zhu Intel Corporation | +1-503-2647073 jing.z.zhu@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D3 for IEEE 802.16 WG Letter Ballot #30b |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Co-Located Coexistence Text Changes to AAI_RNG-REQ, AAI_SBC-REQ, AAI_SBC-RSP, and AAI_REG-RSP

Jing Zhu

Intel Corporation

- Introduction

The contribution proposes the text changes to D3 [1]

- References

[1] IEEE P802.16m/D3

-------------------------------------------------- Begin Proposed Text Change 1-----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 28, page 46. New texts are underlined in blue font.>

Table 674—Parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| CSGID(s) | One or more identifiers for one or more CSG(s). | Sent by AMS to aid quick CSG membership detection by Femto ABS as well as to aid ABS reselection |
| CLC-INFO | The information of co-located coexistence request (Table 690). | It may be sent by AMS to request reactivating CLC classes during network reentry after an uncoordinated handover. The Information Type field of CLC-INFO shall be set to 1 to indicate it provides CLC request information. |

-------------------------------------------------- End Proposed Text Change 1-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 2-----------------------------------------------

<Note to Editor: Insert the following new table in IEEE P802.16m/D3, starting on line 49, page 50. New texts are underlined in blue font.>

Table xxx—Parameters for AAI_ SBC-REQ

| Name | Value | Usage |
| --- | --- | --- |
| CLC-INFO | The information of co-located coexistence request (Table 690) | It may be sent by AMS to request activating CLC classes during network entry. The Information Type field of CLC-INFO shall be set to 1 to indicate it provides CLC request information. |

-------------------------------------------------- End Proposed Text Change 2-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 3-----------------------------------------------

<Note to Editor: Insert the following new table in IEEE P802.16m/D3, starting on line 25, page 51. New texts are underlined in blue font.>

Table yyy—Parameters for AAI_ SBC-RSP

| Name | Value | Usage |
| --- | --- | --- |
| CLC-INFO | The information of co-located coexistence response (Table 690) | It shall be included if the AMS includes CLC-INFO in AAI_SBC-REQ. The Information Type field of CLC-INFO shall be set to 2 to indicate it provides CLC response information. |

-------------------------------------------------- End Proposed Text Change 3-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 4-----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 35, page 59. New texts are underlined in blue font.>

Table 680 – Parameters for AAI_ REG-RSP

| Name |  | Value |
| --- | --- | --- |
| … | … | … |
| Host configuration capabilities and its parameters | Additional-Host-Configurations IE | Includes additional host configuration options in DHCP Options format as detailed in [3], [4] |
| Co-located coexistence limit parameters | CLC-INFO (Table 690) | It may be included if the CLC limit parameters are different from the default values in Table 759. The Information Type field of CLC-INFO shall be set to 0 to indicate it provides CLC limit information. |

-------------------------------------------------- End Proposed Text Change 4-----------------------------------------------