| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Text Changes to Co-Located Coexistence (16.2.3.18) |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Jing Zhu Intel Corporation | +1-503-2647073 jing.z.zhu@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D3 for IEEE 802.16 WG Letter Ballot #30b |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Text Changes to Co-Located Coexistence (16.2.3.18)

Jing Zhu

Intel Corporation

- Introduction

The contribution proposes the text changes to D3 [1]

- References

[1] IEEE P802.16m/D3

-------------------------------------------------- Begin Proposed Text Change 1-----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 49, page 75. Proposed deletion of text has been shown with red font that has been struck through. New text and a modified figure are underlined in blue font.>

Table 692 – CLC Request parameters

| Name | Size (bits) |
| --- | --- |
| CLC Start Time parameters | 8 |
| Scheduling Impact | 2 |
| Flag | 2 3 |
| Reserved | 4 3 |
| Start AAI subframe Index | 3 |
| CLC active interval of Type I CLC class | 8 |
| CLC active cycle of Type I CLC class | 21 |
| CLC active bitmap of Type II CLC class with subtype 1 | 8 |
| Start AAI subframe Index | 3 |
| CLC Active Interval active interval of Type II CLC class with subtype 2 | 8 |
| CLC Active active cycle of Type II CLC class with subtype 2 | 8 |
| Extended CLC Active Bitmap Indicator | 1 |
| CLC Active Bitmap | 7 |
| CLC active cycle Cycle of Type II CLC class with subtype 3 | 2 |
| Extended CLC Active Bitmap active bitmap of Type II CLC class with subtype 3 | variable |
| CLC Active Interval active interval of Type III CLC class | 8 |
| padding for byte alignment | variable |

CLC Start Time parameters

Its parameters is are listed in Table 694.

Scheduling Impact

0b00 (default) = both DL and UL allocations are prohibited in CLC Active Interval

0b01 = only DL allocations are prohibited in CLC Active Interval

0b10 = only UL allocations are prohibited in CLC Active Interval

0b11 = reserved

Flag

0b000: Type I CLC class;

0b001: Type II CLC class subtype 1;

0b010: Type II CLC class subtype 2;

0b011: Type II CLC class subtype 3;

0b110b100: Type III CLC class

Others: reserved

Start AAI subframe Index

This field is present if (Flag == 0b000) or (Flag == 0b010), and indicates the subframe index of the first subframe of the CLC active interval.

CLC active interval of Type I CLC class

This field is present if (Flag == 0b000), and indicates the number of AAI subframes of the CLC Active Interval active interval for Type I CLC class

CLC active cycle of Type I CLC class

This field is present if (Flag == 0b000), and indicates the number of microseconds of the CLC Active Cycle active cycle for Type I CLC class

Start AAI subframe Index

This field is present if (Flag == 0b01).Setting a bit of the field to "1" indicates the corresponding AAI subframe in each frame is in CLC active interval.

CLC Active Bitmap active bitmap of Type II CLC class with subtype 1

This field is present if (Flag == 0b001), and indicates the number of AAI subframes of the CLC Active Interval for Type II CLC class. Setting a bit of the field to “1” indicates the corresponding AAI subframe in each frame is in CLC active interval.

Extended CLC Active Bitmap Indicator

This field is present if (Flag == 0b10), and indicates whether the Extended CLC Active Bitmap field is used

CLC active interval of Type II CLC class with subtype 2

This field is present if (Flag == 0b010) if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 0), and indicates the number of AAI subframes of the CLC Active Interval for Type II CLC class with subtype 2.

CLC active cycle of Type II CLC class with subtype 2

This field is present if (Flag == 0b010)if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 0), and indicates the number of frames of the CLC Active Cycle for Type II CLC class with subtype 2

CLC Active Cycle active cycle of Type II CLC class with subtype 3

This field is present if (Flag == 0b011)if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 1), and indicates the length of the CLC active cycle in the unit of frame. If the value of this field is x, the length of Extended CLC Active Bitmap is calculated as x + 2. This field shall be set to one of the following values: 0, 1, and 2.

Extended CLC Active Bitmap active bitmap of Type II CLC class with subtype 3

This field is present if (Flag == 0b011)if (Flag == 0b10) and (Extended CLC Active Bitmap Indicator == 1). Setting a bit of the field to "1" indicates the corresponding AAI subframe in each CLC active cycle is in CLC active interval. If the CLC active cycle is x frames, and a frame consists of m AAI subframes, the length of this field shall be xm.

CLC Active Interval active interval of Type III CLC class

This field is present if (Flag == 0b100) if (Flag == 0b11), and indicates the number of superframes of the CLC Active Interval active interval for Type III CLC class

-------------------------------------------------- End Proposed Text Change 1-----------------------------------------------