| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Assignment A-MAP (16.3.6.2.2.4) for P802.16m/D3 |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation - Hyunkyu Yu - Samsung Electronics | Lei.Huang@sg.panasonic.com Yoshii.Isamu@jp.panasonic.com hk.yu@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b: P902.16m/D3 |  |
| Abstract | This document contains the suggested modification on Section 16.3.6.2.2.4 Assignment A-MAP |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Changes to Assignment A-MAP (16.3.6.2.2.4) for 802.16m/D3

- Lei Huang

- Panasonic Singapore Laboratories

- Isamu Yoshii

- Panasonic Corporation

- Hyunkyu Yu

- Samsung Electronics

- Introduction

This contribution proposed some clean-up modifications on Section 16.3.6.2.2.4 Assignment A-MAP in [1] according to the outcome of the previous meetings. The modifications include

- Assignment A-MAP IEs are grouped only according to channel coding rate.

- DL/UL composite persistent A-MAP IEs are removed from the examples of multicast assignment A-MAP IEs.

- Broadcast assignment A-MAP IE shall be placed in the beginning of the assignment A-MAP group, similar to multicast assignment A-MAP IEs.

- Reference

- IEEE P802.16m/D3, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, Dec 2009

- Suggested text modification

- Editorial notes

- Current text in IEEE P802.16m/D1

Remove text from IEEE P802.16m/D1

- Add new text to IEEE P802.16m/D1

------------------------------------------------------------Start of the Text--------------------------------------------------------

- Making the following modification in line 36-54, page 369

Assignment A-MAP IEs are grouped together based on channel coding rate and assignment A-MAP IE sizes. Assignment A-MAP IEs in the same group are transmitted in the same frequency partition with the same channel coding rate and contain the same assignment A-MAP IE size. Each assignment A-MAP group contains several logically contiguous MLRUs. The number of assignment A-MAP IEs in each assignment A-MAP group is signaled through non-user specific A-MAP in the same AAI subframe.

If two assignment A-MAP groups using two channel coding rates MCS levels are present in an A-MAP region, assignment A-MAP group using lower channel coding rate more robust MCS is allocated first, followed by assignment A-MAP group using higher channel coding rate less robust MCS.

If a broadcast assignment A-MAP IE, i.e., the assignment A-MAP IE intended for all the AMSs, exists in a DL AAI subframe, it shall be present at the beginning of either assignment A-MAP group 1 or assignment A-MAP group 3.

All the multicast assignment A-MAP IEs, i.e., all the assignment A-MAP IEs intended for a specific group of more than a single AMSs, present in any assignment A-MAP group, shall occupy a contiguous set of MLRUs starting from the beginning of the assignment A-MAP group. If the broadcast assignment A-MAP IE is present in the assignment A-MAP group, the multicast assignment A-MAP IEs are located right after the broadcast assignment A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE and the DL/UL Composite Persistent A-MAP IE are examples of multicast assignment A-MAP IEs.

-------------------------------------------------------------End of the Text--------------------------------------------------------