| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PMI Combination Ratio (PCR) Selection for UL Multi-BS MIMO in IEEE P802.16m/D3 (16.5.2.1.1) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI | Voice: +886-3-5918371 E-mail: pinghengkuo@itri.org.tw |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a P802.16m/D3 : 16.5.2.1.1 |  |
| Abstract | This contribution proposes the selection mechanism of PCR value as required for UL Multi-BS MIMO. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

PMI Combination Ratio (PCR) Selection for UL Multi-BS MIMO

in IEEE P802.16m/D3 (16.5.2.1.1)

Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

# Introduction

In UL Multi-BS MIMO, the precoder is constructed by combining two precoding matrices. One of them corresponds to serving BS PMI, while another one minimizes the interference to the neighboring BS. How the value of PMI combination ratio (PCR) is chosen is still not available in D3 [1]. Hence, in the objective of this contribution is to propose a selection method for PCR. In particular, PCR value is chosen to maximize the ratio of signal power to serving BS and interference power for neighboring BS.

# Reference

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” December 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 16.5.2.1.1 of P802.16m/D3

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.5.2.1.1 DL Signaling

Table 939 shows the control parameters for UL multi-BS MIMO.

Table 939 – Control parameters for UL Multi-BS MIMO

| Parameter | Description | Value | Control channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| PMImin | PMI minimizing interference | 4 bits if Nt=2 6 bits if Nt=4 | AAI_MultiBS_PMI_COM message |  |
| PCR | PMI combination ratio | 0b00: 0.2 0b00: 0.4 0b10: 0.6 0b11: 0.8 | AAI_MultiBS_PMI_COM message |  |

16.5.2.1.2 Operation procedure

Single BS precoding with Multi-BS Coordination is performed by combining two PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells. One of PMIs maximizes the transmission power of the serving cell; another minimizes interference generated to the neighboring cell.

The operation procedure of PMI combination follows the steps below.

- PMI combination may be triggered by an ABS in an unsolicited manner. By using UL sounding signals, the serving and neighboring ABSs may measure channels of AMSs. Upon the channel measurements, the serving and neighboring ABSs may determine PMI maximizing transmission power and PMImin minimizing interference, respectively. The neighboring ABS may inform the serving ABS of PMImin through the backhaul network.

Two PMIs of the predefined codebook {wi}, should be selected based on the following procedure:

PMI=argmaxi(HSWi)2PMIMin=argmini(HIWi)2																(288)

where Hs and HI denote information of channels from the AMS to its serving and corresponding neighboring ABS, respectively.

- The serving ABS shall inform the PMI in A-MAP IE and PMImin and the preferred PCR (PMI combination ratio) index in an AAI_MultiBS_PMI_COM message to the AMS. The transmitted precoder W combining two precoders (WPMI, WPMImin) should be generated as follows: The preferred PCR index, j*, should be chosen as:

W=PCR⋅WPMI+(1−PCR)⋅WPMIMinPCR⋅WPMI+(1−PCR)⋅WPMIMin,                                                     (289)

j=argmaxj(HSWj)2(HIWj)2,                                                                 (xxx)

where j=1…4 is the possible index of PCR, and Wj is the precoder results from the combination of two precoders (WPMI, WPMImin) with the jth PCR value (PCRj):

Wj=PCRj⋅WPMI+(1−PCRj)⋅WPMIMinPCRjWPMI+(1−PCRj)⋅WPMIMin,	j=1,2,3,4.                                    (yyy)

Once the information of PMImin and the preferred PCR index are received by the AMS, it should construct the precoder, Wj*, using equation (yyy).

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------