| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of SA-Preamble Partitioning for ABS Type (16.3.6) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jinsoo Choi, HanGyu Cho, Jin Sam Kwak LG Electronics Youngbo Cho, Sung-Eun Park Samsung Electronics Co., Ltd | {js.choi, hg.cho, jinsam.kwak}@lge.com {youngbo.cho, se.park}@samsung.com |
| Re: | Commets on P802.16m/D3 for recirculation #30b |  |
| Abstract | The contribution provides the proposed text of SA-Preamble partitioning for ABS type to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of SA-Preamble Partitioning for ABS Type (16.3.6)

Jinsoo Choi, HanGyu Cho, Jin Sam Kwak

LG Electronics

Youngbo Cho, Sung-Eun Park

Samsung Electronics Co., Ltd

- Introduction

In previous Jeju meeting, the high level concept of partitioning for SA-Preamble sequences were accepted for supporting partitioning of different ABS type [1]. This contribution provides a proposed text which modifies the current sequence number for macro ABS as other proper value and suggests details for finer partition of remaining SA-Preamble sequences.

- Contents of this proposal

Current status in D3 regarding SA-Preamble partitioning is

- Hard partitioning for macro ABS: 255 sequences for macro ABS (85 sequences per segment)

- Soft partitioning for non-macro ABS: Indicated in S-SFH SP3 and AAI_SCD message

  - In S-SFH SP3: 4bit SA-Preamble sequence soft partitioning information indicates partitioning for public or CSG-femto ABS

  - In AAI_SCD message: SA_PreamblePartitionforBStype indicates finer partitioning of hotzone/relay/OSG-femto/etc for public ABS and CSG-closed/open femto for CSG-femto ABS

Provided contents in this proposal are following.

- The number of sequences for macro ABS

  - Regarding the number of macro ABS which is hard partitioned as 255 sequences, this value should be corrected by a kind of multiple of 6. Among candidate numbers which doesn’t change the current value a lot, 252, 258, 264, etc can be possible sequence number of macro ABS.

  - To set the proper value, it is recommended that remaining sequences can be clearly divided by 16 cases (4bit S-SFH SP3) and partitioned by sequences based on multiple of 6 to support various ABS partition cases as fine as possible and not disturb the current IDCell rule. In this proposal, 258 sequences for macro ABS is suggested so that 16 partition cases of ABS sequences (30 sequences or multiple of that for each case) for public/CSG-femto ABS can be supported with remaining sequences.

- The soft partitioning for public/CSG-femto ABS in S-SFH SP3

  - Except 258 sequences for macro ABS (assuming this value), remaining 510 sequences can be divided by 16 partition cases between public and CSG-femto ABS. Each partition contains 30 sequences (10 sequences per segmentation) or multiple of that.

Table xxx: IDCell partitioning for indicating public/CSG-femto ABS

| Value indicated in 4bit-SFH SP3 | IDCell for public ABS (Number of sequences per each segmentation) | IDCell for private(CSG-femto) ABS (Number of sequences per each segmentation) |
| --- | --- | --- |
| 0000 | 86+256*n ~ 95+256*n (10) | 96+256*n ~ 255+256*n (160) |
| 0001 | 86+256*n ~ 105+256*n (20) | 106+256*n ~ 255+256*n (150) |
| 0010 | 86+256*n ~ 115+256*n (30) | 116+256*n ~ 255+256*n (140) |
| 0011 | 86+256*n ~ 125+256*n (40) | 126+256*n ~ 255+256*n (130) |
| 0100 | 86+256*n ~ 135+256*n (50) | 136+256*n ~ 255+256*n (120) |
| 0101 | 86+256*n ~ 145+256*n (60) | 146+256*n ~ 255+256*n (110) |
| 0110 | 86+256*n ~ 155+256*n (70) | 156+256*n ~ 255+256*n (100) |
| 0111 | 86+256*n ~ 165+256*n (80) | 166+256*n ~ 255+256*n (90) |
| 1000 | 86+256*n ~ 175+256*n (90) | 176+256*n ~ 255+256*n (80) |
| 1001 | 86+256*n ~ 185+256*n (100) | 186+256*n ~ 255+256*n (70) |
| 1010 | 86+256*n ~ 195+256*n (110) | 196+256*n ~ 255+256*n (60) |
| 1011 | 86+256*n ~ 205+256*n (120) | 206+256*n ~ 255+256*n (50) |
| 1100 | 86+256*n ~ 215+256*n (130) | 216+256*n ~ 255+256*n (40) |
| 1101 | 86+256*n ~ 225+256*n (140) | 226+256*n ~ 255+256*n (30) |
| 1110 | 86+256*n ~ 235+256*n (150) | 236+256*n ~ 255+256*n (20) |
| 1111 | 86+256*n ~ 245+256*n (160) | 246+256*n ~ 255+256*n (10) |

- Finer partitioning in AAI_SCD message

  - Based on sequence partitioning in S-SFH SP3, finer partitioning in AAI_SCD can be applied with same granularity of sequence partitioning in S-SFH SP3 (by 4bit-indicator for each ABS type).

- References

[1] IEEE 802.16m-09/1902r3, “Proposed Text Proposal of SA-Preamble Partition in IEEE 802.16m/D1”

[2] IEEE 802.16m-07/0034r2, “802.16m System Requirements”

[3] IEEE 802.16m/D3, “Advanced Air Interface (Draft)”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Modify the paragraph starting in line 15 of page 350 of P802.16m/D3 as following

-------------------------------------------------------  Text Start  --------------------------------------------------------------

SA-Preamble sequences are partitioned and each partition is dedicated to specific base station type like macrocell ABS, Macro Hotzone ABS, Femto ABS. The base station types are categorized into macro ABS and non-macro ABS cells by hard partition with 255 258 sequences (85 86 sequences per segment * 3 segments) dedicated for macro ABS. The non-macro ABS information is broadcasted in a hierarchical structure, which composes of S-SFH SP3 and AAI_SCD message. In S-SFH SP3, non-macro ABS cell type is partitioned as public and CSG femto base stations. Total 16 cases of IDCell partition for public and CSG-femto ABS are shown in Table xxx, which is composed of IDCell partitions based on 30 sequence (10 sequences per segmentation) granularity. The AAI_SCD message provides finer partition information. The non-macro ABS base stations can be further categorized different types, such as: hotzone, relay, OSG and etc. The CSG femto base stations can be further categorized, such as: CSG-closed and CSG-open. For fine partitioning of each ABS type, each 4bit-indicator of IDCell partitioning per ABS type to be supported is transmitted in the AAI_SCD message, which has same partitioning rule as shown in Table xxx.

Table xxx: IDCell partitioning for public ABS and CSG-femto ABS

| Value indicated in 4bit-SFH SP3 | IDCell partition for public ABS (Number of sequence per each segmentation) | IDCell partition for CSG-femto ABS (Number of sequence per each segmentation) |
| --- | --- | --- |
| 0000 | 86+256*n ~ 95+256*n (10) | 96+256*n ~ 255+256*n (160) |
| 0001 | 86+256*n ~ 105+256*n (20) | 106+256*n ~ 255+256*n (150) |
| 0010 | 86+256*n ~ 115+256*n (30) | 116+256*n ~ 255+256*n (140) |
| 0011 | 86+256*n ~ 125+256*n (40) | 126+256*n ~ 255+256*n (130) |
| 0100 | 86+256*n ~ 135+256*n (50) | 136+256*n ~ 255+256*n (120) |
| 0101 | 86+256*n ~ 145+256*n (60) | 146+256*n ~ 255+256*n (110) |
| 0110 | 86+256*n ~ 155+256*n (70) | 156+256*n ~ 255+256*n (100) |
| 0111 | 86+256*n ~ 165+256*n (80) | 166+256*n ~ 255+256*n (90) |
| 1000 | 86+256*n ~ 175+256*n (90) | 176+256*n ~ 255+256*n (80) |
| 1001 | 86+256*n ~ 185+256*n (100) | 186+256*n ~ 255+256*n (70) |
| 1010 | 86+256*n ~ 195+256*n (110) | 196+256*n ~ 255+256*n (60) |
| 1011 | 86+256*n ~ 205+256*n (120) | 206+256*n ~ 255+256*n (50) |
| 1100 | 86+256*n ~ 215+256*n (130) | 216+256*n ~ 255+256*n (40) |
| 1101 | 86+256*n ~ 225+256*n (140) | 226+256*n ~ 255+256*n (30) |
| 1110 | 86+256*n ~ 235+256*n (150) | 236+256*n ~ 255+256*n (20) |
| 1111 | 86+256*n ~ 245+256*n (160) | 246+256*n ~ 255+256*n (10) |

-------------------------------------------------------  Text End  ----------------------------------------------------------------