| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on Emergency Alert Service (16.2.12) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Giwon Park, Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak LG Electronics | gwpark@lge.com |
| Re: | Category: LB30b / Area: Chapter 16.2.12.7 |  |
| Abstract | This contribution proposes changes to the Section 16.2.12.7 of IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on Emergency Alert Service (16.2.12)

Giwon Park, Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics

- Introduction

Although current D3 [1] specifies emergency alert service. However, there is no specific mechanism defined to achieve this except idle mode.

This proposal proposes methods using which AMS in connected mode, idle mode and sleep mode can ensure that it receives the emergency alert information.

- Proposed text

======================== Start of Proposed Text ========================

[Add the paragraph (in line 11 on page 216) of P802.16m/D3 as follows]

- 16.2.12.7 Emergency Alert Service

Emergency alert service is defined as a service that would provide the public with alerts on imminent emergency events, such as earthquake, storm, etc. The alerts would target subscribers in a specific geographical location.

An emergency alert indicator is included in the broadcast assignment A-MAP IE to notify the AMSs reception of the emergency alert information as shown in table 829. Upon reception of the emergency alert indicator that is set to '1', the AMS shall obtain the emergency alert information. When an AMS in Idle Mode or Sleep Mode receives the Broadcast Assignment A-MAP IE with emergency alert indicator that is set to ‘1’ during its own interval (i.e., Paging Listening Interval in Idle Mode or Listening Window in Sleep Mode), the AMS shall stay awake until it receives the Emergency Alert Information. AAI_L2_XFER is used to send emergency alert information.

[Change table 829 in section 16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE:]

- 16.3.6.5.2.4.13 broadcast assignment A-MAP IE

Table 829 – Broadcast Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| MCS | TBD | TBD |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Emergency Alert Indicator | 1 | 0 = There is no emergency alert information. 1 = There is emergency alert information. When the AMS detects that Emergency Alert Indicator bit is set to 1, AMS in Connected Mode, Idle Mode or Sleep Mode shall receive emergency alert information. |
| Reserved | TBD | Reserved bits |
| } |  |  |

[Modify the paragraph (in line 38 on page 94) of P802.16m/D3 as follows]

- 16.2.3.28 L2 Transfer message (AAI_L2_XFER)

h) Transfer-Type = 8; Emergency Alert Information

ih) Transfer-Type = 89-127; reserved

ji) Transfer-Type = 128-255; Vendor specific types

[Delete the paragraph (in line 48 on page 262) of P802.16m/D3 as follows]

16.2.17.2.1 Broadcast paging message

In addition, an emergency alert indicator is included in the paging message to notify the idle AMSs reception of the emergency information. Upon reception of the emergency alert indicator that is set to '1', the AMS shall decode the A-A-MAP to obtain the emergency information.

[Change table 756 in section 16.2.17.2.1 AAI_PAG-ADV message:]

16.2.17.2.1 Broadcast paging message

Table 756—Parameters for AAI_PAG-ADV Message Format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Deregistration Identifier | 0~1023 | Deregistration Identifier Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Deregistration Identifier Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Reserved | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a predetermined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or sub-frame. |
| Emergency Alert Indication | 0 = There is no emergency information 1 = There is emergency information | If the emergency alert indicator is set to '1', the AMS shall decode the A-A-MAP to obtain the emergency information. |

======================== End of Proposed Text ========================

- Reference

[1] P802.16m/D3, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, December 2009