| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Idle Mode Initiation (16.2.17) |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak LG Electronics | gwpark@lge.com |
| Re: | Category: LB30b / Area: Chapter 16.2.17.1 |  |
| Abstract | This contribution proposes changes to the Section 16.2.17.1 of IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Idle Mode Initiation (16.2.17)

Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics

- Introduction

This contribution proposes a clarification on the idle mode entry procedure with the following point.

- Current action code (0x05, to allow AMS to transmit AMS-initiated idle mode request) of DREG-RSP message does not resolve the following problem.

Case (Msg from the ABS is not received at the AMS)

ABS  AMS (0x05): This message gets lost

AMS  ABS (0x01): AMS sends the AAI_DREG-REQ message with action code 0x01 (AMS initiated idle mode entry)

ABS  AMS (0x05): Although AMS expects to receive the DREG-RSP (0x07) but ABS sends the AAI_DREG-RSP with action code 0x05.

- Proposed text

======================== Start of Proposed Text ========================

[Modify the paragraph (line 44 on page 258) of P802.16m/D3 as follows]

- 16.2.17.1.2 ABS initiated

In this case, ABS shall not start T46 timer. AMS may include Idle Mode Retain Information TLV with in AAI_DREG-REQ message with De-Registration_Request Code = 0x01 transmitted at the REQ-duration expiration. In this case, If the ABS receives the AAI_DREG-REQ (De-registration_Request_Code = 0x01), ABS shall transmit another AAI_DREG-RSP message with Action Code=0x057 including Idle Mode Retain Information TLV.

[Replace Type 2 BAS initiated idle mode entry of Figure 441(on page 258) of P802.16m/D3 to the following figure]

Type 2 ABS initiated idle mode entry

Figure 441 – Type 2 ABS initiated idle mode entry

[Modify Figure 444(on page 261) of P802.16m/D3 to the following figure]

Figure 444 – AMS Procedures during Type 2 ABS initiated idle mode entry

[Modify Figure 445(on page 262) of P802.16m/D3 to the following figure]

Figure 445 – ABS Procedures during Type 2 ABS initiated idle mode entry

======================== End of Proposed Text ========================

Reference

[1] P802.16m/D3, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, December 2009