| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SMS confirmation (16.2.24) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Hyunjeong Kang, Jungshin Park, Simon Kong, Sajin Kim, Jonghyun Won Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16m-09/0073, “IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement” |  |
| Abstract | The contribution proposes a confirmation message on DL SMS during idle mode. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SMS confirmation (16.2.24)

Hyunjeong Kang, Jungshin Park, Simon Kong, Sajin Kim, Jonghyun Won

Samsung Electronics

- Introduction

Current IEEE802.16m draft [1] has a mechanism to support SMS in connected state and idle state as in the table 1.

Table 1 SMS support in connected state and idle state

| State SMS support | Connected state | Idle state |
| --- | --- | --- |
| SMS | DL/UL: AAI_L2-XFER (Transfer-Type=6 & sub-type=SMS data) | UL: AAI_RNG-REQ DL: AAI_RNG-RSP |
| SMS confirmation | DL/UL: AAI_L2-XFER (Transfer-Type=6 & sub-type=SMS confirmation) | UL: not defined DL: AAI_RNG-RSP |

According to table 1, UL SMS confirmation in idle state is not defined that is an acknowledgement on an AAI_RNG-RSP including DL SMS. Therefore we propose a mechanism to send UL SMS confirmation in idle state.

An AAI_MSG-ACK message can be sent as UL SMS confirmation when an idle mode AMS receives a DL SMS through an AAI_RNG-RSP message. The AMS needs UL resource to send the AAI_MSG-ACK message and it shall be granted with the UL resource in unsolicited manner because the AMS has no assigned STID in idle state. The ABS, after sending the AAI_RNG-RSP with SMS, shall grant a CDMA Allocation A-MAP IE for the AMS to send the AAI_MSG-ACK as a UL SMS confirmation by an ACK timer. The MCRC of CDMA Allocation A-MAP IE is masked with a RAID, where the RAID is the same as the RAID masked in CDMA Allocation A-MAP IE for the AAI_RNG-REQ message sent by the AMS. When the AMS receives the AAI_RNG-RSP with SMS, it starts the ACK timer and waits for the CDMA Allocation A-MAP IE to send the AAI_MSG-ACK. The figure 1 shows a scenario of DL SMS and SMS confirmation in idle state.

Figure 1 DL SMS and SMS confirmation in idle state

Reference

[1] P802.16m/D3, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, December 2009

- Suggested remedy

-------------------------------  Text Start  ---------------------------------------------------

[Add the following text after line 65 on page 286 in section 16.2.24 Short Message Service:]

When an AAI_RNG-RSP message includes DL SMS, an AAI_MSG-ACK message is sent as a confirmation of the DL SMS. The ABS grants a CDMA Allocation A-MAP IE for the AAI_MSG-ACK in unsolicited manner by an ACK timer, where MCRC is masked with the same RAID as in CDMA Allocation A-MAP IE for the AAI_RNG-REQ message. When the AMS receives the AAI_RNG-RSP with the DL SMS, the AMS starts the ACK timer and waits the CDMA Allocation A-MAP IE to send the AAI_MSG-ACK message.

-------------------------------  Text End  ---------------------------------------------------