| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MC-ADV message format (16.2.8.2.3.1.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Hyunjeong Kang, Jaehyuk Jang, Sung-Eun Park, Jung Je Son, Rakesh Taori Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-09/0073: “IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement” Section 16.2.8.2.3.1.1 AAI_MC-ADV (multicarrier Advertisement) Message |  |
| Abstract | This contribution provides detailed format of the AAI_MC-ADV message |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_MC-ADV message format (16.2.8.2.3.1.1)

Hyunjeong Kang, Jaehyuk Jang, Sung-Eun Park, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides some clarification on the AAI_MC-ADV message in 16m/D3.

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Modify the section 16.2.8.2.3.1.1 as follows:]

16.2.8.2.3.1.1 AAI_MC-ADV (multicarrier Advertisement) Message

The ABS shall periodically broadcast AAI_MC-ADV message for the reception by all AMSs.

Table 735 —AAI_MC-ADV MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| Multi-carrier configuration change count | 4 | A change count associated with a particular configuration for all possible carriers in the network. Incremented by 1 upon each update. |
| MC configuration broadcast flag | 1 | 0: only broadcast serving BS specific info 1: broadcast multi-carrier definition for fro all available carriers in the network |
| If (MC configuration broadcast flag==1) { |  |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| Uniformity Flag | 1 | 0: All Carriers in one group have the same Protocol Version, Carrier Type Duplexing Mode and Transmit Power 1: Otherwise |
| for (i=1; i<=Number of Carrier Groups; i++) { | — | — |
| Carrier configuration index | 6 | Index associated to Table 770 764 |
| Start frequency assignment index | 6 | Frequency assignment index of the first carrier in carrier group i |
| Number of carriers | 6 |  |
| for (j=i1; j<=Number of carriers’; j++){ | — | — |
| Physical carrier index | 36 | — |
| Preamble index | 610 | — |
| } |  |  |
| if (Uniformity flag==1) { | — | — |
| MAC Protocol version | 82 | 00: IEEE 802.16-2009 only mode 01: AAI and IEEE 802.16-2009 mixed mode 10: AAI only mode 11: reserved Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| Carrier type | 1 | 0: for fully configured 1: for partially configured |
| Duplexing mode | 1 | 0: for TDD 1: for FDD |
| Transmit power | 6 | Maximum transmit power level |
| PA-Preamble transmit power | 6 | Values indicate power levels in 1 dB steps starting from 0 dBm |
| SA-Preamble transmit power | 6 | Values indicate power levels in 1 dB steps starting from 0 dBm |
| } | — | — |
| Physical carrier index of current carrier | 6 | The carrier that ABS broadcast this message |
| else { | — | — |
| for (j=1; j<=Number of carriers; j++){ | — | — |
| MAC Protocol version | 82 | 00: IEEE 802.16-2009 only mode 01: AAI and IEEE 802.16-2009 mixed mode 10: AAI only mode 11: reserved Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| Carrier type | 1 | 0: for fully configured 1: for partially configured |
| Duplexing mode | 1 | 0: for TDD 1: for FDD |
| PA-Preamble transmit power | 6 | Values indicate power levels in 1 dB steps starting from 0 dBm |
| SA-Preamble transmit power | 6 | Values indicate power levels in 1 dB steps starting from 0 dBm |
| } | — | — |
| } | — | — |
| } | — | — |
| } |  |  |
| else { |  | // MC configuration broadcast flag == 0 |
| Serving BS Carrier Number | 6 | //start broadcasting serving BS specific info |
| For(i=0; i<=Serving BS Carrier Number-1;i++){ |  |  |
| Physical carrier index | 6 |  |
| Preamble Index | 10 |  |
| PA-Preamble transmit power | 6 | Values indicate power levels in 1 dB steps starting from 0 dBm |
| SA-Preamble transmit power | 6 | Values indicate power levels in 1 dB steps starting from 0 dBm |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode > 1 |
| For(i=0; i<=P; i++){ |  |  |
| MAC protocol version | 84 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m MAC version. 00: IEEE 802.16-2009 only mode 01: AAI and IEEE 802.16-2009 mixed mode 10: AAI only mode 11: reserved |
| Preamble Index | 6 |  |
| } |  |  |
| } |  |  |
| } |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------