| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Power Control Section of the IEEE 802.16m/D3 Amendment (16.3.9.4) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jeongho Park Samsung Wookbong Lee LG Tim Wilson Motorola Rongzhen Yang Intel | Email : jeongho.jh.park@samsung.com Email : wbong@lge.com Email : Tim.Wilson@motorola.com Email : rongzhen.yang@intel.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement” |  |
| Abstract | The contribution proposes the text on Power Control Section |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Power Control Section (16.3.9.4)

Jeongho Park

Samsung Electronics

Wookbong Lee

LG

Tim Wilson

Motorola

Rongzhen Yang

Intel

# Introduction

This contribution includes the proposed text on the power control section for the IEEE 802.16m/D3 Amendment.

# References

[1] IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Oct. 2009.

# Text proposal for the 802.16m amendment

Instruction to editor;

Black text: existing D3 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy Part 1:

Change the text from line 1 on the page 560, in 16.3.9.4, as follows]

============================== Start of Proposed Text =================================

# 16.3.9.4 Uplink Power Control

Uplink power control is supported for both an initial calibration and periodic adjustment on transmit power without loss of data. The uplink power control algorithm determines the transmission power of an OFDM symbol to compensate for the pathloss, shadowing and fast fading. Uplink power control shall intend to control inter-cell interference level.

A transmitting AMS shall maintain the same transmitted power density, unless the maximum power level is reached. In other words, when the number of active LRU allocated to a user is reduced, the total transmitted power shall be reduced proportionally by the AMS as long as there is no additional change of parameters for power control. When the number of LRU is increased, the total transmitted power shall also be increased proportionally. However, the transmitted power level shall not exceed the maximum levels dictated by signal integrity considerations and regulatory requirements. In closed-loop power control, the AMS shall interpret PC-A-MAP IE as the required changes to the transmitted power density.

For interference level control, current interference level of each ABS may be shared among ABSs.

## 16.3.9.4.1 UL Open-Loop Power Control

When the open-loop power control is used, the The power per subcarrier and per transmission antenna stream shall be maintained for the UL transmission as indicated in calculated according to Equation (269).

(269)

P(dBm)=L+SINRTarget+NI+Offset					(269)

Where:

is the target uplink SINR received by the ABS. The mode used to calculate this value is signaled through a power control message.

P is the TX power level (dBm) per stream and per subcarrier for the current transmission.

L is the estimated average current UDL propagation loss calculated by AMS. It shall include AMS’s Tx antenna gain and path loss.

NI is the estimated average power level (dBm) of the noise and interference per subcarrier at the ABS, not including ABS’s Rx antenna gain as indicated in the AAI_ULPC_NI message

is a correction term for AMS-specific power offset. It is controlled by the AMS. Its initial value is zero. (TBD)

Offset is a correction term for AMS-specific power offset. It is controlled by the ABS through power control messages. There are 2 kinds of Offset values (OffsetData and OffsetControl) that are used for data and control respectively. Further details are given in following sections (16.3.9.4.1, 16.3.9.4.2 and 16.3.9.4.3).

The estimated average current UDL propagation loss, L, shall be calculated based on the total power received on the active subcarriers of the frame preamble.

16.3.9.4.1 Power Control for Data Channel

The Offset_ABSperAMS can be updated by ABS. Once AMS recognized this parameter, AMS should increase and decrease the Offset_ABSperAMS according to this value sent by ABS.

When the user connects to network, it can negotiate the parameters used in Equation (270):+

For data channel transmission, the SINRTarget term in Equation (269) is calculated according to Equation (270):

SINRTarget=10log10 max10(SINRMIN(dB)10), γIoT×SINRDL−α  −β×10log10(TNS), OLPC Mode 1CN, OLPC Mode 2

SINRTarget=10log10max10(SINRMIN(dB)10), γIoT×SIRDL−α−β×10log10(TNS) 				(270)

Where

C/N is the normalized C/N of the modulation/FEC rate for the current transmission, as appearing in Table 911.

OLPC Mode 1 Equation(270) is the target SINR value for IoT control and tradeoff between overall system throughput and cell edge performance, decided by the control parameter  and SINRMINSINRMIN(dB). Each parameter used in OLPC mode1 is explained as follows:

The parameters used in equation 270 are broadcasted in AAI_SCD message (section 16.2.3.29) unless otherwise noted. The explanation for the parameters is as follows:

Where

SINRMIN(dB)SINRMIN is the SINR requirement (SINRmin_Data) for the minimum rate expected by ABS which is set by a unicast power control message has 4 bits to represent the value in dB among{,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5}

is the fairness and IoT control factor, broadcast factor broadcasted by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It can also be different for each frequency partition. Details on signaling of  is found in section 15.2.x.2.

SIRDLis the linear ratio of the downlink signal to interference power, measured by the AMS.

is the factor according to the number of receive antennas at the ABS. It is signaled from MAC power control mode signaling by 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved}.

is set to be zero or one to determine the influence of TNS on SINRTarget by one bit of MAC power control mode signaling.

TNS is the Total Number of Streams in the LRU indicated by UL A-MAP IE. In case of SU-MIMO, this value shall be set to Mt where Mt is the number of streams for AMS. In case of CSM, TNS is the aggregated number of streams

When calculated data channel SINRTarget is higher than SINRmax_Data defined in AAI_SCD message,  SINRTarget shall be set to SINRmax_Data.

TNS is the Total Number of Streams in the LRU indicated by UL A-MAP IE. In case of SU-MIMO, this value shall be set to Mt where Mt is the number of streams for AMS. In case of CSM, TNS is the aggregated number of streams. In case of control channel transmission, this value shall be set to one.

For a data channel transmission, the Offset in Equation 269 shall be set to the value OffsetData conveyed in an AAI UL_POWER_ADJUST message. The value used shall be taken from the most recent message which preceded the first uplink subframe of the frame by more than Tproc. The default value of OffsetData shall be initialized to 0 if AMS never get AAI_UL_POWER_ADJUST message.

16.3.9.4.2 Power Control for Control Channels

In the case of control channel transmission, except for initial ranging and sounding transmission, SINRtarget in equation (269) is set according to Table 911. The parameters in Table 911 are conveyed in the AAI_SCD message.

Table 911 SINRtarget Parameters for Control Channel

| Control Channel Type | SINRtarget Parameters |
| --- | --- |
| HARQ Feedback | SINR_Target_HARQ |
| Synchronized Ranging | SINR_Target_SyncRanging |
| P-FBCH | SINR_Target_PFBCH |
| S-FBCH | SINR_Target_SFBCH_Base |
|  | SINR_Target_SFBCH_Delta |
| Bandwidth Request | SINR_Target_BWRequest |

For HARQ Feedback, Synchronized Ranging, P-FBCH and Bandwidth Request, the SINRTarget values are indicated directly in Table 911.

For S-FBCH channel, the SINRtarget value is defined in equation (271):

SINRTarget(SFBCH)=SFBCHBase+(l−lmin)×SFBCHDelta			(271)

Where

SFBCHBase is base SINRtarget value signaled by SINR_Target_SFBCH_BASE;

SFBCHDelta is differential SINRtarget value signaled by SINR_Target_SFBCH_Delta;

lis S-FBCH payload information bits number defined in 16.3.9.2.1.2;

lminis minimum S-FBCH payload information bits number (lmin= 7 ) defined in 16.3.9.2.1.2;

For a control channel transmission, the Offset in Equation 269 shall be set to the value OffsetControl conveyed in an AAI UL_POWER_ADJUST message. The value used shall be taken from the most recent message which preceded the first uplink subframe of the frame by more than Tproc. The default value of OffsetControl shall be initialized to 0 if AMS never get AAI_UL_POWER_ADJUST message.

Table 911—Normalized C/Nper modulation

| Type | Modulation/FEC rate | Required C/NSINRtarget |
| --- | --- | --- |
| Control Channel | ACK/NAK |  |
|  | CQI |  |
|  | Ranging code |  |
|  | P-FBCH |  |
|  | S-FBCH |  |
|  | Analog Feedback |  |
|  | Bandwidth Request |  |
|  | Sounding |  |
| Rank 1 MCS (Index) | ‘0000’ |  |
|  | ‘0001’ |  |
|  | ‘0010’ |  |
|  | ‘0011’ |  |
|  | ‘0100’ |  |
|  | ‘0101’ |  |
|  | ‘0110’ |  |
|  | ‘0111’ |  |
|  | ‘1000’ |  |
|  | ‘1001’ |  |
|  | ‘1010’ |  |
|  | ‘1011’ |  |
|  | ‘1100’ |  |
|  | ‘1101’ |  |
|  | ‘1110’ |  |
|  | ‘1111’ |  |
| Rank 2 MCS (Index) | ‘0000’ |  |
|  | ‘0001’ |  |
|  | ‘0010’ |  |
|  | ‘0011’ |  |
|  | ‘0100’ |  |
|  | ‘0101’ |  |
|  | ‘0110’ |  |
|  | ‘0111’ |  |
|  | ‘1000’ |  |
|  | ‘1001’ |  |
|  | ‘1010’ |  |
|  | ‘1011’ |  |
|  | ‘1100’ |  |
|  | ‘1101’ |  |
|  | ‘1110’ |  |
|  | ‘1111’ |  |
| (TBD) | (TBD) | (TBD) |

16.3.9.4.23 UL Closed-Loop Power Control Power Correction using PC-A-MAP

To maintain at the ABS a power density consistent with the modulation and FEC rate used by each AMS, the ABS may change the AMS’s TX power of control channel through direct power adjustment signaling such as by PC-A-MAP. Closed loop power control is defined in Equation (271). When AMS receives its PC-A-MAP IE from the ABS, it shall modify its OffsetControl value according to Equation (272).

OffsetControl = OffsetControl + ΔPowerAdjust                                                (272)

where,

ΔPowerAdjust is the power correction value indicated by ABS through PC-A-MAP.

Note that is initially set to zero until AMS receives the first AAI_UL_POWER_ADJUST message. And if AMS receives AAI_UL_Power_ADJUST message at i-th frame, the AMS’s shall be replaced by the value indicated by the message from the (i+1)th frame. Also PC-A-MAP IE from (i+1)th frame is effective on this new value.

Ptx=Plast+ΔSINR+ΔpowerAdjust										(274)

Where

Ptx is transmit powe level per subcarrier

Plast is the latest transmitted maximum power level among different uplink physical channels transmitted concurrently.

ΔSINR is the difference of the desired SINRs between the previous and new MCS levels for a uplink physical channel. Desired SINR for each MCS level is TBD.

ΔpowerAdjust is the value indicated by PC-A-MAP.

Power correction values are defined in Table 912.

Table 912– Power correction offset values

| Power Correction Value | Offset (dB) |
| --- | --- |
| 0b00 | -0.5 |
| 0b01 | 0.0 |
| 0b10 | 0.5 |
| 0b11 | 1.0 |

============================== End of Proposed Text =================================

[Remedy Part 2:

Change the text from line 46 on the page 94, in 16.2.3.29, as follows]

============================== Start of Proposed Text =================================

16.2.3.29 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change			INTEGER (0 ~ 15)

BS_Restart_Count				INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ON				INTEGER (0..1) OPTIONAL

OL-Rank1-Configuration			INTEGER (0..7) OPTIONAL

SB-OL-Region-2Size			INTEGER (0..15) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

ULPC_DataChannel_Parameters :: = SEQUENCE {

GAMMA_IOT_FP0			INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1			INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2			INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3			INTEGER (0..15) OPTIONAL

Alpha ()				INTEGER (0.. 7) OPTIONAL

Beta (ß)					INTEGER (0..1) OPTIONAL

SINRmin_Data				INTERGER (0..15) OPTIONAL

SINRmax_Data				INTERGER (0..15) OPTIONAL

}

ULPC_ControlChannel_Parameters :: = SEQUENCE {

SINR_Target_HARQ			INTEGER (0..15) OPTIONAL

SINR_Target_SyncRanging		INTEGER (0..15) OPTIONAL

SINR_Target_PFBCH			INTEGER (0..15) OPTIONAL

SINR_Target_SFBCH_BASE		INTEGER (0..15) OPTIONAL

SINR_Target_SFBCH_Delta		INTEGER (0..7) OPTIONAL

SINR_Target_BWRequest			INTEGER (0..15) OPTIONAL

GAMMA_IOT_SOUNDING		INTEGER (0..15) OPTIONAL

SINRmin_SOUNDING			INTERGER (0..15) OPTIONAL

SINRmax_SOUNDING			INTERGER (0..15) OPTIONAL

}

BR_Channel Configuration MIN Access ClassINTERGER (0..4) OPTIONAL

Sounding_multiplexing_type CHOICE {

fdm			FDM

cdm			CDM

}

Decimation_value_D/Max_Cyclic_Shift_Index_P_for_soundingENUMERATED {n4, n9, n16,

n18, n36}

RELAY_Support_Parameters :: = SEQUENCE {

16m_Relay_zone_AMS_allocation_indicator	INTEGER (0..1) OPTIONAL

}

}

-- ASN1STOP

ULPC_DataChannel_Parameters Field Descriptions

GAMMA_IOT_FP0, GAMMA_IOT_FP1, GAMMA_IOT_FP2, GAMMA_IOT_FP3, GAMMA_IOT (IoT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3)

Alpha ():  is the factor according to the number of receive antennas at the ABS. It is 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved}.

Beta (ß)  is set to be 0 or 1 by one bit, which is used to indicate disable or enable of the power de-boosting for uplink multi-stream transmission.

SINRmin_Data SINRmin_Data is the SINR requirement for the minimum data rate expected by ABS. SINRmin_Data has 4 bits to represent the value in dB among{,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4}

SINRmax_Data SINRmax_Data is the maximum SINR threshold defined by ABS. SINRmmax_Data has 4 bits to represent the value in dB among{10，12，14，16，18，20，22，24，26，28，30，32，34，36，38，40}

ULPC_ControlChannel_Parameters Field Descriptions

SINR_Target_HARQ SINR_Target_HARQ is the HARQ feedback channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB

SINR_Target_SyncRanging: SINR_Target_SyncRanging is the synchronized channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-9, -8.5, -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2, -1.5} dB

SINR_Target_PFBCH SINR_Target_PFBCH is the P-FBCH target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB

SINR_Target_SFBCH_BASE SINR_Target_SFBCH_BASE is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB.

SINR_Target_SFBCH_Delta SINR_Target_SFBCH_Delta is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23, 0.24, 0.25, 0.26}

SINR_Target_BWRequest SINR_Target_BWRequest is the bandwidth request channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB

GAMMA_IOT_SOUNDING GAMMA_IOT_SOUNDING is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}.

SINRmin_SOUNDING SINRmin_SOUNDING is the minimum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { -4, -3.5, -3, -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5}

SINRmax_SOUNDING SINRmax_SOUNDING is the maximum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20}

============================== End of Proposed Text =================================

[Remedy Part 3:]

[Change the text from line 22 to 52 on the page 564, in 16.3.9.4.4, as follows:]

============================== Start of Proposed Text ==============================

16.3.9.4.4 Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. AMS’s transmit power for UL sounding channel is controlled separately according to its sounding channel target SINR value. The power per subcarrier shall be maintained for the UL sounding transmission as shown in Equation (269) of 16.3.9.4.1.

In Equation (269), SINRtTarget is the sounding channel target SINR, which is set according to the DL SINR of the AMS defined by parameter SIRDL. In order to maintain the UL sounding quality, the different target SINR values are assigned according to the DL SINR of each AMS; the AMS with high DL SINR applies relatively high target SINR and the AMS with low DL SINR applies relatively low target SINR.

The SINRtarget for sounding channel shall be calculated from equation 270 using the following parameter settings: SINRMIN is the minimum SINR requirement expected by ABS which is set to SINRmin_SOUNDING,  is set to IoT control factor for sounding channel GAMMA_IOT_SOUNDING, SINRDL is the ratio of the downlink signal vs. noise and interference power, measured by the AMS, and parameters shall be set to 0.

When calculated sounding SINRtarget is higher than SINRmax_SOUNDING, the SINRtarget shall be set to SINRmax_SOUNDING.

All parameters necessary for sounding channel power control are transmitted through AAI_SCD message in 16.2.3.2829.

============================== End of Proposed Text ================================

[Remedy Part 4:]

[Insert the new text from line 44 on the page 117, under 16.2.3, as follows:]

============================== Start of Proposed Text ==============================

16.2.3.xx AAI_UL POWER ADJUST message

An AAI_UL_POWER_ADJUST message shall be transmitted by the ABS to control the transmit power level of AMS.

-- ASN1START

UL_POWER_ADJUST:: = SEQUENCE {

OFFSET_DATA	         		INTEGER (0..63)

OFFSET_CONTROL			INTEGER (0..63)

}

-- ASN1STOP

UL_POWER_ADJUST message Field Descriptions

OFFSET_DATA OFFSET_DATA is the transmit power adjustment value transmitted by the ABS. It has 6 bits to represent the value among -15.5 to 16 dB with 0.5 dB step; OFFSET_CONTROL OFFSET_CONTROL is the transmit power adjustment value transmitted by the ABS. It has 6 bits to represent the value among -15.5 to 16 dB with 0.5 dB step;

[Remedy Part 5:]

[Insert the new text from line 53 on the page 40, in 16.2.3, as follows:]

============================== Start of Proposed Text ==============================

| Message name | Message description | Security | Connection |
| --- | --- | --- | --- |
| AAI_UL POWER ADJUST | Uplink TX power adjustment | Encrypted/ICV | Unicast |

============================== End of Proposed Text ================================