| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text in frequency partition configuration (16.3.5.2.3, 16.3.8.2.3) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jeongho Park Samsung Electronics | jeongho.jh.park @samsung.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement” |  |
| Abstract | This contribution provides proposal of text modification in frequency partition configuration. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification in frequency partition configuration

Jeongho Park

Samsung Electronics

- Introduction

This contribution provides proposal of text modification in frequency partitioning section.

- Motivation

The definition of partial loading is as follows:

Partial loading = (Number of LRUs in two deboosted resue-3 partitions) / (Total number of LRUs)

For example if partial loading is 25% in 10MHz case, FP in reuse-1 has 30 LRUs for its resource and all other FPs in reuse-3 have 6 LRUs for each FP. On the contrary, partial loading of 63% represents only 3 LRUs and 15 LRUs for FP in reuse-1 and in each reuse-3 respectively.

| MS speed | Partial loading | LRU size (FP0,FP1,FP2,FP3) | Sector Avg. | 5%-tile edge | 10%-tile edge |
| --- | --- | --- | --- | --- | --- |
| 3km/h | 25% | (30, 6, 6, 6) | 0.00% | 0.00% | 0.00% |
|  | 38% | (21, 9, 9, 9) | 3.54% | 14.47% | 2.46% |
|  | 50% | (12, 12, 12, 12) | 4.82% | 14.54% | 3.19% |
|  | 63% | (3, 15, 15, 15) | 1.45% | 9.88% | 1.16% |
| Mixed (6:3:1 = 3:30:120km/h) | 25% | (30, 6, 6, 6) | 0.00% | 0.00% | 0.00% |
|  | 38% | (21, 9, 9, 9) | 3.29% | 7.87% | 12.87% |
|  | 50% | (12, 12, 12, 12) | 3.05% | 11.27% | 10.37% |
|  | 63% | (3, 15, 15, 15) | 2.34% | 16.52% | 7.55% |

As shown in the table, partial loading around 50% shows the best results. Note that the ratio of resources does not reflect the control region, i.e. all LRUs were used for data transmission in the simulation. From the FFR performnace perspective, therefore, having so many FFR configurations is not really necessary.

Considering downlink control region for A-MAP, 9:5:5:5 would be fit for the best case described above. So 1:1:1:1 could be removed. And even though various SLRU size in FP0 is considered, 5:1:1:1 might not be very necessary because 3:1:1:1 and 9:1:1:1 already exist instead.

- Text proposal for the 802.16m/D2

Remedy#1 :

[Change the text from Table 777 on the page 323 to Table 781 on the page 324 in section 16.3.5.2.3 as follows:]

============================== Start of Proposed Text =================================

Table 777—Mapping between DFPC and frequency partitioning for 20 MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 53 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 64 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 75 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 86 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |
| 97-15 | Reserved |  |  |  |

Table 778—Mapping between DFPC and frequency partitioning for 20 MHz>BW>10 MHz

| DFPC | Freq. Partitioning (Approx.) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | FPS1 = NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/4) |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/6) |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/8) |
| 53 | 9 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/12) |
| 64 | 9 : 5 : 5 : 5 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU*5/24) |
| 5-15 | Reserved |  |  |  |

Table 779—Mapping between DFPC and frequency partitioning for 10 MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 53 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 64 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 75 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |
| 6-7 | Reserved |  |  |  |

Table 780—Mapping between DFPC and frequency partitioning for 10 MHz>BW>5 MHz

| DFPC | Freq. Partitioning (Approx.) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | FPS1 = NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/4) |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/6) |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/8) |
| 53 | 9 : 5 : 5 : 5 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU*5/24) |
| 64-7 | Reserved |  |  |  |

Table 781—Mapping between DFPC and frequency partitioning for 5 MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 3 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 54-7 | Reserved |  |  |  |

============================== End of Proposed Text =================================

Remedy#2 :

[Change the text from Table 870 on the page 497 to Table 874 on the page 499 in section 16.3.8.2.3 as follows:]

============================== Start of Proposed Text =================================

Table 870—Mapping between UFPC and frequency partitioning for 20 MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 53 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 64 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 75 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 86 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |
| 97-15 | Reserved |  |  |  |

Table 871—Mapping between UFPC and frequency partitioning for 20 MHz>BW>10 MHz

| UFPC | Freq. Partitioning (Appro.) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | FPS1 = NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/4) |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/6) |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/8) |
| 53 | 9 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/12) |
| 64 | 9 : 5 : 5 : 5 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU*5/24) |
| 75-15 | Reserved |  |  |  |

Table 872—Mapping between UFPC and frequency partitioning for 10 MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 53 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 64 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 75 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |
| 6-7 | Reserved |  |  |  |

Table 873—Mapping between UFPC and frequency partitioning for 10 MHz>BW>5 MHz

| UFPC | Freq. Partitioning (Appro.) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | FPS1 = NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/4) |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/6) |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU/8) |
| 53 | 9 : 5 : 5 : 5 | 4 | NPRU-(FPCT-1)*FPSi (i>0) | floor(NPRU*5/24) |
| 64-7 | Reserved |  |  |  |

Table 874—Mapping between UFPC and frequency partitioning for 5 MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 32 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 3 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 4-7 | Reserved |  |  |  |

============================== End of Proposed Text =================================