| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL MIMO precoding with time diversity (16.3.10) |  |
| Date Submitted | 27/12/09 |  |
| Source(s) | Ariel Doubchak, Alvarion Erez Biton, Alvarion Yaniv Weizman, Alvarion | E-mail: ariel.doubchak@alvarion.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: LB30b / Area: Chapter 16.3.10 (UL MIMO Transmission schemes) |  |
| Abstract | This contribution suggests changing the UL precoding matrix in time to improve retransmission diversity. |  |
| Purpose | [To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to UL Precoding MIMO stream to antenna mapping (16.3.10.1.2) for the IEEE 802.16m/D3

Ariel Doubchak, Yaniv Weizman, Erez Biton

Motivation

In MIMO precoding (both open loop and closed loop), for users receiving multiple MIMO streams it is suggested to change their precoding matrix over time and by that introducing improvement in time diversity, specifically by introducing diversity gain in HARQ retransmissions.

One possible implementation is to switch between the MIMO streams with the same precoding matrix to improve HARQ retransmission diversity. If a specific stream failed to be decoded due to bad precoding vector (precoding vector created a null to the receiver) it is desired to transmit the stream on another precoding vector in the HARQ retransmission.

In open loop it is also desired to change the precoding matrix over time even for a single stream per user for the same reason.

The above recommendation can be implemented in different ways such as:

- Change stream mapping to precoding matrix per retransmission

- Change stream mapping to precoding matrix periodically

The suggested implementation is changing the stream mapping to the precoding matrix periodically (every frame – according to the expected HARQ average delay) to support different kinds of resource units (distributed, localized).

The same reasons are true for DL and UL. The following contribution is for UL.

Text Proposal

[Modify the following text in Section 16.3.10.1.2.1, page 566, line 11]

16.3.10.1.2.1 Non-adaptive precoding

There is no precoding if there is only one transmit antenna at the MS.

With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k) =W2*M, where Nt is the number of transmit antennas, Mt is the numbers of MIMO streams, and k is the physical index of the subcarrier where W(k) is applied. M is a MtxMt matrix defined in equation 273. The matrix W2 is selected from a subset of size Nw precoders of the base codebook for a given rank. W2 belongs to one of the subsets of the base codebook specified in 16.3.10.4.1, according to the type of allocation, MEF, Nt and Mt, as specified in Table 913 and Table 914.

Mi,j= δi,j+pmodMt										 (273)

Where:

p = {4*(super frame number) + (internal frame number)} mod Mt

and Internal frame number is the frame number within the super frame (range 0… 3).

δ(i,j) is the Dirac delta function defined as δi,j=1if i=j0otherwise.

[Modify the following text in Section 16.3.10.1.2.1, page 566, line 49]

In a RU allocated in a AAI subframe with MEF = 0b00 (SFBC) or 0b01 (VE) and non-adaptive precoding, the matrix W2 changes every N1PSC contiguous physical subcarriers according to Equation (273), and it does not depend on the AAI subframe number. The Nt×Mt precoding matrix W2 (k) applied on subcarrier k in physical subband s is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by

[Modify the following text in Section 16.3.10.1.2.2, page 567, line 1]

With adaptive precoding, the precoder is an Nt×Mt matrix W(k) =W2*M. M is a MtxMt matrix defined in equation 273. W2 is derived at the ABS or at the AMS, as instructed by the ABS.

Example

For Non Adaptive VE, Mt = 2, Nt = 2, the code book used is

For subband s = 0 index 0 is used according to equation 221.

With current implmentation, the precoding vector of MIMO stream 1 is therefore [0.7071 -0.7071] and the precoding vector of MIMO stream 2 is [0.7071 0.7071]. Assuming precoding vector [0.7071 -0.7071] creates a null at the receiver, stream 1 will not be detected even with HARQ retransmissions.

With the current contribution, assuming super frame number = 0 and frame number = 0 at the first transmission,  M = [1 0;  0 1], MIMO stream 1 precoding will be [0.7071  0.7071] as in the example above. Assuming the retransmission is done in the next frame (frame number =1), M = [0 1;  1 0]. MIMO stream 1 precoding vector will be [0.7071  0.7071] (not in null), improving the chances of successful decoding.