| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | General Clarification on AMS’s Capability of Supporting Femtocells (16.4) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu, Mary Chion, Hongyun Qu ZTE Corporation | E-mail: {xie.feng2, chen.lin23, lu.zhaohua, liu.yang8, qu.hongyun}@zte.com.cn mchion@zteusa.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.4” |  |
| Abstract | This contribution proposes the texts for general clarification on AMS’s capability for supporting Femtocells (16.4) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

General Clarification on AMS’s Capability of Supporting Femtocells (16.4)

Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu, Mary Chion, Hongyun Qu

ZTE Corporation

# 1. Introduction

In current P802.16m/D3, AMS’s capability of supporting femtocells needs clarification.

1) AMS shall support femtocells, thus all AMSs shall be femto-aware.

2) CSG (Close Subscriber Group) is a set of subscribers authorized by the Femto ABS owner or the network service provider. An AMS subscribed to a CSG shall be configured with a CSG White List for its accessibility check. Thus an AMS with CSG White List can be named as CSG AMS for clearance.

# 2. Text Proposal

Remedy 1:

Accept the following Modifications to 16.4.4.2 CSG White list:

Page 595, Line 53:

======================== Start of Proposed Text =====================

16.4.4.2 CSG White list

The CSG white list, is a list of Femto ABSs to which the AMSs are subscribed and can access. These femto ABSs are distinguished based on the common identifier defined in 16.4.4.

AMS's local white list may contain the allowable BS IDs or common identifiers of CSGs and relevant information to help derivation of the allowable BS IDs from common identifier.

An AMS subscribed to CSG(s), i.e. a CSG AMS, shall be configured with a CSG White List for its accessibility check. The CSG white list may be provided to the AMS by the Femto service provider through the network using messaging that is outside the scope of this standard.

============================== End of Proposed Text ===============

Remedy 2:

Accept the following Modifications 16.4.7.1 Femto ABS detection, identification and selection:

Page 599, Remove the sentences from Line 25 to 30

======================== Start of Proposed Text =====================

The above algorithm shall be applied to a Femto-Aware mobile station. For a legacy MS or a Femto-Unaware AMS, the cell search and cell selection procedures are identical to those specified in IEEE Std 802.16-2009 and this specification, respectively, and some transparent optimizations may be made that are outside the scope of this specification.

============================== End of Proposed Text ===============

Remedy 3:

Accept the following Modifications to 16.4.7.3 Femto ABS Access Restrictions:

Page 599, Line 46

======================== Start of Proposed Text =====================

16.4.7.3 Femto ABS Access Restrictions

For CSG AMS with CSG white list, it should not attempt to access or initiate handover to a CSG-closed femto ABS, which is not contained in the CSG whitelist, except in case of emergency call or manual Femto ABS selection (see 16.4.7.2). Non-CSG AMS should not attempt to access or initiate handover to a CSG-closed femto ABS, except in case of emergency call or manual Femto ABS selection.

============================== End of Proposed Text ===============

Remedy 4:

Accept the following Modifications to 16.4.9 Idle Mode:

Page 602, Line 43

======================== Start of Proposed Text =====================

16.4.7.1 Idle Mode

Femto ABS shall support idle mode.

The Femto ABSs operate like macro ABSs in Idle mode.

An AMS with CSG white list shall not attach to an unsubscribed CSG-Closed Femto ABS in Idle mode.

============================== End of Proposed Text ===============