| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Femto ABS Types (16.4.x) |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | Feng Xie, Zhaohua Lu, Mary Chion, Hongyun Qu, Yanfeng Guan, Huiying Fang ZTE Corporation Linghang Fan, NEC | E-mail: {xie.feng2, lu.zhaohua, qu.hongyun, guan.yanfeng, fang.huiying}@zte.com.cn mchion@zteusa.com Linghang.fan@eu.nec.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.4.x” |  |
| Abstract | This contribution proposes the texts for Femto ABS Types (16.4.x) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Femto ABS Types (16.4.x)

Feng Xie, Zhaohua Lu, Mary Chion, Hongyun Qu, Yanfeng Guan, Huiying Fang

ZTE Corporation

Linghang Fan

NEC

# 1. Introduction

Femto ABSs can also support enterprise use cases. In such cases, multiple CSG femto ABSs can be configured with a same CSG in order to provide services to the CSG users. On the other hand, a CSG femto ABS can be configured with one or more CSGs to provide (possibly different) services to different group of users. For example, several departments with separate CSGs for each department can share the same femto ABS.

# 2. Text Proposal

Accept the following Modifications to 16.4.x:

Page 594, Line 4:

======================== Start of Proposed Text =====================

16.4.1 General Description

A Femto ABS is an ABS with low transmit power, typically installed by a subscriber in the home, or SOHO, or enterprise to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A Femto ABS is typically connected to the service provider’s network via a broadband connection.

Femto ABSs operate in licensed spectrum and may use the same or different frequency as macro BSs. Their coverage may overlap with a macro ABS.

Femto ABS is intended to serve public users like an Open Subscriber Group (OSG), or to serve a one or more Closed Subscriber Groups (CSGs), each of that is a set of subscribers authorized by the Femto ABS owner or the network service provider. CSG can be modified by the service level agreement between the subscriber and the network service provider.

16.4.2 Femto base station subscription types

A Femto ABS may belong to one of the following subscription types:

a)	CSG-Closed Femto ABS: A CSG-Closed Femto ABS is accessible only to the AMSs, which are in its CSG(s), except for emergency services. AMSs which are not the members of the CSG(s), should not try to access CSG-Closed Femto ABSs.

b)	CSG-Open Femto ABS: A CSG-Open Femto ABS is primarily accessible to the AMS’s that belong to its CSG(s), while other AMS’s, outside CSG(s), may also access such Femto ABS, and will be served at lower priority. CSG-Open Femto ABS will provide service to such AMSs’ as long as the QoS of AMSs in its CSG(s) is not compromised.

c)	OSG (Open Subscriber Group) Femto ABS: An OSG Femto ABS is accessible to any AMS.

============================== End of Proposed Text ===============