| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Discovery and Selection Procedures for Femto ABS (16.4.7.1) |  |
| Date Submitted | 2009-01-10 |  |
| Source(s) | Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu, Mary Chion, Hongyun Qu ZTE Corporation | E-mail: {xie.feng2, chen.lin23, lu.zhaohua, liu.yang8, qu.hongyun}@zte.com.cn mchion@zteusa.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.4.7.1” |  |
| Abstract | This contribution proposes the texts for Network Discovery and Selection Procedures for Femto ABS (16.4.7.1) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network Discovery and Selection Procedures for Femto ABS (16.4.7.1)

Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu, Mary Chion, Hongyun Qu

ZTE Corporation

# 1. Introduction

In current P802.16m/D3, the procedures of network discovery and selection of femto ABSs need clarifications.

In this proposal, the procedures femto ABS detection, identification and selection are described.

# 2. Text Proposal

Accept the following Modifications:

======================== Start of Proposed Text =====================

16.4.7.1 Femto ABS detection, identification and selection

The AMS selects the target femto ABS for network entry based on the subscription types of the femto ABS and the membership to the relevant subscribers groups or accessibility to the service providers network (ex: Cell bar).

An AMS shall follow the procedure shown in Figure 574 for discovery of and association with femto ABS. An AMS is considered Femto-Preferred if it has a preference of selecting a femto ABS while macro ABS is within reach.

Figure 574: Procedure for Femto ABS Discovery and Association

+ Figure 574 only applies to macro ABSs, OSG and CSG femto ABSs; nevertheless, if the detected Cell_ID belongs to the set of CSG femto ABSs which may allow limited services to mobile stations that are not subscribed and the AMS is not a member of any CSG, the AMS may continue to detect P-SFH and S-SFH and may proceed to ACCESS STATE, if the AMS cannot be served by other neighboring macro or open OSG femto ABSs.

During network entry or re-entry, the AMS shall begin scanning of the neighbor base stations through RF measurements. The detection of the Cell_ID helps categorizing the ABS type and depending on the preference of the AMS, a macro or femto candidate is selected. Failure in any stage of the cell search and cell selection will result in repeating the scanning and DL synchronization. Cell_ID and the LSB of BS-ID (or the MSB of BS-ID if SP2 is decoded early prior to SP1) will also help the AMS to quickly sort out exclude the unsubscribed CSG home femto ABSs to which it is not subscribed. Full BSID (48 bits) or the derived full CSGID is the exact identifier for the AMS to determine whether it is authorized to access to the target home or enterprise femto ABS, respectively. The AMS shall have the common identifiers (e.g. BSID, CSGID or the CELL_ID) in its White List (i.e., a local table in the AMS containing the identities of all the CSG femto base stations to which the AMS is subscribed and is authorized to access) in order to be authorized to access to the target femto ABS.

The algorithm illustrated in Figure 574, provides the procedures that the AMS shall follow to complete cell selection/re-selection when femto and macro base stations are deployed on the same or a different FA. If the Femto base stations are deployed in a different FA (inter-FA), the same algorithm shall be applicable except that the AMS scans a different frequency band and conducts RF measurements during scanning in that frequency. The other procedures remain intact and the detected Cell_IDs will only belong to open and/or closed subscriber group femto base stations.

If the Femto and Macro base stations are deployed in the same FA (intra-FA), the same algorithm shall be applicable and the AMS conducts RF measurements during scanning in the same frequency. Combination of the inter-FA and intra-FA scanning is also possible where the neighbor macro base stations operate in the same frequency band and femto base stations operate in a different frequency band(s).

The above algorithm shall be applied to a Femto-Aware mobile station. For a legacy MS or a Femto-Unaware AMS, the cell search and cell selection procedures are identical to those specified in IEEE Std 802.16-2009 and this specification, respectively, and some transparent optimizations may be made that are outside the scope of this specification.

============================== End of Proposed Text ===============