| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on AMS scanning of Neighbor Femto ABSs (16.4.8.1.2) |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | Feng Xie, Lin Chen, Yang Liu, Mary Chion, Zhaohua Lu, Hongyun Qu ZTE Corporation | E-mail: {xie.feng2, chen.lin23, liu.yang8, lu.zhaohua, qu.hongyun}@zte.com.cn mchion@zteusa.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.4.8.1.2” |  |
| Abstract | This contribution proposes the texts for AMS scanning procedures for neighbor Femto ABSs (16.4.8.1.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on AMS Scanning of Neighbor Femto ABSs (16.4.8.1.2)

Feng Xie, Lin Chen, Yang Liu, Mary Chion, Zhaohua Lu, Hongyun Qu

ZTE Corporation

# 1. Introduction

In current P802.16m/D3, the procedures of AMS scanning of femto ABSs need clarifications. There are too many features and options. We need to specify the scenarios for the use of those features.

In this proposal, the procedures of AMS scanning of femto ABS are described.

# 2. Text Proposal

Accept the following Modifications for 16.4.8.1.2 AMS scanning of neighbor femto ABSs:

======================== Start of Proposed Text =====================

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

For neighbor Femto ABSs, an AMS performs the scanning procedure as per 16.3.6.1.2 with exceptions described in this subsection. <<Note to editor: insert new paragraph here>>

For intra-frequency scanning of neighbor femto ABSs, an AMS detects the existence of neighbor femto ABSs by performing intra-frequency preamble measurement. Note that the intra-frequency preamble measurement does not interrupt AMS communication with the serving ABS. If the detected CELL_ID matches with the CELL_ID range of femto ABSs indicated in S-SFH SP3 and AAI_SCD message, the AMS has detected a neighbor femto ABS and is able to further distinguish OSG, CSG-Closed, CSG-Open femto ABSs. If a CSG femto ABS is detected, the AMS can quickly exclude inaccessible CSG femto ABSs if the detected CELL_ID is not included in AMS’s CSG White List.

With detected CELL_ID of accessible femto ABSs, an AMS should negotiate with the serving ABS for a scanning interval to decode the SFH and/or CSGID Info of neighbor femto ABSs. Upon receiving the BSID and/or CSGID(s) of neighbor CSG femto ABSs, AMS shall check its accessibility with the CSG White List. If the decoded BSID and/or CSGID(s) is included in CSG White List, AMS can report to the serving ABS and/or initiate HO with the decoded BSID. Detailed procedures for detecting and selecting OSG or CSG femto ABSs can be found in Figure 574.

An AMS may scan femto ABSs according to the FAs included in the broadcast AAI_NBR-ADV message. AMS may scan femto ABSs that are not included in AAI_NBR-ADV based on SA-preamble partitioning information (see 16.4.4). In addition, an AMS may scan allowed femto ABSs based on the CSG White List. Based on location information, AMS may initiate the scanning procedure (See 16.2.6.1.2). The AMS may request additional scanning opportunity by sending AAI-SCN-REQ including the detected SA-preamble index and FA information. Upon reception of the AAI_SCN-REQ, the ABS shall respond with an AAI_SCN-RSP including neighbor accessible Femto ABS list based on the SA-preamble index.

Alternatively, an overlapped Macro ABS may recommend CSG Femto ABS to monitor UL signaling of accessible AMS which is served by the Macro ABS. If CSG Femto ABS is in low-duty mode, the CSG Femto ABS can try to receive A-MAP information from overlapped Macro ABS and then monitor accessible AMS's UL signaling in associated allocations. If CSG Femto ABS is not in low-duty mode, overlapped Macro ABS may trigger dedicated periodic ranging (see 15.4.7.4.2) for the AMS. A CSG Femto ABS can monitor the ranging preamble at the dedicated ranging slot of Macro ABS. When the received signaling quality (e.g RSSI) of an AMS is stronger than a threshold, CSG Femto ABS can request overlapped Macro ABS to send unsolicited AAI_SCN-RSP for the AMS to scan the CSG Femto ABS.

When the AMS has to request for information of neighboring Femto ABSs belonging to a CSG then the AMS can provide the desired CSGID(s) in the AAI_SCN-REQ message to the serving ABS. The ABS can respond with a list of ABSs, addressed by BSID belonging to the requested CSGID(s) in AAI_SCN-RSP message.

ABS may send an unsolicited AAI_SCN-RSP for the AMS to scan the Femto ABS.

After scanning, the AMS may report preamble indices or BS IDs by sending AAI_NBR-REQ or AAI-SCN-REP with neighbor request indication, to which the ABS may unicast an AAI_NBR-ADV that includes a list of femto ABSs which is formed based on the reported preamble index or any other recommended Femto ABSs based on location information.

Upon identifying the existence of any Femto ABSs, the AMS may transmit an AAI_SCN-REP message including the list of the detected Femto ABSs and measurement results according to the Trigger conditions included in the AAI_SCD message.

If the AMS decides to perform HO to any Femto ABS, it may request more detailed system information of the detected neighbor Femto ABSs to the serving ABS using an AAI_NBR-REQ message.

AMS may include the CSGID(s) of the subscribed CSG femto BSs in the AAI_NBR-REQ message.

============================== End of Proposed Text ===============