| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Re-entry from Idle Mode in Case of ABS Cell Bar (16.2.17) |  |
| Date Submitted | 2009-01-06 |  |
| Source(s) | Feng Xie, Mary Chion, Hongyun Qu, Yang Liu, Lei Zhang，Zhaohua Lu, Lin Chen ZTE Corporation Daniel Cohn, Alvarion | E-mail: {xie.feng2, qu.hongyun, liu.yang8, zhang.lei13, lu.zhaohua, chen.lin23}@zte.com.cn mchion@zteusa.com daniel.cohn@alvarion.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.17” |  |
| Abstract | This contribution proposes the texts for Re-entry from Idle Mode in Case of ABS Cell Bar |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Re-entry from Idle Mode in Case of ABS Cell Bar (16.2.17)

Feng Xie, Mary Chion, Hongyun Qu, Yang Liu, Lei Zhang, Zhaohua Lu, Lin Chen

ZTE Corporation

Daniel Cohn

Alvarion

# 1. Introduction

In current P802.16m/D3, cell bar information in S-SFH IE SP1 is set to 1 to prevent AMS from entering the network. However, network re-entry in such cases should also be prohibited.

In this proposal, the Idle Mode re-entry procedures in case of ABS cell bar are described.

# 2. Text Proposal

Accept the following modification for 16.2.17.3 Idle mode termination

======================== Start of Proposed Text =====================

16.2.17.3 Idle mode termination

Idle mode may only be terminated through

•AMS reentry to the network

•Paging controller detection of AMS unavailability through repeated, unanswered paging messages

•Expiration of the idle mode timer

An AMS may terminate idle mode at any time. For the termination of the idle mode, the AMS performs network

reentry with its preferred ABS as described in section 15.2.x.5. If the preferred ABS is cell barred, AMS should re-select a neighbor preferred ABS for network re-entry.

============================== End of Proposed Text ===============