| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC support for Multi-BS operation for the IEEE 802.16m (section 16.2.3.31) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Inuk Jung, Wookbong Lee, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. Shuangfeng Han, David Mazzarese Samsung Electronics | +82-31-450-1856 [inuk.jung, wookbong.lee] @lge.com shuangf.han@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #30b |  |
| Abstract | Text proposal for Multi-BS MIMO section |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC support for Multi-BS operation for the IEEE 802.16m (section 16.2.3.31)

Inuk Jung, Wookbong Lee, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

Shuangfeng Han, David Mazzarese

Samsung Electronics

- Introduction

This contribution includes proposed text to provide MAC support for Multi-BS Co-MIMO in the IEEE802.16m D3 document [1].

- Rationale/Motivation of modifications

The Temp_BSID may be derived in a very simply manner. Referring to the BS_index, the Temp_BSID represents the ABS in orderly manner of the BS_Index included in the AAI_DL-IM Message. Hence, Temp_BSID 1 will be the ABS that is represented by the first BS_Index that appears in the AAI_DL-IM message. The Temp_BSID will be assigned autonomously.

- References

[1] IEEE P802.16m/D3, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

On the 80216m_D2, page 96, line 19, add current text in 16.2.3.31 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.31 AAI_DL-IM Message

An AAI_DL_IM broadcast DL FFR partition resource metric, BC_SI, NIP_threshold, power level for each frequency partition.

-- ASN1START

AAI_DL_IM Message ::= SEQUENCE {

FFR partition resource metric :: = SEQUENCE {

Resource_Metric_FP2		INTEGER 	(TBD) 	OPTIONAL

Resource_Metric_FP3		INTEGER 	(TBD) 	OPTIONAL

}

BC_SI		INTEGER 	(0..15) 	OPTIONAL

NIP_th_1	INTEGER 	(TBD) 	OPTIONAL

NIP_th_2	INTEGER 	(TBD) 	OPTIONAL

FP1_power	INTEGER 	(TBD) 	OPTIONAL

FP2_power	INTEGER 	(TBD) 	OPTIONAL

FP3_power	INTEGER 	(TBD) 	OPTIONAL

Diversity_Set_NUM		INTEGER	(0..15)		OPTIONAL

Change_count 		INTEGER	(0..255)	OPTIONAL

Temp_BSID_SET :: = SEQUENCE (SIZE (1..Diversity_Set_NUM)) OF INTEGER BS_index (0..255) OPTIONAL  OF {

BS_indexINTEGER 		(0..255) 	OPTIONAL

}

}

-- ASN1STOP

Table 704—Parameters for AAI_DL_IM Message

| Name | Value | Usage |
| --- | --- | --- |
| Resource_Metric_FP2 | TBD |  |
| Resource_Metric_FP3 | TBD |  |
| BC_SI | 8 bits for Nt = 2 and 16 bits for Nt = 4 or 8 |  |
| NIP_th_1 | TBD |  |
| NIP_th_2 | TBD |  |
| FP1_power | TBD |  |
| FP2_power | TBD |  |
| FP3_power | TBD |  |
| Diversity_Set_NUM | 4 | The number of ABSs coordinating with the serving ABS for single BS precoding with multi-BS coordination. |
| Change_count | 8 | Change count in AAI_NBR-ADV for the following BS_index |
| BS_index | 8 | Indicates diversity set members Refers to the ABS index of the AAI_NBR_ADV. Temp_BSID is derived from this BS_index. by TBD manner The Temp_BSID is the order of the ABSs in respect of the BS_index. The first Temp_BSID represents the ABS that is referred by the first BS_index in AAI_DL_IM message. |

--------------------------------------------------------Text End -----------------------------------------------------