| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC support for Multi-BS operation for the IEEE 802.16m (section 16.2.3.31) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Inuk Jung, Wookbong Lee, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. Shuangfeng Han, David Mazzarese Samsung Electronics Yuan Zhu, Hujun Yin Intel Corporation Luciano Sarperi, Hui Xiao Fujitsu | +82-31-450-1856 [inuk.jung, wookbong.lee] @lge.com shuangf.han@samsung.com yuan.y.zhu@intel.com luciano.sarperi@uk.fujitsu.com hui.xiao@uk.fujitsu.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #30b |  |
| Abstract | Text proposal for Multi-BS MIMO section |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC support for Multi-BS operation for the IEEE 802.16m (section 16.2.3.31)

Inuk Jung, Wookbong Lee, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

Shuangfeng Han, David Mazzarese

Samsung Electronics

Yuan Zhu, Hujun Yin

Intel Corporation

Luciano Sarperi, Hui Xiao

Fujitsu

- Introduction

This contribution includes proposed text for AAI_DI_IM message to make it complete in IEEE802.16m D3 document [1].

- References

[1] IEEE P802.16m/D3, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

-------------------------------------------------Text Change #1 Start -----------------------------------------------------

Replace the whole section 16.2.3.31 AAI_DL_IM Message as below

16.2.3.31 AAI_DL_IM Message

An AAI_DL_IM broadcast DL FFR partition resource metric, BC_SI, NIP_threshold, power level for each frequency partition.

-- ASN1START

AAI_DL_IM Message ::= SEQUENCE {

FFR partition resource metric :: = SEQUENCE {

Resource_Metric_FP2		INTEGER 	(TBD) 	OPTIONAL

Resource_Metric_FP3		INTEGER 	(TBD) 	OPTIONAL

}

BC_SI   INTEGER 	(0..15) 	OPTIONAL

NIP_th_1	INTEGER 	(TBD) 	OPTIONAL

NIP_th_2	INTEGER 	(TBD) 	OPTIONAL

FP1_power	INTEGER 	(TBD) 	OPTIONAL

FP2_power	INTEGER 	(TBD) 	OPTIONAL

FP3_power	INTEGER 	(TBD) 	OPTIONAL

Diversity_Set_NUM		INTEGER	(0..15)		OPTIONAL

Change_count 		INTEGER	(0..255)	OPTIONAL

Temp_BSID_SET :: = SEQUENCE OF {

BS_indexINTEGER 		(0..255) 	OPTIONAL

}

}

-- ASN1STOP

Table 704—Parameters for AAI_DL_IM Message

| Name | Value | Usage |
| --- | --- | --- |
| Resouce_Metric_FP2 | TBD |  |
| Resouce_Metric_FP3 | TBD |  |
| BC_SI | 8 bits for Nt=2 and 16 bits for Nt =4 or 8 |  |
| NIP_th_1 | TBD |  |
| NIP_th_2 | TBD |  |
| FP1_power | TBD |  |
| FP2_power | TBD |  |
| FP3_power | TBD |  |
| Diversity_Set_NUM | 4 | The number of ABSs coordinating with the serving ABS for single BS precoding with multi-BS coordination. |
| Change_count | 8 | Change count in AAI_NBR-ADV for the following BS_index |
| BS_index | 8 | Indicates diversity set members Refers to the ABS index of the AAI_NBR_ADV. Temp_BSID is derived from this BS_index. by TBD manner |

The AAI_DL_IM broadcast message may be transmitted by the ABS to define FFR and multi-BS MIMO parameters.

AAI_DL_IM ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

-- ffrPartitionResouceMetric to fp3Power are only transmitted when FPCT > 1

-- resourceMetricFP2 is transmitted only when FPS2 >0

resourceMetricFP2            INTEGER (0..15) OPTIONAL,

-- resourceMetricFP3 is transmitted only when FPS3>0

resourceMetricFP3            INTEGER (0..15) OPTIONAL,

fp2Power                       INTEGER (0..31) OPTIONAL,

-- fp3Power is transmitted only when FPS3>0

fp3Power                       INTEGER (0..31) OPTIONAL,

-- bsSiInfo to tempBsIdSET are transmitted only when multi-BS MIMO is operated

bcSiInfo                       CHOICE {

-- in case of two transmit antenna, refer 16.3.7.2.5.6

twoTxBCSI                    BIT STRING (SIZE (8)),

-- in case of four or eight transmit antenna, refer 16.3.7.2.5.6

otherTxBCSI                  BIT STRING (SIZE (16))

} OPTIONAL,

nipTh1                         INTEGER (0..15) OPTIONAL,

nipTh2                         INTEGER (0..15) OPTIONAL,

cinrTh                         INTEGER (0..15) OPTIONAL,

diversitySetNum                INTEGER (0..15) OPTIONAL,

-- Change count in AAI_NBR-ADV for the following BS_index. This

-- is only transmitted only when diversitySetNum is higher than 0

changeCount            INTEGER (0..255) OPTIONAL,

-- tempBsidSET is composed of diversitySetNum number of BS_indices.

-- tempBsidSET is only transmitted only when diversitySetNum is

-- higher than 0

tempBsidSET                    SEQUENCE {

bsidSet1                     INTEGER (0..255) OPTIONAL,

bsidSet2                     INTEGER (0..255) OPTIONAL,

bsidSet3                     INTEGER (0..255) OPTIONAL,

bsidSet4                     INTEGER (0..255) OPTIONAL,

bsidSet5                     INTEGER (0..255) OPTIONAL,

bsidSet6                     INTEGER (0..255) OPTIONAL,

bsidSet7                     INTEGER (0..255) OPTIONAL,

bsidSet8                     INTEGER (0..255) OPTIONAL,

bsidSet9                     INTEGER (0..255) OPTIONAL,

bsidSet10                    INTEGER (0..255) OPTIONAL,

bsidSet11                    INTEGER (0..255) OPTIONAL,

bsidSet12                    INTEGER (0..255) OPTIONAL,

bsidSet13                    INTEGER (0..255) OPTIONAL,

bsidSet14                    INTEGER (0..255) OPTIONAL,

bsidSet15                    INTEGER (0..255) OPTIONAL}

}

Table 704—Parameters for AAI_DL_IM Message

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| resourceMetricFP2 | 4 | Resource_Metric_FP2 Resource Metric of the first power de-boosted frequency partition which is defined in Table 761. There is no this field if FPCT=1 which is defined in Table 777 to Table 781 |
| resourceMetricFP3 | 4 | Resource_Metric_FP3 Resource Metric of the second power de-boosted frequency partition which is defined in Table 761. There is no this filed if FPS3=0 which is defined in Table 777 to Table 781 |
| fp2Power | 5 | FP2_power[dB] = 0b00000: -Inf Otherwise -10+(fp2Power-1)*0.5 |
| fp3Power | 5 | FP3_power[dB] = 0b00000: -Inf Otherwise -10+(fp3Power-1)*0.5 |
| bcSiInfo | 8 bits for Nt=2 and 16 bits for Nt =4 or 8 | BC_SI If LSB#n is ‘1’, the nth codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. The code books or its subset are defined in section 16.3.7.2.5.6 for Nt=2, 4 or 8 respectively. |
| nipTh1 | 4 | NIP_th_1 The value shall be interpreted as unsigned 4bits with units of 0.5 dB, 0b0000 is interpreted as -7.5dB and 0b1111 is interpreted as 0.0dB NIP threshold for Single BS precoding with Multi-BS Coordination trigger |
| nipTh2 | 4 | NIP_th_2 The value shall be interpreted as unsigned 4bits with units of 0.5 dB, 0b0000 is interpreted as -7.5dB and 0b1111 is interpreted as 0.0dB NIP threshold for DL Multi-BS Joint MIMO Processing trigger |
| cinrTh | 4 | CINR_th The value shall be interpreted as signed 4bits with units of 0.5 dB, 0b1111 is interpreted as -4.0dB and 0b0111 is interpreted as 3.5dB Used together with NIP_th_1 or NIP_th_2 for Multi-BS MIMO trigger |
| diversitySetNum | 4 | The number of ABSs coordinating with the serving ABS for single BS precoding with multi-BS coordination. |
| changeCount | 8 | Change count in AAI_NBR-ADV for the following BS_index |
| tempBsidSET | 8 | Indicates diversity set members Refers to the ABS index of the AAI_NBR_ADV. Temp_BSID is derived from this BS_index. The Temp_BSID is the order of the ABSs in respect of the BS_index. The first Temp_BSID represents the ABS that is referred by the first BS_index in AAI_DL_IM message. |

---------------------------------------------------Text Change #1 End -----------------------------------------------------

---------------------------------------------------Text Change #2 Starts ----------------------------------------------------

Add the default power loading values from section 16.2.3.31 AAI_DL_IM Message to SFH SP3 in page 381 line 48.

Add one row at the end of SFH SP3

| fpPowerConfig | 4 | Refer to Table XXX |
| --- | --- | --- |

fpPowerConfig: The power boosting/de-boosting values are listed in table XXX.

Table XXX—Power boosting/de-boosting values

| fpPowerConfig | FPCT=3 or 4 and FPSi>0 for 1≤i≤3 |  |  | FPCT=2 or 3 and FPS3=0 |  |
| --- | --- | --- | --- | --- | --- |
|  | FP1_power(dB) | FP2_power(dB) | FP3_power(dB) | FP1_power(dB) | FP2_power(dB) |
| 0b000 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 0b001 | 1.0 | -0.5 | -0.5 | 0.5 | -0.5 |
| 0b010 | 2.0 | -1.5 | -1.5 | 1.0 | -1.5 |
| 0b011 | 2.5 | -2.0 | -2.0 | 1.5 | -2.5 |
| 0b100 | 3.0 | -3.0 | -3.0 | 2.0 | -4.5 |
| 0b101 | 3.5 | -4.5 | -4.5 | 2.5 | -6.0 |
| 0b110 | 4.0 | -6.0 | -6.0 | 3.0 | -inf |
| 0b111 | 4.75 | -inf | -inf | reserved | reserved |

The power loading values of the power de-boosted partition will be overwritten by fp2Power and fp3Power in AAI_DL_IM message

---------------------------------------------------Text Change #2 Ends -----------------------------------------------------