| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the CDMA Allocation A-MAP IE (D3 - 16.2.11.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes some missing fields into CDMA Allocation A-MAP IE |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the CDMA Allocation A-MAP IE (D3 - 16.2.11.2)

Heejeong Cho, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the CDMA Allocation A-MAP IE in the IEEE 802.16m DRAFT Amendment [1]. The CDMA Allocation A-MAP IE has some missing or ambiguous explanation.

- Modifications and key-descriptions

- Clarification of the CDMA Allocation A-MAP IE

  - Resource Assignment Information

    - Resource Assignment Information field is in the CDMA Allocation A-MAP IE, but it hasn’t clearly defined.

    - Therefore, we propose that the field should be removed and Resource Index, Long TTI indicator and ISizeOffset (ranging only) should be added.

- A-MAP IE for resource assignment of AAI_RNG-RSP

  - An AAI_RNG-RSP including a Temp-STID is transmitted in response to the AAI_RNG-REQ when an ABS doesn’t assign TSTID to an AMS.

  - What identifier is CRC in an A-MAP IE for the AAI_RNG-RSP masked by?

    - Method 1 : To use Broadcast / Multicast STID

      - If DL basic assignment A-MAP IE is reused, AMS should decode all of the AAI_RNG-RSPs.

      - If A-MAP IE including random access attributes is newly designed, AMS should decode the AAI_RNG-RSP if the random access attributes in the A-MAP IE is the same as those of the AMS.

    - Method 2 : To use RAID

      - AMS should decode the AAI_RNG-RSP if the CRC in the A-MAP IE is masked by its own RAID.

      - It is efficient to reuse the CDMA Allocation A-MAP IE if method 2 is used.

      - We need to add Uplink/Downlink indicator field in the CDMA Allocation A-MAP IE.

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

On the IEEE P802.16m/D3, page 209, line 11, make the following changes to subclause 16.2.11.2

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.11.2 Grant

In the contention-based random access bandwidth request procedure, the grant for BW-REQ message BR header is allocated by CDMA Allocation A-MAP IE, which is shown in Table 743. CDMA Allocation A-MAP IE is used for allocation of DL/UL bandwidth to a user that requested bandwidth using a ranging preamble code or BR code preamble sequence. AMS decodes the IE and checks the MCRC field by its specific 12-bit RA-ID and 4-bit Masking Indicator. The RA-ID is calculated by a hash function with the AMS' random access attributes (frame number index (4 bits), ranging code/BR code preamble index (6 bits) and opportunity index (2 bits) as defined below:

RA-ID = (frame_index | ranging_code preamble_index | opportunity_index)

Masking indicator indicates the identifier used for CRC masking, as shown in Table 744. The allocation shall be in the first UL AAI subframe relevant to the A-MAP region regardless of DL/UL ratio and NAAI subframe, A-MAP. The maximum number of the HARQ retransmission of the data (e.g., BR header and AAI_RNG-REQ/RSP) sent through the given resource shall be limited to is TBD.

Table 743—CDMA Allocation A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  | — |
| A-MAP IE type | 4 | CDMA Allocation A-MAP IE |
| If MCRC is masked with RAID and masking indicator for BR { |  |  |
| Resource Assignment Information Resource Index | TBD11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Reserved | TBD |  |
| } |  |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging { |  |  |
| Uplink/Downlink Indicator | 1 | Indicates whether the following fields are for resource assignment in the uplink or in the downlink. 0b0: Uplink 0b1: Downlink |
| Resource Assignment Information Resource Index | TBD11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| ISizeOffset | 5 | Offset used to compute burst size index |
| HFA | 3 | HARQ Feedback Allocation |
| Reserved | TBD |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | 16 | CRC masked by RA-ID and Masking Indicator |
| } |  | — |

-----------------------------------------------------------End of the Text---------------------------------------------------------