| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the unsolicited transmission of PHY corrections (D3 - 16.2.2.1.3) |  |
| Date Submitted | 2010-01-09 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, HyunWoo Lee, Eunjong Lee, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes text for method to transmit PHY corrections. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the Unsolicited Transmission of PHY Corrections (D3 - 16.2.2.1.3)

Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, HyunWoo Lee, Eunjong Lee,

Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the unsolicited transmission of PHY adjustment information in the IEEE 802.16m DRAFT Amendment [1]. An AAI_RNG-ACK message is sent by ABS in response to the CDMA ranging request. It can be also transmitted asynchronously to specific user in order to provide PHY corrections. However, AAI_RNG-ACK is designed only as response to the CDMA ranging request. In case of transmission in unsolicited manner, ABS does not have to send RNG-ACK bitmap, number of received codes and code index fields. Some necessary fields (ranging status, timing offset adjustment, power level adjustment and frequency offset adjustment) are only included on the specific condition (number of received code > 0 & ranging status == continue). Format of the AAI_RNG-ACK should be modified.

Furthermore, ABS may send a response message for the received UL data or MAC management message because the PHY corrections are based on measurements that have made on other received data or MAC management message. If there are some PHY corrections in that case, ABS should transmit both response message and AAI_RNG-ACK. When two MAC management messages (e.g., other message and unsolicited AAI_RNG-ACK) are sent at the same time, total length of the MAC PDU (except the other message) is 15 bytes, as shown in Figure 1. However, if ABS transmits the PHY corrections through an extended header, we can save UL bandwidth (minimum 2 LRUs in Table 1). That’s because it is not necessary to add the followings:

- In case of multiplexing, one MEH (3 bytes) for multiplexing two messages, one MCEH (1 byte) and message type (1 byte) for AAI_RNG-ACK;

- In case of concatenating, one AGMH (2 bytes), extended header group length (1 byte), MCEH (1 byte), message type (1 byte) and CRC (2 bytes) for AAI_RNG-ACK.

Table 1 – The number of required RUs according to each method to transmit PHY corrections in unsolicited manner

| MAC PDU Isizeoffset | Using AAI_RNG-ACK - Multiplexing |  |  |  |  |  | Using Extended Header (Proposal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Header | AAI_RNG-ACK | Other message | CRC | Total length | RU# | Header | Other message | CRC | Total length | RU# |
| 0 | 10 | 5 | 10 | 2 | 27 | 13 | 10 | 10 | 2 | 22 | 10 or 11 |
| 1 | 10 | 5 | 10 | 2 | 27 | 12 | 10 | 10 | 2 | 22 | 9 |
| 2 | 10 | 5 | 10 | 2 | 27 | 10 or 11 | 10 | 10 | 2 | 22 | 8 |

| MAC PDU Isizeoffset | Using AAI_RNG-ACK - Concatenating |  |  |  |  |  | Using Extended Header (Proposal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Header | AAI_RNG-ACK | Other message | CRC | Total length | RU# | Header | Other message | CRC | Total length | RU# |
| 0 | 10 | 5 | 10 | 4 | 29 | 14 or 15 | 10 | 10 | 2 | 22 | 10 or 11 |
| 1 | 10 | 5 | 10 | 4 | 29 | 13 | 10 | 10 | 2 | 22 | 9 |
| 2 | 10 | 5 | 10 | 4 | 29 | 12 | 10 | 10 | 2 | 22 | 8 |

Figure 1 - Example of the MAC PDU formats when ABS sends PHY corrections with other MAC management message - Multiplexing

Although an unsolicited AAI_RNG-ACK is only sent, total length of the MAC PDU is 9 bytes, as shown in Figure 2. However, if ABS transmits the PHY corrections through a signaling header, we can save UL bandwidth (minimum 2 LRUs in Table 2).

Table 2 – The number of required RUs according to each method to transmit PHY corrections in unsolicited manner

| MAC PDU Isizeoffset | Using AAI_RNG-ACK |  |  |  |  | Using Signaling Header (Proposal) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Header | AAI_RNG-ACK | CRC | Total length | RU# | Header | CRC | Total length | RU# |
| 0 | 4 | 5 | 2 | 11 | 6 | 6 | 2 | 8 | 4 |
| 1 | 4 | 5 | 2 | 11 | 5 | 6 | 2 | 8 | 1~3 |

Figure 2 - Example of the MAC PDU formats when ABS sends only PHY corrections

Therefore, we propose a signaling header and extended header for transmission of the PHY corrections based on measurements that have made on other received data or MAC management message.

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D3, page 22, line 34, make the following changes to subclause 16.2.2.1.3]

16.2.2.1.3 MAC Signaling Header

Table 656—Type field encodings for MAC signaling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR with STID |
| 0001 | BR without STID |
| 0010 | Service specific BR without STID |
| 0011 | Sleep Control |
| 0100 | PHY Corrections |
| 01001- 1111 | Reserved |

[On the IEEE P802.16m/D3, page 27, line 22, add the following changes to subclause 16.2.2.1.3]

16.2.2.1.3 MAC Signaling Header

16.2.2.1.3.5 PHY Corrections Header (PCH)

PHY Corrections Header is sent in unsolicited manner to the specific AMS, which needs to provide PHY-level adjustments (e.g., timing offset, power level and frequency offset) based on measurements that have made on other received data or MAC management message. As a result, the AMS shall be prepared to receive a PCH at any time, not just following a ranging preamble code transmission. Upon receiving a PCH with continue status, the AMS shall adjust its PHY parameters in the PCH and perform the ranging process with a periodic ranging preamble code randomly chosen. Upon receiving a PCH with success status, the AMS shall adjust its PHY parameters in the PCH and restart timer T4. The PHY Corrections Header format is defined in Table xxx.

Table xxx—PHY Correction Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PHY Corrections header() { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type = 0b0100 |
| Ranging request | 1 | If receipt of AAI-RNG-CFM is required, this field is set to ‘0b1’. |
| Status | 1 | 0b0 : success, 0b1 : continue |
| Timing offset adjustment | 11 | Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs. |
| Power level adjustment | 3 | Relative change in transmission power level. Signed in units of 1 dB |
| Frequency offset adjustment | 7 | Relative change in transmission frequency. The correction is 0.1% of the subcarrier spacing (Δf) multiplied by the 7-bit number interpreted as a signed integer. |
| Padding | variable |  |
| } |  |  |

[On the IEEE P802.16m/D3, page 28, line 29, make the following changes to subclause 16.2.2.2]

16.2.2.2 Extended header formats

Table 662—Description of Extended Header Types

| Extended Header Types | Names | Description |
| --- | --- | --- |
| - 0b0000 | Fragmentation and packing extended header | See 16.2.2.2.1 |
| - 0b0001 | MAC Control extended header | See 16.2.2.2.2 |
| 0b0000 0b0010 | Multiplexing extended header | See 16.2.2.2.3 |
| 0b0001 0b0011 | Message ACK extended header | See 16.2.2.2.4 |
| 0b0010 0b0100 | Sleep Control extended header | See 16.2.2.2.5 |
| 0b0011 0b0101 | Correlation matrix feedback extended header | See 16.2.2.2.6 |
| 0b0100 0b0110 | MIMO feedback extended header | See 16.2.2.2.7 |
| 0b0101 0b0111 | Piggybacked bandwidth request extended header | See 16.2.2.2.8 |
| 0b1000 | MAC PDU length extended header | See 16.2.2.2.9 |
| 0b1001 | PHY Corrections extended header | See 16.2.2.2.10 |
| 0b01100b1010-0b1111 | Reserved |  |

[On the IEEE P802.16m/D3, page 37, line 64, make the following changes to subclause 16.2.2.2]

16.2.2.2 Extended header formats

16.2.2.2.10 PHY Corrections extended header (PCEH)

PHY Corrections Extended Header is added to MAC PDU when PHY-level adjustments (e.g., timing offset, power level and frequency offset) are sent in unsolicited manner. As a result, the AMS shall be prepared to receive a PHY Corrections Extended Header at any time, not just following a ranging preamble code transmission. Upon receiving a PCEH with continue status, the AMS shall adjust its PHY parameters in the PCEH and perform the ranging process with a periodic ranging preamble code randomly chosen. Upon receiving a PCEH with success status, the AMS shall adjust its PHY parameters in the PCEH and restart timer T4. The PHY Corrections Extended Header format is defined in Table xxx.

Table xxx—PHY Corrections Extended Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PCEH () { |  |  |
| Type | 4 | Extended header type = 0b1001 |
| Ranging request | 1 | If receipt of AAI-RNG-CFM is required, this field is set to ‘0b1’. |
| Status | 1 | 0b0 : success, 0b1 : continue |
| Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication |
| If (API bit #0 == 1) { |  |  |
| Timing offset adjustment } | 11 | Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs. |
| If (API bit #1 == 1) { |  |  |
| Power level adjustment } | 3 | Relative change in transmission power level. Signed in units of 1 dB |
| If (API bit #2 == 1) { |  |  |
| Frequency offset adjustment } | 7 | Relative change in transmission frequency. The correction is 0.1% of the subcarrier spacing (Δf) multiplied by the 7-bit number interpreted as a signed integer. |
| Padding | variable |  |
| } |  |  |

[On the IEEE P802.16m/D3, page 46, line 40, make the following changes to subclause 16.2.3.2]

16.2.3.2 AAI_RNG-RSP

An AAI_RNG-RSP, to which HARQ operation is applied, shall be transmitted by the ABS in response to a received AAI_RNG-REQ. In addition, it may also be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. As a result, the AMS shall be prepared to receive an AAI_RNG-RSP at any time, not just following an AAI_ RNG-REQ transmission.

[On the IEEE P802.16m/D3, page 48, line 35, make the following changes to subclause 16.2.3.3]

16.2.3.3 AAI_RNG-ACK

The AAI_RNG-ACK message is sent by the ABS in response to the CDMA ranging request during initial ranging, period ranging and HO ranging in order to provide PHY-level adjustment (e.g. timing offset, power level and frequency offset). Upon reception of ranging codes, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE using broadcast STID. When the AAI_RNG-ACK message is sent unsolicitedly to specific user, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE with unicast STID.

[On the IEEE P802.16m/D3, page 113, line 33, make the following changes to subclause 16.2.3.42]

16.2.3.42 AAI-RNG-CFM

The AMS shall send the AAI-RNG-CFM message to the ABS upon successful ranging initiated by an unsolicited PHY Corrections signaling header or PHY Corrections extended header AAI-RNG-RSP message with the Ranging Request bit to be one.

-----------------------------------------------------------End of the Proposal Text ---------------------------------------------------------