| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the CDMA-based Ranging Procedure (D3 - 16.2.3.11) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Heejeong Cho, HyunWoo Lee, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes CDMA-based ranging procedure. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the CDMA-based Ranging Procedure (D3 - 16.2.3.11)

Heejeong Cho, HyunWoo Lee, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the ranging procedure in the IEEE 802.16m DRAFT Amendment [1]. Ranging is a collection of processes by which the AMS and ABS maintain the quality of the RF communication link between them. It consists of three procedures — initial ranging, periodic ranging and handover ranging. Initial ranging allows an AMS joining the network to acquire correct transmission parameters, such as time offset and power level, so that the AMS can communicate with the ABS. Periodic ranging allows the AMS to adjust transmission parameters so that the AMS can maintain UL communications with the ABS. Handover ranging is allows an AMS re-entering the network to acquire correct transmission parameters so that the AMS can communicate with the target ABS. If the CDMA_RNG_FLAG in AAI_HO-CMD is set to 1, the AMS shall perform the CDMA-based handover ranging during network re-entry. In order to minimize handover interruption time [2], the ABS needs to transmit AAI_RNG-ACK for the handover ranging as soon as possible.

- Modifications and key-descriptions

- HO ranging procedure

  - HO preparation phase

    - Target ABS informs AMS of transmission timing of AAI_RNG-ACK (RNG_ACK_TX_Time) via the Serving ABS through the AAI_HO-CMD.

  - HO Execution phase

    - AMS behavior

      - AMS transmits a HO ranging preamble code.

      - If AMS does not receive any AAI_RNG-ACK or CDMA Allocation A-MAP IE until RNG_ACK_TX_Time is expired, AMS considers that it is an implicit-NACK.

      - If AMS does not find in the AAI_RNG-ACK message that at least one ranging preamble code has been successfully received in the ranging opportunity selected by the AMS or HO ranging preamble code selected by the AMS has been successfully received in the ranging opportunity, AMS considers that it is an explicit-NACK.

    - ABS behavior

      - If detecting at least one handover ranging preamble code in a frame, target ABS should send an AAI_RNG-ACK in response to the detected handover ranging preamble code(s) within RNG_ACK_TX_Time.

      - However, if ABS assigns UL bandwidth before sending the response, ABS can omit to transmit the response.

Figure 1 - Proposed CDMA-based HO ranging procedure when RNG_ACK_TX_Time is sent to an AMS

- Initial / periodic ranging procedure

  - Differences between HO ranging and initial/periodic ranging

    - RNG_ACK_TX_Time is not sent. (MAX_RNG_ACK_TX_Time ≥ RNG_ACK_TX_Time)

    - If AMS does not receive any AAI_RNG-ACK until MAX_RNG_ACK_TX_Time is expired, AMS considers that it is an implicit-ACK.

Figure 2 - Proposed CDMA-based initial/periodic ranging procedure

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] IEEE 802.16m-07/002r9, “IEEE 802.16m System Requirements”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D3, page 62, line 1, make the following changes to subclause 16.2.3.11]

16.2.3.11 AAI_HO-CMD

Table 683—Parameters for AAI_HO-CMD message

| Name | Value |  | Usage |
| --- | --- | --- | --- |
| Action Time | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number |  | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| CDMA_RNG_FLAG |  | Shall be included for each target ABS when Mode=0b00.Shall be included when Mode = 0b01. |  |
| Dedicated CDMA ranging code/opportunity |  | May be included when CDMA_RNG_FLAG=1 |  |
| RNG_ACK_TX_Time | Indicates that target ABS transmits AAI_RNG-ACK within RNG_ACK_TX_Time from the frame where the AMS sends a ranging preamble code. MAX_RNG_ACK_TX_Time ≥ RNG_ACK_TX_Time | Shall be included when CDMA_RNG_FLAG=1 Units of frame |  |
| HO process optimization | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines |  | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |

[On the IEEE P802.16m/D3, page 165, line 59, make the following changes to subclause 16.2.6.3.5.1]

16.2.6.3.5.1 CDMA-based HO Ranging Procedure

If the serving ABS indicates in AAI_HO-CMD that CDMA-based HO ranging is not omitted by setting CDMA_RNG_FLAG=1 and a dedicated ranging preamble code/opportunity is assigned to the AMS by target ABS, the AMS shall transmits the dedicated ranging preamble code to the target ABS during network re-entry. If a ranging channel opportunity is scheduled by the target ABS for handover purpose only, the AMS should use that ranging channel opportunity in order to avoid excessive multiple access interference.

Upon reception of the dedicated ranging preamble code and if ranging is successful, the target ABS shall allocate uplink resources for AMS to send AAI_RNG-REQ message and/or UL data. If the target ABS does not receive the dedicated ranging preamble code within Ranging_Initiation_Deadline Timer, the target ABS shall discard the pre-assigned STID of the AMS.

If CDMA-based HO ranging is not omitted and if an AMS does not have a dedicated ranging preamble code or a dedicated ranging opportunity at the target BS, the AMS shall transmit a random handover ranging preamble code at the earliest available ranging opportunity.

If CDMA-based HO ranging is not omitted, target ABS shall inform AMS of transmission timing of AAI_RNG-ACK (RNG_ACK_ TX_Time) via the Serving ABS through the AAI_HO-CMD.

An AAI_RNG-ACK message shall be sent within RNG_ACK_TX_Time from the frame where some HO ranging preamble code(s) is detected. If the ABS provides UL BW allocation before sending the response to the successfully received ranging preamble code, the ABS shall omit the response in the AAI_RNG-ACK. If no successfully received ranging preamble code is included in the AAI_RNG-ACK, the ABS shall omit the transmission of the AAI_RNG-ACK.

The AMS may restart CDMA-based HO ranging procedure when any of the following conditions is met:

- Any AAI_RNG-ACK message or CDMA Allocation A-MAP IE is not received until the RNG_ACK_TX_Time is expired;

- The AMS does not find in the AAI_RNG-ACK message that at least one ranging preamble code has been successfully received in the ranging opportunity selected by the AMS until the RNG_ACK_TX_Time is expired;

- The AMS finds in the AAI_RNG-ACK message that no ranging preamble code has been successfully received in the ranging opportunity selected by the AMS;

- The AMS does not find a response in the AAI_RNG-ACK message to its HO ranging preamble code, but the AMS finds that at least one ranging preamble code has been successfully received in the ranging opportunity selected by the AMS;

- The T3 timer expires.

There are three possible responses from ABS to AMS provided in the AAI_RNG-ACK message:

  - Continue: The ABS provides the needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of "continue". In this case, the AMS shall adjust its parameters accordingly and continue the ranging process as done on the first entry.

  - Success: The ABS provides a status notification of "success". In this case, the ABS shall provide the AMS an UL BW allocation for the AMS to send AAI_RNG-REQ message.

  - Abort: The ABS provides a status notification of "abort". The ABS requests the AMS to abort the ranging process.

[On the IEEE P802.16m/D3, page 243, line 8, make the following changes to subclause 16.2.15.3.1]

16.2.15.3.1 Contention-based initial ranging and automatic adjustments

As specified in 16.3.9.2.4.1, ranging channel and ranging preamble codes shall be used for contention-based random access initial / handover ranging. Each ranging channel indicates a ranging opportunity.

An AMS that wishes to perform initial ranging shall take the following steps:

- The AMS, after acquiring downlink synchronization and uplink transmission parameters, shall select one Ranging Slot ranging opportunity based on the random backoff. The random backoff shall use a binary truncated exponent algorithm. After selecting the Ranging Slot ranging opportunity, the AMS shall choose a ranging sequence preamble code (from the Initial Ranging domain) using a uniform random process. The AMS shall send the selected ranging sequence preamble code to the ABS in the selected Ranging Slot ranging opportunity.

- The ABS should respond with an AAI_RNG-ACK message in a predefined, subsequent DL AAI subframe within MAX_RNG_ACK_TX_Time from the frame where some initial ranging preamble code(s) is detected. The AAI_RNG-ACK format is defined in 16.2.3.3. as define in 16.2.3.19 The AAI_RNG-ACK message provides responses to all the successfully received and decoded IR requests initial ranging preamble codes in all the initial ranging slots opportunities in a previous UL AAI subframe . If the ABS provides UL BW allocation before sending the response to the successfully received ranging preamble code, the ABS shall omit the response in the AAI_RNG-ACK. If no successfully received ranging preamble code is included in the AAI_RNG-ACK, the ABS shall omit the transmission of the AAI_RNG-ACK. If all the ranging statuses of the detected initial ranging preamble codes are equal to ‘success’, the AAI_RNG-ACK may be omitted.

- If the AMS finds in the RNG-ACK bitmap of the AAI_RNG-ACK message that no initial ranging preamble code sequence has been successfully decoded in the ranging slot selected by the AMS, or it does not find a response in the AAI_RNG-ACK message to its IR access attempt initial ranging preamble code, but the AMS finds that at least one ranging preamble code has been successfully received in the ranging opportunity selected by the AMS, the AMS considers its initial ranging request is failed and may restarts the initial ranging procedure. For all other cases, the AMS considers its initial ranging request is successfully received.

- After successfully receive an IR request initial ranging preamble code, there are three possible responses from ABS to AMS provided in the AAI_RNG-ACK message:

  - IR cContinue: tThe ABS provides the needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of "continue". In this case, the AMS resends randomly chosen initial ranging preamble code after the adjustments.

  - IR sSuccess: tThe ABS provides a status notification of "success", and has no adjustment suggestions to the AMS. In this case, the ABS shall provide the AMS an UL BW allocation for the AMS to send AAI RNG-REQ AAI_RNG-REQ message.

  - IR aAbort: The ABS provides a status notification of "abort". tThe ABS requests the AMS to abort the ranging process.

- Upon receiving a Continue status notification and parameter adjustments in the AAI_RNG-ACK message, the AMS shall adjust its parameters accordingly and continue the ranging process as done on the first entry (using random selection rather than random backoff) with ranging sequence preamble code randomly chosen from the initial ranging domain sent on the initial ranging region opportunity.

- Upon receiving a Success status notification, the AMS shall wait for the ABS to provide UL BW allocation. When receiving an UL BW allocation, the AMS shall send the AAI_RNG-REQ message. If the granted BW allocation cannot accommodate the entire AAI_RNG-REQ message, the AMS may fragment the AAI_RNG-REQ message to fit the provided BW allocation, and requests additional UL bandwidth through PBREH as defined in 156.2.2.2.98 for the remaining fragments. In response to the PBREH, the ABS shall provide UL BW allocation through CDMA Allocation A-MAP IE, which is identified by the same RA-ID used for the previous BW allocation. The RA-ID is used until AAI_RNG-REQ tranmission is completed, but if AMS does not receive UL bandwidth allocation in T3(TBD) or the AAI_RNG-REQ transmission is not completed in 64 frames, it sends ranging code to perform ranging procedure again

- Initial ranging process is over after receiving AAI_RNG-RSP message, which includes a TSTID (temporary station ID) to be used until STID is received at successful registration.

- The timer required for AMS to wait for AAI_RNG-RSP or CDMA Allocation A-MAP IE, is defined by T3 (TBD). Unless the AMS receives AAI_RNG-RSP or CDMA Allocation A-MAP IE within T3, it may restart the initial ranging procedure.

[On the IEEE P802.16m/D3, page 48, line 44, make the following changes to subclause 16.2.3.3]

16.2.3.3 AAI_RNG-ACK

If no HO ranging preamble code is detected and all the ranging statuses of the detected initial /periodic ranging preambles are equal to 'success', the AAI_RNG-ACK may be omitted. The AAI_RNG-ACK shall include the following parameters:

Table 676—Parameters for AAI_RNG-ACK

| Name | Value | Description |
| --- | --- | --- |
| Management Message Type | AAI_RNG-ACK |  |
| Number of frames (N_Frames) | The number of frame indices included in this RNG-ACK message | The size of this N_Frames is decided by MAX_RNG_ACK_TX_Time. |
| Frame index | Indicates LSB TBD bits of superframe number and 2 bits of frame index in a superframe | The size of this Frame index is decided by MAX_RNG_ACK_TX_Time. Shall be included for each frame if N_Frames > 0 |
| RNG-ACK Bitmap | The size of this bitmap is decided by the number of ranging slots in the previous frame. Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected | The size of this bitmap is decided by the number of ranging opportunities in the frame corresponding to Frame index. |
| Number of received codes | The number of ranging preamble code indices detected in this corresponding ranging opportunity. | The number of ranging code indices detected in this corresponding ranging opportunity. Shall be included for each bit=0b1 in the RNG-ACK Bitmap |
| Ranging preamble code index | Ranging preamble code index received in the ranging opportunity | Code index received in this ranging opportunity. Shall be included for each received ranging codes if Number of received codes > 0 |
| Ranging status | Used to indicated whether UL messages are ranging preamble code is received within acceptable limits by the ABS 0b01: continue 0b11: abort 0b00: success | Shall be included for each received ranging code if Number of received codes > 0 |
| Adjustment parameters indication (API) | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | Shall be included for each received ranging code when Ranging status is 0b01 |
| Timing offset adjustment | 10 bits Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs. | Shall be included when Time offset adjustment indication bit is 1 |
| Power level adjustment | 3 bits Relative change in transmission power level. Signed in units of 1 dB | Shall be included when Power level adjustment indication bit is 1 |
| Frequency offset adjustment | 8 bits Relative change in transmission frequency. The correction is 0.4% of the subcarrier spacing (Δf) multiplied by the TBD-bit number interpreted as a signed integer. | Shall be included when Frequency offset adjustment indication bit is 1 |

-----------------------------------------------------------End of the Text---------------------------------------------------------