| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The clarification of the zone configuration in Relay frame structure (16.6.3) |  |
| Date Submitted | 2010-01-12 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak, and Bin-Chul Ihm LG Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution provides the clarification of the zone configuration in Relay frame structure. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The clarification of the zone configuration in Relay frame structure

Jinyoung Chun, Jin Sam Kwak and Bin-Chul Ihm

LG Electronics

- Introduction

In the last meeting, cell-specific resource mapping for relay zone is accepted and it is indicated by AAI_System Configuration Descriptor (SCD) Message whether resource mapping in relay zone is separated with access zone or not. If resource mapping of relay zone is separated, AMSs served by ABS can’t be allocated in the relay zone. Moreover AMSs with long TTI is allocated during access zone and the AMSs shall know the number of access zone. Therefore I suggest some clarification related the parameter.

- Text proposal in the P802.16m/D3

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

[Modify the text in line 47, page 629 as below]

-----------------------------------------------   Text Start  -----------------------------------------------------------

When an ARS is supported, the long TTI allocations shall span the entire 16m Access zone or 16m Relay zone in any of DL or UL directions depending on where data transmission is allocated.

When an ARS is supported, the long TTI allocations in ABS-ARS link shall span the entire 16m Relay zone in any DL or UL directions and the long TTI allocations in ARS-AMS link shall span the entire 16m Access zone in any DL or UL directions. When 16m_Relay_zone_AMS_allocation_indicator is equal to 0, the long TTI allocations in ABS-AMS link shall span the entire 16m Access zone in any DL or UL directions. When 16m_Relay_zone_AMS_allocation_indicator is equal to 1, the long TTI allocations in ABS-AMS link shall span the entire DL or UL subframes in a TDD frame or occupy 4 AAI subframes for both DL and UL in a FDD frame.

-----------------------------------------------   Text End  -----------------------------------------------------------