| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Basic Multicarrier Aware Operation for AMS (16.2.8) |  |
| Date Submitted | 2010-1-13 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim LG Electronics Kelvin Chou MediaTek Inc. Kamran Etemad Intel Corporation Yuqin Chen ZTE Corporation Hyunjeong Kang Samsung Electronics Inc. | E-mail: eunjong.lee@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com, ronnyyoungho.kim@lge.com Kelvin.Chou@mediatek.com Kamran.etemad@intel.com chen.yuqin@zte.com.cn hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Basic Multicarrier Aware Operation for AMS (16.2.8)

Eunjong Lee, Kiseon Ryu, Inuk Jung, Youngsoo Yuk and Yongho Kim
LG Electronics

Kelvin Chou
MediaTek Inc.

Kamran Etemad
Intel Corporation

Yuqin Chen

ZTE Corporation

Hyunjeong Kang

Samsung Electronics Inc.

- Introduction

According to the current AWD, a single carrier supported AMS should perform inter-frequency HO procedure even when changing its carrier between two carriers managed by a common MAC although this may lead to unnecessary procedure, such as network reentry. In addition, current amendment is unclear for what or how single carrier supported AMS operates in the multi-carrier deployment. Therefore, this contribution clarifies the multi-carrier operation for single carrier supported AMS and proposes a carrier change procedure through AAI_CM-CMD message that a single carrier supported AMS can simply change its carrier to one of the fully configured carriers managed by a common MAC.

- Discussions

For the primary carrier and fully configured carrier, current AWD (section 16.2.8.1) has described as follows.

  - A primary carrier is a carrier used by the ABS to exchange traffic and PHY/MAC control signaling (e.g., MAC control messages) with an AMS. An ABS may be deployed with multiple carriers but each AMS in the ABS has only one primary carrier which is also used when AMS is operating in single carrier mode.

  - Fully configured carrier: a standalone carrier for which all control channels including synchronization, broadcast, multicast and unicast control signaling are configured. Fully configured carrier supports both single carrier AMS and multicarrier AMS.

According to the above description, the AMS in single carrier mode can operate under multi-carrier deployment, but Figure 416 in current D3 seems to be able to use the multi-carrier operations only when both the ABS and AMS supports any of multi-carrier modes.

In addition, the current AWD describes the primary carrier change procedure, but a single carrier supported AMS cannot correctly decode the AAI_CM-CMD message because this procedure has been designed only for multi-carrier supported AMS. In order to change their carrier, single carrier AMS should perform inter-frequency HO including network reentry procedure through HO-CMD/IND message. However, the AMS in single carrier mode can be also supposed to change its carrier to one of the fully configured carriers managed by a common MAC and they don’t need to perform the network reentry procedure because a single ABS manages all carriers within a common MAC.

Therefore, this contribution clarifies some multi-carrier operations for single carrier supported AMS and proposes to define the carrier change for single carrier supported AMS and modify the AAI_CM-CMD message.

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

Text proposal 1

[Modify the Section 16.2.8.2.3 on page 184 as follows:]
----------------------------------------------------------- Text Start ---------------------------------------------------------

16.2.8.2.3 Network Entry

The network entry in multicarrier mode is the same as single carrier mode as defined in 165.2.15, where the AMS and ABS also indicate their support for multicarrier modes during the registration. An AMS can only perform network entry (or network reentry) procedures with a fully configured carrier. Once the AMS detects the A-PREAMBLE on a fully configured carrier, the AMS may proceed with reading SFH or Extended system parameters and system configuration information.

During the initial network entry, AMS will inform ABS of its support of multicarrier transmission by AAI_REG-REQ message and the ABS will indicate if it supports any of multicarrier modes for that AMS through AAI_REG-RSP message. The basic multicarrier capability exchange uses a two bit code in AAI_REG-REQ/RSP message with the following indications:

Table 734—Multicarrier capability in AAI_REG-REQ/RSP message

| b1, b2 | Multicarrier Capabilities |
| --- | --- |
| 00 | No MC mode |
| 01 | Basic MC mode Multicarrier aggregation |
| 10 | Multicarrier Aggregation Multicarrier switching |
| 11 | Both Multicarrier aggregation and Switching |

The Basic MC mode involves AMS awareness of multicarrier operation at the ABS which includes support for Primary Carrier Changes as defined in section 16.2.8.2.11.2 as well as optimized scanning of carriers involved multicarrier operation defined in section 16.2.8.10.

Support for Both Multicarrier Aggregation and Switching does not imply EMBS support, which is negotiated separately.

The procedure for initialization of an AMS, following network entry, to prepare for subsequent multicarrier operation shall be as shown in Figure 416. This procedure includes

•Obtaining the multicarrier configuration for available carriers at the ABS

•Obtaining information about Assigned Carriers consisting of two steps

1.Provide ABS with information on AMS's supportable carriers and their combined multicarrier configurations

2.Obtain information about the subset of available carrier, hereby referred to as the Assigned Carriers, which ABS may utilize in subsequent multicarrier operation for that AMS.

The AMS does not perform any MAC or PHY processing on an assigned carrier until such carrier is activated per ABS's direction.

Figure 416—AMS initialization overview to support multicarrier transmission

----------------------------------------------------------- Text End ---------------------------------------------------------

Text proposal 2

[In the P802.16m/D3, modify section 16.2.8.2.9.2 at page 191, line 49 as follows:]

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.8.2.9.2 Multicarrier handover (MCHO) procedure

The multicarrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers, which includes multi-carrier EBB HO <<15.2.9.2.9.2.2.1>> and HO with secondary carrier pre-assignment <<15.2.9.2.9.2.2.2>> as described in this section. For an  AMS supporting basic MC mode handover from one carrier to another carrier of the same ABS, the handover procedure follows the primary carrier change procedure per 16.2.8.2.11.2.

16.2.8.2.9.2.1 MCHO preparation

An AMS in multicarrier operation follows the handover operations defined in 15.2.6.216.2.6.3. MAC control messages in relation with handover preparation and initiation between the AMS and the serving ABS are transmitted over the primary carrier of the AMS.

During HO preparation, the AMS may request or be requested by the serving ABS to perform MCHO procedure through AAI_HO-REQ/AAI_HO-CMD messages. The serving ABS informs the AMS the carrier information (e.g. target primary carrier index) of the target ABSs through AAI_HO-CMD message. The serving ABS may communicate with the target ABS(s) to help the AMS obtain the assigned secondary carriers before handover execution. The serving ABS will forward the information received from AAI_MC-REQ message to the target ABS(s) for secondary carrier pre-assignment. The serving ABS will reply the secondary carrier pre-assignment results to the AMS if Carrier_Preassignment_Indication is set to 1 in the AAI_HO_CMD message.

16.2.8.2.9.2.2 MCHO execution and network reentry with HO_Reentry_mode=1

Multicarrier EBB HO reentry procedure

The AMS with multicarrier capability follows the network reentry procedure per section <<15.2.6.2.5>>16.2.6.3.5. When HO_Reentry_Mode is set to 1 and HO_Reentry_Interleaving_Interval is set to 0, the AMS performs network reentry to the target ABS on one carrier and maintains normal communication with the serving ABS on another carrier not performing network reentry procedure. The AMS may use the original primary carrier for network reentry to the target ABS, as illustrated in Figure 419. It may also use another carrier different from its original primary carrier for network reentry procedures, as illustrated in Figure 420. In this case, Disconnect_time should be long enough that network reentry procedure to target ABS can be completed prior to the expiration of Disconnect_time. In case of AAI_HO-CMD message with multiple target ABS and carriers, the physical index of each candidate carrier provided by each target ABS should also be indicated in the AAI_HO-CMD message The AMS may inform the serving ABS through AAI_HO-IND the carrier to be used for network reentry operations to avoid resource allocation by the serving ABS on that carrier.

{No change to Figure 419 and 429}

From AMS point of view, if network entry is completed (see 15.2.6), the AMS shall stop communicating with the serving ABS. Then, the AMS may send UL data or BW-REQ message to the target ABS.

------------------------------------------------------  Text End  ---------------------------------------------------------------

Text proposal 3

[Modify section 16.2.8.2.11 on page 195, line 38 as follows:]
----------------------------------------------------------- Text Start ---------------------------------------------------------

16.2.8.2.11 Carrier management

16.2.8.2.11.1 Secondary Carrier managementActivation/De-activation

The activation or deactivation of secondary carrier(s) is decided by the ABS based on QoS requirement, load condition of carriers and other factors. The ABS activates and/or deactivates secondary carrier with the AAI_CM-CMD MAC control message. The ABS sends the AAI_CM-CMD MAC control message on the primary carrier and includes the following information:

•Indication Type per DL/UL: Activation, Deactivation

•List of Secondary Carriers: (referred by logical carrier index)

•Ranging indicator for the activated carrier

The AAI_CM-CMD MAC control message may allocate feedback channel for the activated carriers which does not have the paired UL like partially configured carrier or asymmetrically activated DL carrier.

The ABS sends the AAI_CM-CMD message with Polling set to 1 in MCEH and the AMS, upon receiving the AAI_CM-CMD message, transmits an AAI_MSG-ACK message or Message ACK EH to inform that the AAI_CM-CMD message has been successfully received. In response to the AAI_CM-CMD MAC control message, tThe AMS transmits the AAI_CM-IND MAC control message through the primary carrier. T, where this message confirms with the ABS that the AMS has successfully activated/deactivated the carriers listed in the AAI_CM-CMD message. In case of activation, the confirmation AAI_CM-IND message is sent by the AMS when DL/UL of the newly activated carrier is ready to be used to transport data traffic.

When AMS performs secondary carrier activation while AMS covering the primary carrier and secondary carrier with one radio carrier, AMS reconfigures its hardware setting (e.g. RF center frequency). After  completing the hardware reconfiguration and synchronization on the new carrier, AMS notifies the ABS its readiness of the new carrier by sending AAI_CM-IND message and then resume communication with ABS.

After the ABS receives the AAI_CM-IND MAC control message, the ABS may start transmitting data on such active secondary carrier(s).

Note that the AAI_CM-CMD and AAI_CM-IND message cannot be sent until the secondary carrier and the corresponding logical carrier index are assigned to an AMS.

16.2.8.2.11.2 Primary Carrier Change

The Primary Carrier change involves changing the serving carrier for an AMS in a multicarrier ABS without changing the MAC layer security and mobility contexts and unlike normal inter-FA handover. An AMS which is Multicarrier Aware shall support primary carrier change. The ABS may instruct the AMS, through the AAI_CM-CMD MAC control message on the current primary carrier, to change its primary carrier to one of the assigned fully configured carriers within the same ABS for load balancing purpose, carriers' varying channel quality or other reasons. When an AMS receives the AAI_CM-CMD MAC control message with Polling set to 1 in MCEH, the AMS transmits the AAI_CM-IND MAC control messagethe AAI_MSG-ACK message or Message ACK EH in response to the AAI_CM-CMD MAC control message and the AMS disconnects control signal on the serving carrier and switches to the target fully configured carrier at action time specified by the ABS.

If the AMS supports carrier aggregation mode and the target carrier is one of the active secondary carriers of the AMS, the AMS may receive data and control signal on the target carrier immediately after switching. Otherwise, the AMS first reconfigures its hardware setting (e.g. RF center frequency) and switches to target carrier. If Ranging indicator in AAI_CM-CMD message is set to ‘1’, the AMS shall perform the periodic ranging procedure with target carrier. After successfully completing this action, the AMS then mayshall transmit a bandwidth requestAAI_CM-IND message to notify its readiness of the target carrier to the ABS. If the AMS has UL data to be transmitted, it follows the normal bandwidth request procedure. If the AMS does not have any UL data, it transmits a bandwidth request with zero bandwidth request size. The ABS may transmit data and control signal after a bandwidth request the AAI_CM-IND message is received from the AMS through the target primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. The logical carrier indices of the serving and target primary carrier are swapped after the primary carrier change. The ABS may direct an AMS to change the primary carrier without scanning. For the multi-carrier supported AMS, the logical carrier indices of the serving and target primary carrier are swapped after the primary carrier change.

The ABS may instruct the AMS to perform scanning on other carriers which are not serving the AMS. The AMS reports the scanning results back to the serving ABS, which may be used by the ABS to determine the carrier for the AMS to switch to. In this case, if the target carrier is not currently serving the AMS, the AMS may perform synchronization with the target carrier if required.

The AAI_CM-CMD MAC control message for the primary carrier change is transmitted on the primary carrier and shall include the following information:

  - Target primary carrier index

  - Indication of the next state of carrier serving primary
: if the AMS does not support carrier aggregation, this field shall be always set to ‘0’.

  - Action Time

  - Ranging indicator

The serving primary carrier will be kept active or deactivated depending on the indication of the next state of serving primary carrier.

Figure xx1. Primary Carrier Change procedure when target carrier is one of the active carriers

Figure xx2. Primary Carrier Change procedure when target carrier is one of the inactive carriers

16.2.8.2.11.3  Multicarrier Aggregation

The ABS activates one or more assigned secondary carriers of an AMS through AAI_CC-CMD message. The ABS may make concurrent resource allocation using A-MAP messages on multiple active carriers, including primary carrier. The Multicarrier allocation and aggregation may be used independently in the downlink or uplink and the ABS performs the operations based on QoS, loading and AMS’s capabilities.

The AMS in multicarrier aggregation mode is assigned the same STID to be used across primary and secondary carriers. The AMS supporting Multicarrier aggregation mode shall monitor all active carriers and follow the resource allocations while in active mode.

When supported by AMS, serving and target ABS the carrier aggregation mode may be maintained during the handover procedure through secondary carrier pre-assignment procedure per 16.2.8.2.9.

16.2.8.2.11.33 Primary to Secondary Carrier sSwitching mode

Primary to secondary carrier switching in multicarrier mode is used for EMBS only. The EMBS operation on the multi-carrier deployment is defined in 1516.9.2.1

----------------------------------------------------------- Text End ---------------------------------------------------------

Text proposal 4

[Modify section 16.2.8.2.11.4 on page197, line 1 as follows:]

----------------------------------------------------------- Text Start ---------------------------------------------------------

16.2.8.2.11.4 MAC Control Messages for Carrier Management

16.2.8.2.11. 4.1 Carrier Management Command (AAI_CM-CMD) MAC control message

Table 736—AAI_CM-CMD MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management Command message format(){ |  |  |
| Control Message Type | 8 | AAI_CM-CMD |
| Action code | 1 | 0: Secondary carrier management 1: Primary carrier change |
| If (Action code == 0){ |  | This message is for secondary carrier activation and/or deactivation |
| Indication Type | 2 | For Activation and/or Deactivation Bit #0: ‘0’: No action, ‘1’: Activation Bit #1: ‘0’: No action, ‘1’: Deactivation |
| If (Indication Type #0 == 1) |  |  |
| Num of target carrier | [TBD]4 | The number of newly activated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD]3 | Target logical carrier index for activation |
| Activation of DL/UL | 1 | ‘0’: Both DL/UL are activated ‘1’: DL is activated but UL is not activated |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |
| If (Indication Type #1 == 1){ |  |  |
| Num of target carrier | [TBD]4 | The number of deactivated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD]3 | Target carrier logical index for deactivation |
| Deactivation of DL/UL | 1 | ‘0’: Both DL/UL are deactivated ‘1’: UL is deactivated but DL is kept active |
| } |  |  |
| } |  |  |
| } |  |  |
| If (Action code == 1){ |  | This message is for primary carrier change. |
| Physical carrier index of Target carrier index | [TBD]6 | Physical carrier index of target carrier Target carrier index for primary carrier change If the AMS supports multicarrier operation, the carrier shall be one of the assigned carriers. |
| Action Time | [TBD]3 | LSB bits of Superframe number at the time to switch to the target carrier. |
| Next state of serving primary carrier | 1 | ‘0’: Serving carrier will be deactivated after primary carrier change. If the AMS does not support carrier aggregation, this field shall be always set to ‘0’. ‘1’: Serving carrier is kept active after primary carrier change |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------