| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Rank-1 Pilot Selection for Femto ABS on Section 16.3.5.4.1 |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Chung-Lien Ho, Yan-Xiu Zheng, Chang-Lan Tsai, Yu-Chuan Fang ITRI | Email: clho@itri.org.tw |
| Re: | Contribution in support of a comment on LB30 Section 16.3.5.4.1 pilot patterns |  |
| Abstract | The contribution proposes the text of rank-1 pilot selection for Femto ABS to be included in the 802.16m AWD (P802.16m/D3). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD (P802.16m/D3). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Rank-1 Pilot Selection for Femto ABS on Section 16.3.5.4.1

Chung-Lien Ho, Yan-Xiu Zheng, Chang-Lan Tsai, Yu-Chuan Fang

ITRI

- Introduction

The contribution proposes the text of rank-1 pilot selection for Femto ABS to be included in the 802.16m AWD ([4]). The rank-1 pilot selection for Femto ABS is proposed to mitigate the pilot collision among neighboring Femto ABSs for transmissions with one stream and neighboring macro ABSs for transmissions with one stream or two streams. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([1]), it is compliant to the 802.16m SRD ([2])).

- Base Station Types

In 16m, each cell ID has an integer value IDcell from 0 to 767. The IDcell is defined by segment index and an index per segment as follows:

()

where n is the index of secondary advanced (SA) Preamble carrier-set 0, 1, and 2 representing segment index, and

()

with q being a running index 0 to 255.

SA-Preamble sequences are partitioned and each partition is dedicated to specific base station (BS) type like Macrocell advanced BS (ABS), Macro Hotzone ABS, or Femto ABS. The BS types may be categorized into macro ABS and non-macro ABS cells by hard partition with 252 sequences (84 sequences per segment * 3 segments) dedicated for macro ABS. The non-macro ABS information is broadcasted in hierarchical structure. Non-macro ABS cell type is partitioned as public and Closed Subscriber Group (CSG) femto BSs. The non-macro ABS BSs can be further categorized different types, such as: hotzone, relay, Open Subscriber Group (OSG) femto BSs and etc. The CSG femto BSs can be further categorized, such as: CSG-closed and CSG-open BSs, as shown in Figure 1. In the following, we assume that Cell ID of CSG Femto ranges from z to 255, where z = 169.

Figure 1: BS types (hierarchical structure) in 16m.

- Rank-1 Pilot Selection for Macrocell-Only Network

At Session #64 IEEE meeting, two proposals have been proposed for support of macrocell. One is proposed by Samsung based on the CeII ID and mobile station ID (STID) [5], and the other is proposed by LGE based on the Cell ID [6].

  - Samsung’s Rank-1 Selection

The Samsung’s rank-1 pilot patterns are proposed as shown in Figure 2. As depicted in Figure 2, 6 pilot patterns are proposed and categorized into 3 interlaced pilot pattern sets (in x-axis direction) and further 2 pilot pattern stream sets (in y-axis direction).

Figure 2: Samsung’s rank-1 pilot patterns [5].

The associated rank-1 pilot selection is proposed as function of Cell ID k and STID m as follows

()

Based on (3), the deployment of pilot selection over several cells is illustrated in Figure 3. From Figure 3, we can see that all “different” pilot pattern stream sets (i.e., s1 and s2) may be simultaneously used in a 3-sector cell (e.g., Cell ID 0, 256 and 512).

Figure 3: Deployment of Samsung’s rank-1 pilot selection method.

  - LGE’s Rank-1 Selection

The LGE’s rank-1 pilot patterns are proposed as shown in Figure 4. As depicted in Figure 4, 6 pilot patterns are also proposed and categorized into 3 interlaced pilot pattern sets and further 2 pilot pattern stream sets.

Figure 4: LGE’s rank-1 pilot patterns [6].

The associated rank-1 pilot selection is proposed as a function of Cell ID k only as follows

()

Based on (4), the deployment of pilot selection over several cells is illustrated in Figure 5. From Figure 5, we can see that the “same” pilot pattern stream set (i.e., either s0 or s1) may be simultaneously used in a 3-sector cell (e.g., Cell ID 0, 256 and 512).

Figure 5: Deployment of LGE’s rank-1 pilot selection method.

- Proposed Rank-1 Femtocell Pilot Seleciton for Macro-Femto Hybrid Network

In this section, the rank-1 pilot pattern selection method for support of Macro-Femto hybrid transmission network is proposed. Actually, the proposed method can be applied to both Samsung’s and LGE’s rank-1 pilot selection schemes to mitigate the pilot collision among neighboring Femto ABSs for transmissions with one stream and neighboring macro ABSs for transmissions with one or two streams. The basic design philosophy of proposed method is that use the different rank-1 pilot pattern from its overlay macrocell’s rank-1 and/or rank-2 pilot patterns.

  - Pilot Pattern Selection Algorithm

The proposed pilot selection algorithm for support of macro-femto hybrid transmission networks is summarized as follows:

- Step 1: Select the femtocell’s Cell ID as

()

where  (or ) is the femtocell segment index with n being the macrocell segment index in (1), and l is the selected based on (2) such that IDcell in (5) belongs to femto ABS. In (5), the femtocell segment index i can be chosen as follows

  - Mode 0: 	()

  - Mode 1:

where mode 0 is the base mode used at the initial stage, and mode 1 may be used if available when the same pilot pattern is used by the neighboring femtocells.

- Step 2: Select the interlaced pilot pattern set based on its Cell ID IDcell in (5) as

()

- Step 3: Select the pilot pattern stream set based on its Cell-ID IDcell in (5) as

()

From (6)-(8), the proposed femtocell rank-1 pilot pattern selection is the same as the LGE’s macrocell rank-1 pilot pattern selection except the Cell ID selection. In our proposed method, the femtocell Cell ID selection is done based on the given macrocell Cell ID, and two modes for the Cell ID selection is also proposed to avoid the interference caused by the possible pilot collision among femtocells.

The overall signal processing of the proposed pilot pattern selection algorithm is depicted in Figure 6. First, the femtocell segment index is chosen based on the given marcocell segment index using the base mode (Mode 0). With femtocell segment index obtained, the femtocell’s rank-1 pilot pattern can be calculated via a proposed pilot selection engine as shown in Figure 7. The pilot selection engine with the input i (femtocell segment index) and output pxsy (rank-1 pilot pattern), x = 0, 1, 2, and y = 0, 1, has three main processing steps including the femtocell Cell ID selection, interlaced pilot pattern set selection and pilot pattern stream set selection. Note that the interlaced pilot pattern set px and pilot pattern stream set sy are computed based on the femtocell Cell ID. Next, determine whether the selected pilot pattern is the same as that of neighboring femtocells. If no, the pilot pattern selection is done, or re-chooses the femtocell segment index using Mode 1 and then goes to the pilot selection engine to regenerate the femtocell’s rank-1 pilot pattern.

Figure 6: Signal processing of the proposed femcell rank-1 pilot selection algorithm.

Figure 7: Proposed pilot selection engine.

  - Examples of Proposed Pilot Pattern Selection

- Example 1: Femtocells are close to their overlay Macro ABS

When Femtocells are close to their overlay Macro ABS, both cross-tier (femto-macro) and intra-tier (femto-femto) interferers may be arisen in pilot transmission as shown in Figure 8.

Figure 8: Illustration of Example 1 of the proposed pilot pattern selection.

- Example 2: Femtocells are very close to their overlay Macro ABSs

When Femtocells are very close to their overlay Macro ABSs, again both cross-tier (femto-macro) and intra-tier (femto-femto) interferers may be arisen in pilot transmission as shown in Figure 9. Note that Example 2 rarely occurs because the femtocells in general are deployed at the macrocell’s cell edge for coverage enhancement.

Figure 9: Illustration of Example 2 of the proposed pilot pattern selection.

- Example 3: Femtocells are near their overlay Macro ABSs’ cell edge

When Femtocells are near their overlay Macro ABSs’ cell edge, again both cross-tier (femto-macro) and intra-tier (femto-femto) interferers may be arisen in pilot transmission as shown in Figure 10. Note that Example 3 often occurs because the femtocells in general are deployed at the macrocell’s cell edge for coverage enhancement.

Figure 10: Illustration of Example 3 of the proposed pilot pattern selection.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m/D2, “802.16m Amendment Working Document (AWD)”

[5]	C80216m-09/2507, Samsung, Nov. 2009

[6]	C80216m-09/2367, LGE, Nov. 2009

- Text proposal for inclusion in the 802.16m AWD

------------------------------------------------------------ Text Start ------------------------------------------------------------

Add the following text on line 14 of page 337 in Section 16.3.5.4.1:

For the support of Macro-Femto hybrid network, the femtocell’s Cell ID (IDcell = k) is determined according to the following equation:

where i = mod((n + 1), 3) (or i = mod((n  1), 3)) is the femtocell segment index with n being the macrocell segment index, and  is selected such that Cell ID k belongs to femto ABS. The femtocell segment index i can be chosen as follows:

  - Mode 0:

  - Mode 1:

where mode 0 is the base mode used at the initial stage, and mode 1 may be used if available when the same pilot pattern is used by the neighboring femtocells. The index of the rank-1 pilot pattern set of femtocell is determined by its Cell ID IDcell according to the following equation:

------------------------------------------------------------- Text End -------------------------------------------------------------