| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Dynamic Allocation of E-MBS Region for non-macro diversity transmission mode (16.3.6) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak LG Electronics | chegal@lge.com, youngsoo.yuk@lge.com, js.choi@lge.com, ksryu@lge.com, ronnykim@lge.com, samji@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b |  |
| Abstract | Proposed text change for dynamic allocation of E-MBS region in non-macro diversity mode |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Dynamic Allocation of E-MBS Region for non-macro diversity transmission mode (16.3.6)

Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak

- LG Electronics

# Introduction

According to IEEE P802.16m/D3, the size of E-MBS MAP is of variable because an E-MBS-DATA IE contains resource information for multiple MBS bursts.

This contribution proposes the E-MBS Region A-MAP to inform the location of E-MBS region with dynamic multiplexing of E-MBS region with normal data region in case of non-macro diversity mode.

# Dynamic Allocation of E-MBS Region in non-macro diversity mode

Without macro-diversity, the DRU resource region should be used for obtaining diversity gain. Because DRU region is hardly used with fixed allocation, the dynamic allocation should be applied.

Figure 1 shows the proposed scheme. By using E-MBS zone STID which is broadcast by AAI_EMBS-CFG, an AMS may find DL Basic Assignment A-MAP IE or DL Persistent Allocation A-MAP IE whose CRC is masked by E-MBS zone STID.

The proposed scheme has following advantages.

- Diversity gain without MBSFN

- Less impact for scheduling of Unicast Data

- DRU and/or CRU can be used for E-MBS region

- No restriction for E-MBS data allocation

Though the drawback of the proposed scheme is additional effort to decode the DL Basic Assignment A-MAP IE at every MSI, it’s not a big problem considering the gains. In addition, if we use DL persistent A-MAP IE, AMS need not to decode A-MAP every MSI.

Figure 1. Dynamic allocation of E-MBS MAP and burst

# Text proposal for the IEEE 802.16m amendment

- --------------Proposed Text #1 ---------------------------------------------------------------------------

- [On the IEEE P802.16m/D3, page 117, line 45, add the following subsections]

16.2.3.49 AAI_E-MBS-CFG Message

An AAI_E-MBS-CFG message is broadcast by the ABS to inform the configuration of E-MBS region.

Table XXX AAI_E-MBS-CFG message

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| AAI_MBS-CFG Message Format () { | - | - |
| Control Message Type | 8 |  |
| Num_E-MBS_Zones | 3 | The number of E-MBS zones the ABS is associated. |
| For (i=0; i< Num_E-MBS_Zones; i++) { | - |  |
| E-MBS_ZONE_ID | 7 | The EMBS_Zone_ID to which this EMBS MAP applies |
| E-MBS Transmission Mode | 1 | This field indicates E-MBS Transmission mode. 0b0 : Non-macro diversity mode 0b1 : Macro diversity mode |
| MSI Length | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms 0b01: 4 superframes, 80 ms 0b10: 8 superframes, 160 ms 0b11: 16 superframes, 320 ms |
| If(E-MBS Transmission Mode =0b0){ |  |  |
| E-MBS Zone STID | 12 | It is used for AMS to decode the DL Assignment A-MAP IE which indicates the resource allocation of E-MBS MAP |
| } |  |  |
| } |  |  |
| } |  |  |

- --------------Proposed Text #1 ---------------------------------------------------------------------------

--------------Proposed Text #2 ---------------------------------------------------------------------------

- [On the IEEE P802.16m/D3, page 374, line 52, add the following subsections]

- 16.3.6.3.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

A 16-bit CRC is generated based on the contents of the E-MBS MAP.  Following randomization, the resulting sequence of bits shall be encoded by the CTC described in 16.3.11. 1.

If the non-macro diversity mode is used, a DL Basic Assignment A-MAP IE or a DL persistent allocation A-MAP IE are used for indicating the E-MBS region in the subframe which contains E-MBS MAP. The DL Basic Assignment A-MAP IE or the DL persistent allocation A-MAP IE indicate the MCS, the resource allocation of an E-MBS MAP. The DL Basic Assignment A-MAP IE is transmitted at the Assignment A-MAP region of the first frame of MSI and CRC of the DL Basic Assignment A-MAP IE is masked by the E-MBS Zone STID.  If a DL persistent allocation A-MAP IE is used, the resource allocation of E-MBS MAP is persistently maintained within an allocation period. The unit of the allocation period is an MSI.

--------------Proposed Text #2 ---------------------------------------------------------------------------

--------------Proposed Text #3 ---------------------------------------------------------------------------

- [On the IEEE P802.16m/D3, page 643, line 54 ,Delete Table 944]

Table 944-

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_E-MBS-CFG Message Format () { | - | - |
| Num_E-MBS_Zones | 3 | The number of E-MBS zones the ABS is associated. |
| For (i=0; i< Num_E-MBS_Zones; i++) { | - |  |
| E-MBS_ZONE_ID | 7 | The EMBS_Zone_ID to which this EMBS MAP applies |
| MSI Length | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms 0b01: 4 superframes, 80 ms 0b10: 8 superframes, 160 ms 0b11: 16 superframes, 320 ms |
| E-MBS MAP Resource Index | [TBD] | Resource index includes location and allocation size |
| E-MBS MAP ISizeOffset | 5 |  |
| } |  |  |
| } |  |  |

--------------Proposed Text #3 ---------------------------------------------------------------------------