| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes Related to TDD UL HARQ timing in 802.16m/D3 (16.2.14) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Dongseung Kwon ETRI Yujian Zhang, Yi Hsuan Intel Corporation | Voice: +82-42-860-5748 jsp@etri.re.kr yujian.zhang@intel.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30b on IEEE P802.16m/D3 |  |
| Abstract | The contribution proposes the text changes to TDD UL HARQ timing in 802.16m/D3. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes Related to TDD UL HARQ timing in 802.16m/D3 (16.2.14)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, Dongseung Kwon

ETRI

Yujian Zhang, Yi Hsuan

Intel Corporation

- Introduction

This contribution proposes the text changes to TDD UL HARQ timing defined in the current 802.16m draft [1]. In the current draft, the A-MAP region is located in all DL AAI subframes for both DL and UL transmission, and the data burst with the long TTI begin in 0-th AAI subframe spanning all DL AAI subframes and all UL AAI subframes for DL HARQ and UL HARQ, respectively [1][2].

For the TDD frame with DL to UL ratio of the number of DL AAI subframes (D): the number of UL AAI subframes (U), if D is less than U, the frame partition is not expected to be a commonly used configuration, such as 3:5, and this frame partition ratio has been proposed to be only supported for the data burst with the long TTI [3]. Then it was adopted in “Description/Notes” of “Long TTI Indicator” field of the A-MAP IEs related to UL allocation relevance in the current draft [1]. Also, HARQ timing of the data burst with the long TTI is different from it for the default TTI for UL. In the case of that D is less than U or equal to 1 for TDD UL HARQ timing, it is necessary for UL basic assignment A-MAP to include the AAI subframe index of HARQ subpacket when l = 0 or l = D–1 according to the current draft [1]. However, the AAI subframe index field of the A-MAP IEs related to UL allocation relevance has lost its usefulness, because this case is only supported for the data burst with the long TTI. Therefore, these should be applied in TDD UL HARQ timing in the case of that D is less than U.

In this contribution, it is proposed to the text remedies to clarify the TDD UL HARQ timing and to include an editorial correction for implementations of this standard for above that reason.

- References

- IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” December 8, 2009.

- Mihyun Lee, Hyunkyu Yu, Jaeweon Cho, Hokyu Choi, “A-MAP relevance and HARQ timing for long TTI transmission (Section 15.3.9. 2.2),” C802.16m-09/1923r2, August 2009.

- Roshni Srinivasan, Hujun Yin, Yi Hsuan, Hyunkyu Yu, “Proposed Changes Related to Allocation Relevance in the DL and UL Basic Assignment A-MAP Information Elements in the 802.16m AWD (AWD - 15.3.6.5.2.2),” C802.16m-09/1329r1, July 2009.

- Proposed Changes for inclusion in the 802.16m/D3

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Remedy: Change the following modification in Table 753 of page 234, section 16.2.14.2.2.2.2 Uplink as follows]

Table 753– TDD UL HARQ timing

| Content | AAI Subframe index | Frame index |
| --- | --- | --- |
| Basic Assignment A-MAP IE Tx in DL | l | i |
| HARQ Subpacket Tx in UL | For D ≥ U, | j = (i + v) mod 4 |
|  | m= 0, for 0≤l<Kl−K, for K≤l<U+K |  |
|  | For 1 < D < U, |  |
|  | m = 0 m=0, …, or l−K, for l=0 l−K , for 0<l<D−1 |  |
|  | For D = 1 |  |
|  | m = 0, 1, … , or U – 1 for l = 0 |  |
| HARQ feedback in DL | l | k = (j + 1 + w) mod 4 |
| HARQ Subpacket ReTx in UL | m | p = (k + v) mod 4 |

------------------------------------------------------- End of the Text -------------------------------------------------------