| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes Related to CDMA Allocation A-MAP IE for Grant in 802.16m/D3 (16.2.11) |  |
| Date Submitted | 2010-01-12 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi ETRI | jsp@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30b on IEEE P802.16m/D3 |  |
| Abstract | The contribution proposes the text changes to CDMA Allocation A-MAP IE in 802.16m/D3. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes Related to CDMA Allocation A-MAP IE for Grant in 802.16m/D3 (16.2.11)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi

ETRI

- Introduction

This contribution propose the text changes to CDMA Allocation A-MAP IE to clean up long TTI transmission of MAC control message and to include small modification defined in the current 80216m draft [1].

- References

- IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” December 8, 2009.

- Proposed Changes for inclusion in the 802.16m/D3

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Remedy: the text changes in Table 743 of page 209, section 16.2.11.2 Grant as follows]

Table 743– CDMA Allocation A-MAP IE* format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  |  |
| A-MAP IE type | 4 | CDMA Allocation A-MAP IE |
| If MCRC is masked with RAID and masking indicator for BR { |  |  |
| Resource Assignment Information | TBD |  |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1. |
| HFA | 3 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Reserved | TBD |  |
| } |  |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging { |  |  |
| Resource Assignment Information | TBD |  |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1. |
| HFA | 3 | HARQ Feedback Allocation |
| Reserved | TBD |  |
| } |  |  |
| MCRC | 16 | CRC masked by RA-ID and Masking Indicator |
| } |  |  |

* A 16 bit CRC is generated based on the contents of the CDMA Allocation A-MAP IE. The CRC is masked by RA-ID and Masking Indicator.

------------------------------------------------------- End of the Text -------------------------------------------------------