| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes Related to Feedback Polling A-MAP IE in 802.16m/D3 (16.3.6) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung ETRI | Voice: +82-42-860-5748 jsp@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30b on IEEE P802.16m/D3 |  |
| Abstract | The contribution proposes the text changes to Feedback Polling A-MAP IE in 802.16m/D3. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes Related to Feedback Polling A-MAP IE in 802.16m/D3 (16.3.6)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung

ETRI

- Introduction

This contribution proposes the text changes to Feedback Polling A-MAP IE defined in the current 802.16m draft [1]. A data burst may occupy contiguous multiple AAI subframe with long TTI transmission not only for the motive or purpose of the usage of long TTI, but also for the reason that a specific frame partition ratio for TDD had been proposed to be only supported for the data burst with the long TTI. If the number of DL AAI subframes (D) is less than the number of UL AAI subframes (U) in TDD frame partition, only long TTI transmission is used for TDD uplink process, since this TDD frame partition is not expected to be a commonly used configuration such as 3:5, and then have been proposed to be only supported for the data burst with the long TTI [2]. So it needs to add the field of Long TTI Indicator for dedicated UL allocation in Feedback Polling A-MAP IE.

Additionally, the dedicated UL allocation is used for the MIMO feedback using a MAC control message or an extended header according to the requested feedback content in Feedback Polling A-MAP IE. AMS sends MIMO feedback using MAC control message such as AAI_SingleBS_MIMO_FBK or AAI_MultiBS_MIMO_FBK message, or using an extended header such as correlation Matrix Feedback Extended Header (CMFEH) or MIMO Feedback Extended Header (MFEH). With a standalone, an extended header may be sent by AMS. HARQ operation should be applied to all the unicast MAC control messages for MAC management reliability as well as unicast data traffic in both downlink and uplink, respectively. Therefore HARQ operation should enable for the dedicated UL allocation as the MIMO feedback in Feedback Polling A-MAP IE.

In this contribution, it is proposed to the text changes to be related to HARQ operation for MIMO feedback and to include a clarification for the transmission length of data burst and some editorial corrections for implementation of this standard for above that reason.

- References

- IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” December 8, 2009.

- Roshni Srinivasan, Hujun Yin, Yi Hsuan, Hyunkyu Yu, “Proposed Changes Related to Allocation Relevance in the DL and UL Basic Assignment A-MAP Information Elements in the 802.16m AWD (AWD - 15.3.6.5.2.2),” C802.16m-09/1329r1, July 2009.

- Proposed Changes for inclusion in the 802.16m/D3

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Remedy #1: Change the following modification in line 1 of page 437, section 16.3.6.5.2.4. as follows]

16.3.6.5.2.4.11 Feedback Polling A-MAP IE

The information element shown in Table 827 is used by the ABS to schedule MMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

When the Dedicated UL Allocation bit is set to 0b1, a dedicated UL allocation shall be included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE may be transmitted to signal control information for HARQ retransmission of the dedicated MIMO feedback. When the Dedicated UL Allocation bit is set to 0b0, no dedicated UL allocation shall be included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback and the ABS shall include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE.

The AMS shall send the feedback in an extended header when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The AMS shall send the feedback in a MAC control message when the following feedback information is requested:

- subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6

- multi-BS feedback

[Remedy #2: Change the following modification in Table 827 of page 437, section 16.3.6.5.2.4. as follows]

Table 827– Feedback Polling A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Feedback_Polling_A-MAP_IE (){ |  |  |
| A-MAP IE Type | 4 | Feedback Polling A-MAP IEFeedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback transmission is released. If d == 0b111, the pre-scheduled feedback transmission shall be valid until the BS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1. |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b000) { |  | Feedback allocation |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1. |
| HFA | 3 | HARQ feedback channel allocation for Dedicated UL allocation of MIMO Feedback |
| ACID | 3 | HARQ channel identifier |
| MEF | 1 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b0: SFBC 0b1: VE with Mt = 2 if Nt > 1, or VE with Mt = 1 if Nt = 1 Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Period (p) | 3 | Transmit feedback every 4p frame. The first report shall start at the next frame. |
| MIMO_feedback_IE_type | 1 | 0b0: feedback for single-BS MIMO operation 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | Single-BS MIMO feedback request |
| Transmit_Correlation_Matrix | 1 | 0b0: feedback of the transmit correlation matrix is indicated by CM 0b1: feedback of the quantized BS transmit correlation matrix only Transmit correlation matrix shall be feedback if CM = 0b1 |
| If (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antenna Nt at the ABS If Nt=2 (Any MFM): 0b0: 1 0b1: 2 If Nt =4 (Any MFM): 0b00: 1 0b01: 2 0b10: 3 0b11: 4 If Nt =8 (SU-MIMO MFM 0, 1, 2, 3, 4): 0b00: 1 0b01: 2 0b10: 4 0b11: 8 If Nt =8 (MU-MIMO MFM 5, 6, 7): 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| MFM | 3 | MIMO Feedback Mode for which the AMS shall transmit feedback |
| If (MFM = 2, 3, 5, 6) { |  | Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1< Num_best_subbands ≤ YSB |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | Feedback of CQI and CSI for CL SU and MU MIMO |
| CM | [TBD] | [TBD] |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  | Indication of report for OL region |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| else { |  | Multi-BS MIMO feedback request |
| TRU | 2 | Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

------------------------------------------------------- End of the Text -------------------------------------------------------