| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to ACID cycling method in Persistent allocation (16.2.7) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes the texts related to ACID cycling method in persistent scheduling |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to ACID cycling method in Persistent allocation (16.2.7)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the Persistent scheduling in the IEEE 802.16m DRAFT Amendment [1].

- Current ACID cycling method of persistent scheduling [1], [2]

  - Initial ACID = Value of ACID field included in DL/UL Persistent Allocation A-MAP IE.

  - N_ACID = Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1

  - Example : Initial ACID = 0, N_ACID = 3

Figure 1

- Changes of persistently assigned resource

  - To facilitate Link adaptation or to reduce resource holes

  - Signaling: DL/UL Persistent allocation A-MAP IE

  - In this case, if AMS has a pre-assigned persistent resource in the same subframe, the new persistent allocation replaces the original allocation.

    - That is, when an AMS receives the DL/UL Persistent Allocation A-MAP IE, the AMS can know whether this persistent allocation is new allocation or re-allocation.

  - N_ACID is not changed because the related system parameters for determining N_ACID are the same as those of the previous persistent allocation.

- Motivation

- In re-allocation procedure of persistent scheduling [2]

  - As mentioned in [2], when the re-allocation occurs, if there are not the consecutive ACIDs for re-allocation except for the ACIDs allocated in the previous persistent allocation, the ABS will set ACID field to the initial ACID of the previous persistent allocation. At that time, if the HARQ subpacket of the previous persistent allocation corresponding to initial ACID is retransmitted, two resources with the same ACID are allocated in the same subframe. It will result in confusion of the persistent scheduling.

- Proposal

If the persistently assigned resource is re-allocated, initial ACID and N_ACID used for re-allocation are set to initial ACID and N_ACID of the previous persistent allocation. When a DL/UL Persistent Allocation A-MAP IE is transmitted for re-allocation, the value of ACID field indicated by the DL/UL Persistent Allocation A-MAP IE is used for the first DL burst transmitted via the resource assigned by the DL/UL Persistent Allocation.

Figure 2

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] IEEE C802.16m-09/2555, “Proposed Text on Determining ACID for Persistent Allocation (15.2.7.1)”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Change1: Modify Section 16.2.7.1 as follows]

16.2.7.1 Allocation Mechanism

For persistent allocation in the DL/UL, the ABS shall transmit the DL/UL Persistent Allocation A-MAP IE. Allocation of the persistently assigned resource begins in the DL/UL AAI subframe that is referenced by the DL/UL Persistent Allocation A-MAP IE and repeats after an allocation period that is specified in the DL/UL Persistent Allocation A-MAP IE. The configuration parameters of the persistently allocated resource are maintained as per the DL/UL Persistent Allocation A-MAP IE. The values of ACID field and N_ACID field in the DL/UL Persistent Allocation A-MAP IE are used together to specify an implicit cycling of HARQ channel identifiers. The allocation period and number of ACIDs required for persistent operation are configured in the DL/UL Persistent Allocation A-MAP IE.

In order to facilitate link adaptation and avoid resource holes, the configuration parameters of a persistently allocated resource can be changed. To change a persistent assignment, the ABS shall transmit the DL Persistent Allocation A-MAP IE for DL reallocation and the UL Persistent Allocation A-MAP IE for UL reallocation respectively. If an AMS has an existing persistent allocation in a particular AAI subframe and receives a new persistent allocation in the same AAI subframe, the new persistent allocation replaces the original allocation (i.e., the original persistent allocation is de-allocated).

When the ABS sends a Persistent Allocation A-MAP IE to reallocate a persistently assigned resource, a different HARQ feedback channel must be assigned in the Persistent Allocation A-MAP IE used for reallocation. Reception of an ACK in the newly assigned HARQ feedback channel for the persistently assigned resource with the changed attributes will confirm that the reallocation A-MAP IE was received correctly.

When a persistently assigned resource is reallocated, the value of ACID field indentified by DL/UL Persistent Allocation A-MAP IE is used for the initial DL/UL burst transmitted via the resource indicated by DL/UL Persistent Allocation A-MAP IE. The initial ACID value and N_ACID value for re-allocation are set to the initial ACID value and N_ACID value of the previous persistent allocation.

[Change2: Modify the table 823 as follows]

Table 823: DL Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent Allocation A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| If (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation!= 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| Si | 4 | Index to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 In re-allocation, this ACID field indicates the HARQ channel identifier of initial HARQ subpacket identified by this Persistent Allocation A-MAP IE and the implicit cycling method is adjusted from this ACID value. The initial ACID value and N_ACID value for re-allocation are set to the initial ACID value and N_ACID value of previous persistent allocation. |
| Reserved | 1 | Reserved bit |
| } | - | - |
| } | - | - |

[Change3: Modify the table 824 as follows]

Table 824: UL Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent Allocation A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation Period != 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ |  | Parameters for CSM |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| else if ( TNS == Mt) { |  | Parameters without CSM |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| Reserved | 1 | Reserved bit |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation |
| ACID | 3 | HARQ channel identifier N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 In re-allocation, this ACID field indicates the HARQ channel identifier of initial HARQ subpacket identified by this Persistent Allocation A-MAP IE and the implicit cycling method is adjusted from this ACID value. The initial ACID value and N_ACID value for re-allocation are set to the initial ACID value and N_ACID value of previous persistent allocation. |
| Reserved | 2 | Reserved bits |
| } | - | - |
| } | - | - |

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------