| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS MAP and S-SFH update for E-MBS (16.3.6/16.9.2) |  |
| Date Submitted | 2009-01-14 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, HanGyu Cho Heejong Cho, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com , hgcho@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes the texts related to E-MBS MAP and S-SFH update for E-MBS |  |
| Purpose | For E-MBS DG discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS MAP and S-SFH update for E-MBS (16.3.6/16.9.2)

Jeongki Kim, Youngsoo Yuk, HanGyu Cho, Heejong Cho, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak

- LG Electronics

# Introduction

As described in the Section 16.3.6.5.1.2 in 16m Draft 3[1], an S-SFH SP2 carries the information for the DL frequency partitioning (e.g., DSAC/DFPC/DFPSC) which is information to determine E-MBS resource region and the S-SFH SP2 is transmitted with the periodically.

An AMS which subscribes an E-MBS in Idle mode awakes at the superframe which E-MBS MAP is transmitted, receives the E-MBS MAP and checks whether E-MBS bursts for the AMS are transmitted in a MSI.

If the AMS does not receive the changed S-SFH SP2 as shown in the Figure 1, the Idle mode AMS cannot decode the E-MBS MAP.

Figure 1

If the Idle mode AMS awakes every S-SFH SP2 transmission period, the power consumption of the AMS will be increased.

To remove this problem, an E-MBS MAP will contain S-SFH SP2 update indicator as shown in Table 1. The S-SFH SP2 update indicator indicates whether the changed S-SFH SP2s will be transmitted in a MSI or not. So if S-SFH SP2 update indicator is set to 1, AMSs which subscribe in Idle mode will wake up every S-SFH SP2 transmission period until the AMS receives the changed S-SFH SP2.

Table 1: E-MBS MAP Structure

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| S-SFH SP2 update indicator | 1 | Indicates whether the changed S-SFH SP2 will be transmitted in this MSI. If this field is set to 1, the time offset which the changed S-SFH SP2 is transmitted will be included in this E-MBS MAP. |
| If (S-SFH SP2 update indicator ==1) { |  | AMS shall wake up every S-SFH SP2 transmission period until the AMS receives the changed S-SFH SP2. |
| S-SFH SP2 transmission time offset | Variable | Indicates the superframe offset which the changed S-SFH SP2 is changed. The size of this field depends on MSI MSI == 0b00: 1 bits MSI == 0b01: 2 bits MSI == 0b10: 3 bits MSI == 0b11: 4 bits |
| } |  |  |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Text #1 ---------------------------------------------------------------------------

[Modify Section 16.3.6.2.3 as follows]

16.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries some configuration information for enhanced multicast broadcast service for one EMBS Zone. The E-MBS MAP is pointed by the AAI_E-MBS-CFG MAC Control Message described in 156.9.3.1and is transmitted in the first several resources of the E-MBS region in the beginning of the MSI.

Table 798: E-MBS MAP Structure

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| S-SFH SP2 update indicator | 1 | Indicates whether the changed S-SFH SP2 will be transmitted in this MSI. If this field is set to 1, the time offset which the changed S-SFH SP2 is transmitted will be included in this E-MBS MAP. |
| If (S-SFH SP2 update indicator ==1) { |  | AMS shall wake up every S-SFH SP2 transmission period until the AMS receives the changed S-SFH SP2. |
| S-SFH SP2 transmission time offset | Variable | Indicates the superframe offset which the changed S-SFH SP2 is changed. The size of this field depends on MSI MSI == 0b00: 1 bits MSI == 0b01: 2 bits MSI == 0b10: 3 bits MSI == 0b11: 4 bits |
| } |  |  |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

--------------Proposed Text #1 ---------------------------------------------------------------------------