| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the information for ranging frequency resource allocation (16.3.9.2.4) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Hwasun Yoo and Chiwoo Lim Samsung Electronics Co., Ltd. | hwasun.yoo@samsung.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement” |  |
| Abstract | The contribution proposes the text changes to the information for ranging frequency resource allocation of the 802.16m draft amendment, IEEE P802.16m/D3. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the information for ranging frequency resource allocation (16.3.9.2.4)

Hwasun Yoo and Chiwoo Lim
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of the information for ranging frequency resource allocation to be included in the 802.16m amendment. In the current draft amendment [1], the information for ranging frequency resource allocation is determined by the IDcell and the allocated number of subbands KSB according to following equation.

ISB = mod(IDcell, KSB)

However, KSB is cell common value not cell-specific value. The problem is that each cell may have a different value for number of subband based CRUs.

If we use KSB for the calculation of ISB, non-existing subband index can be obtained by calculation because total number of subbands available for ranging channel can be less than KSB.

- References

[1] IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” December 2009.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Modify the text on page 532 line 20-29 as following text]

The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands KSB according to the Equation (261), where IDcell is defined in 16.3.6.1.2 and KSB is defined in 16.3.8.2.1.

ISB = mod(IDcell, KSB) 									(261)

where ISB denotes the subband index (0,…, KSB -1) for ranging resource allocation among KSB subbands.

The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands YSB according to the Equation (261), where IDcell is defined in 16.3.6.1.2 and YSB is defined in 16.3.6.5.2.4.3 with exception of LSB-CRU,FPi  is the number of allocated subband CRUs in section 16.3.8.3.

ISB = mod(IDcell, YSB) 									(261)

where ISB denotes the subband index (0,…, YSB -1) for ranging resource allocation among YSB subbands.

[Remedy-2: Modify the text on page 535 line 27-36 as following text]

The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands KSB according to the Equation (261), where IDcell is defined in 16.3.6.1.2 and KSB is defined in 16.3.8.2.1.

ISB,s = mod(IDcell+1, KSB) 								(264)

where ISB,s denotes the subband index (0,…, KSB -1) for ranging resource allocation among KSB subbands.

The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands YSB according to the Equation (261), where IDcell is defined in 16.3.6.1.2 and YSB is defined in 16.3.6.5.2.4.3 with exception of LSB-CRU,FPi  is the number of allocated subband CRUs in section 16.3.8.3.

ISB,s = mod(IDcell+1, YSB) 								(264)

where ISB,s denotes the subband index (0,…, YSB -1) for ranging resource allocation among YSB subbands.

[Remedy-3: Correct a term on page 535 line 61 as follow]

KSB =1   YSB =1

---------------------------------------------------  Text End  ---------------------------------------------------