| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text changes to the S-SFH-IE in IEEE P802.16m/D3 (16.3.6.5.1.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Xianming Chen, Yanfeng Guan, Lei Zhang, Feng Xie, Yang Liu ZTE Corporation | Guan.yanfeng@zte.com.cn |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b |  |
| Abstract | The contribution proposes text changes to IEEE P802.16m/D3 |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text changes to the S-SFH-IE in IEEE P802.16m/D3 (16.3.6.5.1.2)

Xianming Chen, Yanfeng Guan, Lei Zhang, Feng Xie, Yang Liu

ZTE Corporation

- 1. Introduction

This contribution provides the text changes to S-SFH SPx IE in order to resolve the following issue.

If the indication information for multi-cell resource mapping part in SP2 is changed, that for cell-specific part in SP1 also need to be changed correspondingly. For example, assuming that initial “DFPSC” value in SP2 is 2, and initial “DCASi” value in SP1 also is 2, and if “DFPSC” value is changed into 1 subsequently, then it is impossible that “DCASi” value remains unchanged. That means, in this case, the information fields related to resource mapping in SP1 and SP2 need to be taken effect simultaneously.

For making each SP’s transmission period and update process independent, it is proposed all indication information for the resource mapping is packed into one SP1 or SP2.

- Proposed text with markups

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

- 2. References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”

- 3. Text proposal

------------------------------------------  Text Start  ---------------------------------------------------

Remedy #1 : Adopt the following modification in page376, line 60, Table 801 in section 16.3.6.5.1.2

Table 801 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| Start superframe offset where new SP1 information is used | 2 |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSB of 48 bit ABS MAC ID | 12 | Specifies the 12 least bit of ABS ID |
| Number of UL ACK/NACK channels per HARQ feedback region | 2 | Describes LHFB in 15.3.8.3.3.2.Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 MHz band, 6, 12, 18, 24 For 10 MHz band, 6, 12, 24, 30 For 20 MHz band, 12, 24, 48, 60 |
| Number of DL ACK/NACK channels per HF-A-MAP region | 2 | Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 MHz band, 4, 8, 12, 16 For 10 MHz band, 8, 16, 24, 32 For 20 MHz band, 16, 32, 48, 64 |
| Power control channel resource size indicator | 2 |  |
| Non-user specific A-MAP location | 1 | 0b0: reuse 1 region 0b1: reuse 3 region |
| A-A-MAP MCS selection | 1 | 0b0: QPSK 1/2 and QPSK 1/4 can be used for assignment A-MAP. 0b1: QPSK 1/2 and QPSK 1/8 can be used for assignment A-MAP. |
| DCAS_SB0 | 5/4/3 | See 16.3.5.3.1 DL CRU/DRU allocation For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| DCAS_MB0 | 5/4/3 | See 16.3.5.3.1 DL CRU/DRU allocation For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| DCAS_i | 3/2/1 | See 16.3.5.3.1 DL CRU/DRU allocation For 20 MHz, 3 bits For 10 MHz, 2 bits For 5 MHz, 1 bit |
| UCAS_SB0 | 5/4/3 | See 16.3.8.3.1UL CRU/DRU allocation For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| UCAS_MB0 | 5/4/3 | See 16.3.8.3.1UL CRU/DRU allocation For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| UCAS_i | 3/2/1 | See 16.3.8.3.1UL CRU/DRU allocation For 20 MHz, 3 bits For 10 MHz, 2 bits For 5 MHz, 1 bits |
| DSAC | 5/4/3 | See 16.3.5.2.3 DL Frequency partitioning For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| DFPC | 4/3/3 | See 16.3.5.2.3 DL Frequency partitioning For 20 MHz, 4 bits For 10 MHz, 3bits For 5 MHz, 3 bits |
| DFPSC | 3/2/1 | See 16.3.5.2.3 DL Frequency partitioning For 20 MHz, 3 bits For 10 MHz, 2 bits For 5 MHz, 1 bits |
| USAC | 5/4/3 | See 16.3.8.2.3 UL Frequency partitioning For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| UFPC | 4/3/3 | See 16.3.8.2.3 UL Frequency partitioning For 20 MHz, 4 bits For 10 MHz, 3 bits For 5 MHz, 3 bits |
| UFPSC | 3/2/1 | See 16.3.8.2.3 UL Frequency partitioning For 20 MHz, 3 bits For 10 MHz, 2bits For 5 MHz, 1 bits |
| Ranging channel allocation periodicity | 2 | Indicates the periodicity of ranging channel allocation according to the Table 890. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 893. The range of values is 0 ≤ OSF ≤ 3 |
| If ( FDM-based UL PUSC zone support ) { |  | Indicates the WirelessMAN-OFDMA support in the UL with FDM mode, when the Frame Configuration Index in S-SFH SP2 is equal to - 5 or 7 or 9 for 5/10/20MHz channel bandwidth according to Table 771 - 4 or 6 or 8 for 8.75MHz channel bandwidth according to Table 772 - 3 (with CP=1/8) for 7MHz channel bandwidth according to Table 773. |
| Start code information of ranging channel | 4 | Indicates the kns which is the parameter for start of code group (S). S= 16×kns+1 The range of values is 0 ≤ kns ≤ 15. |
| Ranging preamble code partition information | 4 | Indicates the number of initial, handover and periodic codes (N, O and M) according to the Table [TBD]. |
| Ranging channel location in the frequency domain | 3 | Indicate the start distributed LRU index for ranging channel. |
| } else { |  |  |
| Start code information of ranging channel for non-synchronized AMSs | 4 | Indicates the kns which is the parameter controlling the start root index of ranging preamble codes (rns0). rns0(kns)=4×kns+1 for ranging channel format 0. rns0(kns)=16×kns+1 for ranging channel format 1. The range of values is 0 ≤ kns ≤ 15. |
| Ranging preamble code partition information for non-synchronized AMSs | 4 | Indicates the number of initial and handover ranging preamble codes (NIN and NHO) according to the Table 889. |
| Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs | 2 | Indicates the number of cyclic shifted codes per root index (Mns) for ranging preamble codes according to the Table 888. |
| Ranging channel formats for non-synchronized AMSs | 1 | Indicates the ranging channel formats number of Table 883. |
| } |  |  |
| ABS EIRP | 7 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for any new initial entry |
| Uplink AAI subframe for sounding | 3 | This value represents UL AAI subframe indexes where sounding symbols are located. |
| Reserved | TBD |  |
| } |  |  |

Remedy #2 : Adopt the following modification in page379, line 21, Table 802 in section 16.3.6.5.1.2

Table 802 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Start superframe offset where new SP2 information is used | 2 |  |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table 771, Table 772, and Table 773 |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency | 6 |  |
| UL bandwidth | 3 |  |
| } |  |  |
| MSB bytes of 48 bit ABS MAC ID | 36 | Specifies 36 MSB of BS ID |
| MAC protocol revision | 4 | Version number of AAI supported on this channel |
| DSAC | 5/4/3 | See 16.3.5.2.3 DL Frequency partitioning For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| DFPC | 4/3/3 | See 16.3.5.2.3 DL Frequency partitioning For 20 MHz, 4 bits For 10 MHz, 3bits For 5 MHz, 3 bits |
| DFPSC | 3/2/1 | See 16.3.5.2.3 DL Frequency partitioning For 20 MHz, 3 bits For 10 MHz, 2 bits For 5 MHz, 1 bits |
| USAC | 5/4/3 | See 16.3.8.2.3 UL Frequency partitioning For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| UFPC | 4/3/3 | See 16.3.8.2.3 UL Frequency partitioning For 20 MHz, 4 bits For 10 MHz, 3 bits For 5 MHz, 3 bits |
| UFPSC | 3/2/1 | See 16.3.8.2.3 UL Frequency partitioning For 20 MHz, 3 bits For 10 MHz, 2bits For 5 MHz, 1 bits |
| MS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed AMS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| EIRxPIR,min | 5 |  |
| reserved |  |  |
| } |  |  |

----------------------------------------------  Text End  ---------------------------------------------------