| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimized and Simplified Resource Allocation field in Subband Assignment A-MAP IE |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yanfeng Guan, Xiangyu Liu, Ying Liu, Huiying Fang ZTE Corporation | guan.yanfeng@zte.com.cn |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D3” 15.3.6 Downlink Control Structure & 15.3.8 Uplink Control Structure |  |
| Abstract | This contribution proposes text optimizing the “Resource Allocation” filed in the Subband A-A-MAP IEs. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimized and Simplified Resource Allocation field in Subband Assignment

A-MAP IE

Yanfeng Guan, Xiangyu Liu, Ying Liu, Huiying Fang

ZTE Corporation

- Introduction

In the current 802.16m draft amendment [1], four types of Assignment A-MAP IEs are defined and used for dynamic scheduling. They are DL/UL Basic Assignment A-MAP IEs for an allocation of contiguous LRUs and DL/UL Sub-band Assignment A-MAP IEs for an allocation of subband-based LRUs in DL/UL subframes. For these four IEs, the main difference is the interpretation for the “Resource Index” field.

- Problem

For RA field in DL/UL Subband Assignment A-MAP IEs, the indexing method means that the ABS can only use two different resource allocation granularities (a subband and a half subband) for the different bandwidth or different number of subband. But there are some problems:

  - The resource allocation of odd discontinuous SLRUs should be supported in order to guarantee the whole system efficiency. For example, if the proper allocation size is odd (for example 3 SLRUs), ABS can only assign 2 or 4 SLRUs for AMS because SLRU indexing method in Draft 3 only support even SLRUs resource allocation. When the assignment is 2 SLRUs, the actual coding rate will be remarkably reduced, and the error bit rate will be increased. Otherwise, the assignment maybe is 4 SLRUs, the resource scheduling efficiency will decrease because 3 SLRUs are enough under the given CQI and burst size of the AMS. For all the odd number except 1, 3 SLRUs or 5SLRUs should be especially supported because these two sizes will be often used in the resource scheduling process.

  - The resource allocation of odd discontinuous SLRUs should be supported to meet the channel coding. The channel coding method uses the rate matching mechanism, and the actual and final coding rate mainly depends on the nominal MCS for CQI feedback from the AMS.

Table 1 Nominal MCS and the Ratio of the adjacent two MCS Rate

| MCS Index | Modulation | Code Rate | Ratio of the adjacent two MCS Rate |
| --- | --- | --- | --- |
| 0000 | QPSK | 31/256 | 48/31=1.55 |
| 0001 | QPSK | 48/256 | 71/48=1.48 |
| 0010 | QPSK | 71/256 | 101/71=1.42 |
| 0011 | QPSK | 101/256 | 135/101=1.34 |
| 0100 | QPSK | 135/256 | 171/135=1.27 |
| 0101 | QPSK | 171/256 | 204/171=1.19 |
| 0110 | 16QAM | 102/256 | 128/102=1.25 |
| 0111 | 16QAM | 128/256 | 155/128=1.21 |
| 1000 | 16QAM | 155/256 | 184/155=1.19 |
| 1001 | 16QAM | 184/256 | 810/736=1.10 |
| 1010 | 64QAM | 135/256 | 157/135=1.16 |
| 1011 | 64QAM | 157/256 | 181/157=1.15 |
| 1100 | 64QAM | 181/256 | 205/181=1.13 |
| 1101 | 64QAM | 205/256 | 225/205=1.10 |
| 1110 | 64QAM | 225/256 | 237/225=1.05 |
| 1111 | 64QAM | 237/256 | / |

Table 2 Ratio of Difference Size and Expected Size

| Expected Allocation Size | Assigned Allocation Size | Difference size | Ratio of Difference Size and Expected Size |
| --- | --- | --- | --- |
| 3 SLRUs | 2 SLRUs | 1 SLRU | 33.3% |
| 3 SLRUs | 4 SLRUs | 1 SLRU | 33.3% |
| 5 SLRUs | 4 SLRUs | 1 SLRU | 20.0% |
| 5 SLRUs | 6 SLRUs | 1 SLRU | 20.0% |
| 7 SLRUs | 6 SLRUs | 1 SLRU | 14.3% |
| 7 SLRUs | 8 SLRUs | 1 SLRU | 14.3% |

For a given MCS, we can see that the maximum possible ratio of the adjacent two Nominal MCS rate is 1.55, and more ratios are less than 1.33, even 1.25. That means if AMS only needs 3 SLRUs and ABS assign 2 or 4 SLRUs, the coding matching mechanism will leads that the final coding rate is not best or most efficient for the AMS, because the change of resource size has been more than 20%, even 33%.

  - For the smaller system bandwidths, more indexing bits are reserved and wasted, so we can enhance the indexing ability by using those reserved bits. Most importance is the total Subband number is low, for example, the subband number is less than 4 for 5MHz system, the SLRU resource is rare and costful compared to the system with wider bandwidth. In this case, more granularities are preferred compared only even number of SLRUs. Figure 1 and 2 shows the enhanced indexing method and that in Draft 3.

Figure 1- SLRU Indexing Method in Draft 3 when YSB< 4

Figure 2- Enhanced SLRU Indexing Method in Draft 3 when YSB< 4

Figure 3- Enhanced SLRU Indexing Method in Draft 3 when YSB= 4

Obviously, the enhanced method has more strong indexing ability than that in Draft 3. It can index any needed case in actual resource scheduling process.

  - For making the actual system simpler, the system with wider bandwidth prefers to reuse the code or module in the lower bandwidth system. Regarding this principle, the SLRU Indexing method for 10MHz system should use the same method as that for 5MHz system when YSB< =4, because YSB <= 4 in 5MHz system.

  - For making the actual system simpler, another way is trying to reduce the indexing case because of the system bandwidth or the value YSB. Therefore, if the two different indexing methods are used for YSB1 and YSB2, we can only use one same method when we can get the same indexing ability. For example, in 10MHz, we can use the method only for YSB=6 to indicate the SLRU allocation when YSB=5 or 6.

Figure 4- Enhanced SLRU Indexing Method in Draft 3 when 5=<YSB< =6

  - For making the actual system simpler, the indexing for 20MHz should be simplified. ONLY ONE IE is needed. If YSB <= 10, it will be same as the indexing for 10MHz, otherwise, only a Subband granularity is supported. In addition, the maximum subband number is limited to 20, not 21, 16 DLRUs for control channel and possible data are necessary.

  - For simplifying all the related text, all the pictorially illustrations are removed; only the equivalent equation is reserved.

Based on the considerations above, we propose some enhanced modifications on the “Resource Allocation” field in DL/UL Basic/Subband-Assignment A-MAP IE.

- References

[1] "DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface", P802.16m/D3 Dec. 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r7, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/1327r1, “Proposed Changes to the DL and UL A-MAP Information Element Types in AWD ”

[6] IEEE 802.16m-09/1331r2, “Proposed AWD text specifying the “Resource Allocation” field in the A-MAP IEs AWD ”

[7] IEEE 802.16m-09/1334r1, “AWD Proposal for DL/UL Resource Indexing in IEEE 802.16m ”

[8] IEEE 802.16m-09/1516r “Proposed “Resource Allocation” Field in Basic Assignment A-MAP IE in IEEE802.16m ”

[9] IEEE 802.16m-09/1814r “Proposed “Resource Allocation” Field in Basic Assignment A-MAP IE in IEEE802.16m ”

- Proposed Text

Remedy 1: Modify the text from line 15 to 57 on page 404 in the 15.3.6.5.2.4.3 DL Sub-band Assignment A-MAP IE as follows:

------------------------------------------------------ Proposed Text 1 Starts -------------------------------------------------

Nominal Channel Bandwidth = 5MHz

For a 5 MHz system:

- A single instance of a resource allocation shall be made using a single IE.

- It may be noted that YSB <= 4 for a 5 MHz system. The AMS shall interpret the RA field as defined below.

  - Each of the j bit-positions in the RA field, 0 <= j < 11, indicates the allocation or non-allocation of 1or 2 SLRUs within a particular sub-band.

  - When YSB < 4, pPosition j in the RA field corresponds to the 1or 2 SLRUs with indices k such that or 3(/2) =j.

c) Denoting RA[j] as the jth position in the RA field, 0 <= j < 11, with j = 0 being the LSB,

RA[j]=1=> The 1or 2 SLRUs with indices k such that or 3(/2) =j have been allocated.

RA[j]=0 => The 1or 2 SLRUs with indices k such that or 3(/2) =j have not been allocated.

c) When YSB=4, the interpretation of RA field for SB index 0, 1 and 2 is same as the interpretation when YSB < 4. RA[9] and RA[10] in the RA field indicate the allocation or non-allocation of 2 SLRUs within a sub-band 3.

RA[j]=1=> The 2 SLRUs with indices k such that =j have been allocated.

RA[j]=0 => The 2 SLRUs with indices k such that =j have not been allocated.

------------------------------------------------------ Proposed Text 1 Ends----------------------------------------------

Remedy 2: Replace the subband indexing for 10MHz from line 60 on page 404 to line 60 on page 408 as follows:

------------------------------------------------------ Proposed Text 2 Starts -------------------------------------------------

Nominal Channel Bandwidth=10MHz

For a 10 MHz system a single instance of a resource allocation shall be made using a single IE. It should be noted that YSB<= 10 for a 10 MHz system. For a particular value of YSB, the AMS shall interpret the bits in the Resource Allocation field as indicated by the row corresponding to that value of YSB in Table 814.

Table 814- Interpretation of the RA Field in a 10 MHz.

| Total # of sub-bands, YSB | RA Field Interpretation |  |
| --- | --- | --- |
| 6<YSB <11 | The 11 LSB bits of the RA field are denoted as the “Resource Indexing Field” (RIF). Denote RIF[j] as the jth bit position in the RIF, 0 <= j < 11, with j = 0 being the LSB. Each of the RIF[j], 0 <= j < 10, indicates the allocation or non-allocation of all 4 SLRUs within a particular sub-band. RIF[j] = 1: The 4 SLRUs with indices k such that = j have been allocated. RIF[j]= 0: The 4 SLRUs with indices k such that = j have been not allocated |  |
| 4<YSB<7 | The 2 MSB bits of the RA field are denoted as the “Indication Type field” (ITF). The 9 LSB bits of the RA field are denoted as the “Resource Indexing Field” (RIF). Denote RIF[j] as the jth bit position in the RIF, 0 <= j < 9, with j = 0 being the LSB. |  |
|  | ITF == 00 | Each of the RIF[j], 0 <= j < 6, indicates the allocation or non-allocation of all 4 SLRUs within a particular sub-band. RIF[j] = 1: The 4 SLRUs with indices k such that = j have been allocated. RIF[j] = 0:The 4 SLRUs with indices k such that = j have been not allocated |
|  | ITF == 01 | The 9 bits in the RIF index into the 9 half sub-bands (out of 12 or 10) with the higher indices. Each of the RIF[j], 0 <= j < 9, indicates the allocation or non-allocation of the 1st or last 2 CRUs within a particular sub-band. RIF[j] = 1: The 2 SLRUs with indices k such that= j +3 have been allocated. RIF[j] = 0: The 2 SLRUs with indices k such that= j+3 have not been allocated. |
|  | ITF == 10 | The 9 bits in the RIF index into the 9 half sub-bands (out of 12 or 10) with the lower indices. Each of the RIF[j], 0 <= j < 9, indicates the allocation or non-allocation of the 1st or last 2 SLRUs within a particular sub-band. RIF[j] = 1: The 2 SLRUs with indices k such that= j have been allocated. RIF[j] = 0: The 2 SLRUs with indices k such that= j have not been allocated. |
| YSB <= 4 | Reference to the nominal channel bandwidth=5MHz. |  |

----------------------------------------------------- Proposed Text 2 Ends ---------------------------------------------------

Remedy 3: Replace the subband indexing for 20MHz from line 62 on page 408 to line 65 on page 423 as follows:

----------------------------------------------------- Proposed Text 3 Starts---------------------------------------------------

nominal channel bandwidth=20MHz

For a 20 MHz system a single instance of a resource allocation shall be made using a single IE. It should be noted that YSB <= 20 for a 20 MHz system. For a particular value of YSB, the AMS shall interpret the bits in the Resource Allocation field as indicated by the row corresponding to that value of YSB in Table 815.

Table 815- Interpretation of the RA Field in a 20 MHz system

| Total # of sub-bands, YSB | RA Field Interpretation |  |
| --- | --- | --- |
| 10<YSB <21 | The 1 MSB bit of the RA field is denoted as the “Indication Type field” (ITF). The 10 LSB bits of the RA field are denoted as the “Resource Indexing Field” (RIF). Denote RIF[j] as the jth bit position in the RIF, 0 <= j < 10, with j = 0 being the LSB. |  |
|  | ITF==0 | Each of the RIF[j], 0 <= j < 10, indicates the allocation or non-allocation of all 4 SLRUs within a particular sub-band. RIF[j] = 1: The 4 SLRUs with indices k such that = 2j have been allocated. RIF[j]= 0:The 4 SLRUs with indices k such that = 2j have been not allocated |
|  | ITF==1 | Each of the RIF[j], 0 <= j < 10, indicates the allocation or non-allocation of all 4 SLRUs within a particular sub-band. RIF[j] = 1: The 4 SLRUs with indices k such that =2j+1 have been allocated. RIF[j]= 0:The 4 SLRUs with indices k such that = 2j+1 have been not allocated |
| YSB < 11 | Refer to the nominal channel bandwidth=10 MHz. |  |

----------------------------------------------------- Proposed Text 3 Ends-------------------------------------------------

Remedy 4: Remove the item DSAC= 21 in line 46 on page 317.