| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Parameter Configuration for HARQ operations in 802.16m (16.2.14.2.1.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou and Paul Cheng MediaTek Inc. | E-mail: yihshen.chen@mediatek.com, kelvin.chou@mediatek.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot#30b” |  |
| Abstract | This contribution proposes parameter configuration for HARQ operations |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Parameter Configuration for HARQ Operations in 802.16m

Yih-Shen Chen, Kelvin Chou and Paul Cheng

MediaTek Inc.

- Introduction

In current IEEE 802.16m specification [1], asynchronous retransmission scheme and synchronous retransmission scheme are applied in DL HARQ and UL HARQ, respectively. For DL HARQ, two parameters, T_ReTx_Interval and N_MAX_ReTx, are not defined yet. For UL HARQ, N_MAX_ReTx is not defined yet. In this contribution, we discuss the parameter configurations for HARQ operation and provide the cleanup text.

- Discussion on HARQ Parameter Configuration

Based on Section 15.2.14.2.1.1, “T_ReTx_Interval” is defined as the maximal delay between two consecutive HARQ transmissions of the same data burst. The value is configurable and related to delay requirement of a service flow and BS’ scheduler algorithm. Accordingly, there are two options:

- Option-1: configuration per service flow

- Option-2: configuration per BS

Option-1 provides the most flexibility to meet the delay requirements of each service flow. On the other hand, option-2 is simpler from the perspective of scheduler implementation because the delay bound between two consecutive retransmissions is the same for all the service flows. The maximal possible delay of a DL HARQ transmission can be calculated by

MaxDLHARQDelay=T_ReTxInterval∗NMAX_ReTx

The overall delay can be controlled by T_ReTx_Interval or N_MAX_ReTx. Therefore, it is recommended to adopt Option-2 for T_ReTx_Interval and broadcast the parameter value in AAI_SCD.

- Proposed Text

Insert the following text into section 16.2.14.2.1.1

------------------------------------------- Start of text #1----------------------------------------------

16.2.14.2 Generic HARQ signaling and timing

16.2.14.2.1 HARQ Signaling

16.2.14.2.1.1 Downlink

….

In DL, the maximum number of total HARQ channels per AMS is 16. The delay between two consecutive HARQ transmissions of the same data burst shall not exceed the maximum [T_ReTx_Interval] . The number of retransmissions of the same data burst shall not exceed the maximum [DL_N_MAX_ReTx]. Those HARQ parameters are specified in AAI_SCD.

-------------------------------------------End of text #1---------------------------------------------

Insert the following text into section 16.2.14.2.1.2

------------------------------------------- Start of text #2----------------------------------------------

16.2.14.2 Generic HARQ signaling and timing

16.2.14.2.1 HARQ Signaling

16.2.14.2.1.2 Uplink

….

In UL, the maximum number of total HARQ channels per AMS is 16. The number of retransmissions of the same data burst shall not exceed the maximum [UL_N_MAX_ReTx], which is specified in AAI_SCD.

-------------------------------------------End of text #2---------------------------------------------

Insert the following text into section 16.2.3.28

------------------------------------------- Start of text #3----------------------------------------------

15.2.3.28 AAI_System Configuration Descriptor (SCD) Message

….

ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change	INTEGER (0 ~ 15)

BS_Restart_Count		INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ON		INTEGER (0..1) 	OPTIONAL

OL-Rank1-Configuration	INTEGER (0..7) 	OPTIONAL

SB-OL-Region-2Size	INTEGER (0..15) 	OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0		INTEGER (0..15) 	OPTIONAL

GAMMA_IOT_FP1		INTEGER (0..15) 	OPTIONAL

GAMMA_IOT_FP2		INTEGER (0..15) 	OPTIONAL

GAMMA_IOT_FP3		INTEGER (0..15) 	OPTIONAL

Alpha (a)			INTEGER (0.. 7) 	OPTIONAL

Beta (ß)				INTEGER (0..1) 	OPTIONAL

SINRmin			INTERGER (0..15) 	OPTIONAL

T_ReTx_Interval			FFS

DL N Max_ReTx			INTEGER (4..8)

UL N Max_ReTx			INTEGER (4..8)

}-- ASN1STOP

-------------------------------------------End of text #3---------------------------------------------

Reference

[1] IEEE 802.16m/D3, Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface