| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Parameter Configuration for HARQ operations in 802.16m (16.2.14.2.1.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou and Paul Cheng MediaTek Inc. Mihyun Lee Samsung Electronics | E-mail: yihshen.chen@mediatek.com, kelvin.chou@mediatek.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.14.2.1.1” |  |
| Abstract | This contribution proposes parameter configuration for HARQ operations |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Parameter Configuration for HARQ Operations in 802.16m

Yih-Shen Chen, Kelvin Chou and Paul Cheng

MediaTek Inc.

Mihyun Lee 
Samsung Electronics

- Introduction

In current IEEE 802.16m specification [1], asynchronous retransmission scheme and synchronous retransmission scheme are applied in DL HARQ and UL HARQ, respectively. For DL HARQ, two parameters, T_ReTx_Interval and N_MAX_ReTx, are not defined yet. For UL HARQ, N_MAX_ReTx is not defined yet. In this contribution, we discuss the parameter configurations for HARQ operation and provide the cleanup text.

- Discussion on HARQ Parameter Configuration

Based on Section 15.2.14.2.1.1, “T_ReTx_Interval” is defined as the maximal delay between two consecutive HARQ transmissions of the same data burst. The value is configurable and related to delay requirement of a service flow and BS’ scheduler algorithm. Accordingly, there are two options:

- Option-1: configuration per service flow

- Option-2: configuration per BS

Option-1 provides the most flexibility to meet the delay requirements of each service flow. On the other hand, option-2 is simpler from the perspective of scheduler implementation because the delay bound between two consecutive retransmissions is the same for all the service flows. The maximal possible delay of a DL HARQ transmission can be calculated by

MaxDLHARQDelay=T_ReTxInterval∗NMAX_ReTx

The overall delay can be controlled by T_ReTx_Interval or N_MAX_ReTx. Therefore, it is recommended to adopt Option-2 for T_ReTx_Interval and broadcast the parameter value in AAI_SCD.

- Proposed Text

Insert the following text into section 16.2.14.2.1.2

------------------------------------------- Start of text #1----------------------------------------------

[Remedy#1: Modify the text, line 41 in page 228, section 16.2.14.2.1]

16.2.14.2.1 HARQ Signaling

16.2.14.2.1.1 Downlink

Upon receiving a DL Basic Assignment A-MAP IE, AMS attempts to receive and decode the data burst as allocated to it by the DL Basic Assignment A-MAP IE. If the decoding is successful, AMS shall send a positive acknowledgement to ABS; otherwise, AMS shall send a negative acknowledgement to ABS.

The process of retransmissions shall be controlled by ABS using the ACID and AI_SN fields in the DL Basic Assignment A-MAP IE. If the AI_SN field for the ACID remains same between two HARQ bursts allocation, it indicates retransmission. Through the DL Basic Assignment A-MAP IE for retransmission, ABS may allocate different resource allocation and transmission format. If AI_SN field for the ACID is toggled, i.e., from 0 to 1 or vice versa, it indicates the transmission of a new HARQ burst, and the retransmission process for the previous HARQ burst with the same ACID is terminated if the retransmission process is in progress.

In DL, the maximum number of total HARQ channels per AMS is 16. The delay between two consecutive HARQ transmissions of the same data burst shall not exceed the maximum [T_ReTx_Interval]. The number of retransmissions of the same data burst shall not exceed the maximum [ DL N_MAX_ReTx]. The values for DL N_MAX_ReTx are 4 or 8. The ABS shall configure a maximum number of UL HARQ retransmission via S-SFH SP1 IE.

The timing for transmitting DL Basic Assignment A-MAP IE, DL HARQ subpacket, and HARQ feedback in UL are described in 16.2.14.2.2.

16.2.14.2.1.2 Uplink

Upon receiving a UL Basic Assignment A-MAP IE, AMS shall transmit the subpacket of HARQ data burst through the resource assigned by the UL Basic Assignment A-MAP IE.

ABS shall attempt to decode the data burst. If the decoding is successful, ABS shall send a positive acknowledgement to AMS; otherwise, ABS shall send a negative acknowledgement to AMS.

Upon receiving the negative acknowledgement, AMS shall trigger retransmission procedure.

In the retransmission procedure, if AMS does not receive a UL Basic Assignment A-MAP IE for the HARQ data burst in failure, AMS shall transmits the next subpacket through the resources assigned to the latest subpacket transmission with the same ACID. A UL Basic Assignment A-MAP IE may be sent to signal control information for retransmission with the corresponding ACID and AI_SN being not toggled. Upon receiving the UL Basic Assignment A-MAP IE, AMS shall perform the HARQ retransmission as instructed in this UL Basic Assignment A-MAP IE. As an example, ABS may change the resource index of the HARQ data burst or may command to skip retransmission in the corresponding AAI subframe. If AI_SN field for the ACID is toggled, i.e., from 0 to 1 or vice versa, it indicates the transmission of a new HARQ burst, and the retransmission process for the previous HARQ burst with the same ACID is terminated if the retransmission process is in progress.

In UL, the maximum number of total HARQ channels per AMS is 16. The number of retransmissions of the same data burst shall not exceed the maximum. [UL N_MAX_ReTx]. The values for UL N_MAX_ReTx are 4 or 8. The ABS shall configure a maximum number of UL HARQ retransmission via S-SFH SP1 IE.

The timing for transmitting UL Basic Assignment A-MAP IE, UL HARQ subpacket, and HARQ feedback in DL are described in 16.2.14.2.2.

[Remedy #2: Add the following parameter into Table 801 in page 376, section 16.3.6.5.1.2]

Table 801– S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| ... |  |  |
| UL N_MAX_ReTx | 1 | Specifies the maximum retransmission number for UL HARQ 0b0: 4 0b1: 8 |
| DL N_MAX_ReTx | 1 | Specifies the maximum retransmission number for DL HARQ 0b0: 4 0b1: 8 |
| } |  |  |

-------------------------------------------End of text #2---------------------------------------------

Insert the following text into section 16.2.3.28

------------------------------------------- Start of text #3----------------------------------------------

15.2.3.28 AAI_System Configuration Descriptor (SCD) Message

….

ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change	INTEGER (0 ~ 15)

BS_Restart_Count		INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ON		INTEGER (0..1) 	OPTIONAL

OL-Rank1-Configuration	INTEGER (0..7) 	OPTIONAL

SB-OL-Region-2Size	INTEGER (0..15) 	OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0		INTEGER (0..15) 	OPTIONAL

GAMMA_IOT_FP1		INTEGER (0..15) 	OPTIONAL

GAMMA_IOT_FP2		INTEGER (0..15) 	OPTIONAL

GAMMA_IOT_FP3		INTEGER (0..15) 	OPTIONAL

Alpha (a)			INTEGER (0.. 7) 	OPTIONAL

Beta (ß)				INTEGER (0..1) 	OPTIONAL

SINRmin			INTERGER (0..15) 	OPTIONAL

T_ReTx_Interval			INTEGER (1..8)

}-- ASN1STOP

-------------------------------------------End of text #3---------------------------------------------

Reference

[1] IEEE 802.16m/D2, Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface