| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging Channel Allocation for 802.16m (16.3.6.5.2.4) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yih-Shen Chen Kelvin Chou Pei-Kai Liao MediaTek Inc. Daniel Cohen Alvarion | Yihshen.Chen@mediatek.com Peikai.Liao@mediatek.com Kelvin.Chou@mediatek.com daniel.cohn@alvarion.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot#30b” |  |
| Abstract | This contribution proposes method for ranging channel allocation |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging Channel Allocation for 802.16m

Yih-Shen Chen, Kelvin Chou, Pei-Kai Liao

MediaTek Inc.

Daniel Cohen

Alvarion

- Introduction

In current IEEE 802.16m specification [1], the channel allocation information for initial/handover ranging and bandwidth request ranging are defined in SP1 IE and SP3 IE of SFH, respectively. An AMS who intends to perform the ranging operations shall obtain the SFH information first. Since the information is transmitted over SFH, the ranging channel allocation is semi-static; that is, the ranging channels are allocated in a pre-defined region unless SFH is updated. Actually, it is also stated in section 16.3.9.2.4.1 that

“….The ranging channel for handover ranging can also be allocated by AMAP based on ABS scheduling decision

in any AAI subframe, except the AAI subframe that has already been used for a regular allocation…”

Unfortunately, in current specification, the corresponding A-AMP IE has not been defined yet. We, therefore, are motivated to propose a new A-MAP IE for ranging channel allocation, named Ranging Channel Allocation A-MAP IE. The proposed IE can be applied to all kinds of ranging purposes, including unsynchronized ranging, synchronized ranging and bandwidth request ranging. We think the A-MAP IE can provide BS more scheduling flexibility to allocate ranging channels, in addition to static configuration.

Proposed text:

Insert the following text into section 16.3.6.5.2.4

-------------------------------------------------------------------- Start of text #1-------------------------------------------------------

16.3.6.5.2.4 Assignment A-MAP IE

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | DL Subband Assignment A-MAP IE |
| 0b0011 | UL Subband Assignment A-MAP IE |
| 0b0100 | Feedback Allocation A-MAP IE |
| 0b0101 | UL Sounding Command A-MAP IE |
| 0b0110 | CDMA Allocation A-MAP IE |
| 0b0111 | DL Persistent Allocation A-MAP IE |
| 0b1000 | UL Persistent Allocation A-MAP IE |
| 0b1001 | DL Group Resource Allocation A-MAP IE |
| 0b1010 | UL Group Resource Allocation A-MAP IE |
| 0b1011 | Feedback Polling A-MAP IE |
| 0b1100 | BR-ACK A-MAP IE |
| 0b1101 | Broadcast Assignment A-MAP IE |
| 0b1110 | Reserved Ranging Channel Allocation A-MAP IE |
| 0b1111 | Reserved |

--------------------------------------------------------------------End of text #1---------------------------------------------------------

-------------------------------------------------------------------- Start of text #2-------------------------------------------------------

16.3.6.5.2.4.14 Ranging Channel Allocation A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP() { |  |  |
| A-MAP IE Type | 4 | Ranging Channel Allocation A-MAP IE |
| Ranging Channel Type | 2 | 0b00: Unsynchronized ranging channel 0b01: Synchronized ranging channel 0b10: BR ranging channel 0b11: Reserved |
| Number of Ranging Opportunities (N) | 2 |  |
| For(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Sub-band Index | 5 |  |
| } |  |  |
| Reserved |  |  |
| } |  |  |

--------------------------------------------------------------------End of text #2---------------------------------------------------------

-------------------------------------------------------------------- Start of text #3-------------------------------------------------------

16.3.9.2.4.1 Ranging channel for non-synchronized AMSs

…..

The ranging channel for handover ranging can also be allocated by AMAP based on ABS scheduling decision

in any AAI subframe, except the AAI subframe that has already been used for a regular allocation.

--------------------------------------------------------------------End of text #3---------------------------------------------------------

Reference

[1] IEEE 802.16m/D3, Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface