| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal on the AAI_RNG-ACK Message for Unicast (16.2.3.3) |  |
| Date Submitted | 2009-1-10 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30b on the Draft Amendment (IEEE P802.16m/D3) |  |
| Abstract | Proposed text on the AAI_RNG-ACK message format for unicast transmission |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal on the AAI_RNG-ACK Message for Unicast (16.2.3.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Introduction

This contribution clarifies the description and parameters of AAI_RNG-ACK message to accommodate its unsolicited unicast to a specific user.

Proposed Text

In the P802.16m/D3, page 46, line 40, modify the text as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.2 AAI_RNG-RSP

An AAI_RNG-RSP, to which HARQ operation is applied, shall be transmitted by the ABS in response to a received AAI_RNG-REQ. In addition, it may also be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. As a result, the AMS shall be prepared to receive an AAI_RNG-RSP at any time, not just following an AAI_ RNG-REQ transmission.

--------------------------------------------------  Text End -----------------------------------------------------------------

In the P802.16m/D3, page 48, line 37, modify the text and Table 676 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.3 AAI_RNG-ACK

The AAI_RNG-ACK message is sent by the ABS in response to the CDMA ranging request during initial ranging, period ranging and HO ranging in order to provide PHY-level adjustment (e.g. timing offset, power level and frequency offset). In addition, it may also be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. As a result, the AMS shall be prepared to receive an AAI_RNG-ACK at any time, not just following a ranging preamble transmission. Upon reception of ranging codes, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE using broadcast STID. When the AAI_RNG-ACK message is sent unsolicitedly to specific user, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE with unicast STID. When the AMS receives an unsolicited AAI_RNG-ACK message, it shall reset the periodic ranging timer and adjust its PHY parameters as notified in the AAI_RNG-ACK message. If all the ranging statuses of the detected ranging preambles are equal to 'success', the AAI_RNG-ACK may be omitted. The AAI_RNG-ACK shall include the following parameters:

Table 676—Parameters for AAI_RNG-ACK

| Name | Value | Description |
| --- | --- | --- |
| Management Message Type | AAI_RNG-ACK |  |
| Ranging Request bit | If set to 1, ABS request AMS to perform periodic ranging by replying AAI_RNG-CFM | Shall be included when the AAI_RNG-ACK is sent with unicast STID for a specific user |
| RNG-ACK Bitmap | The size of this bitmap is decided by the number of ranging slots in the previous frame. Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. | Shall be included when the AAI_RNG-ACK is sent with broadcast STID |
| Number of received codes |  | The number of ranging code indices detected in this corresponding ranging opportunity. Shall be included for each bit=0b1 in the RNG-ACK Bitmap when the AAI_RNG-ACK is sent with broadcast STID |
| Code Index |  | Code index received in this ranging opportunity. Shall be included for each received ranging codes if Number of received codes > 0 when the AAI_RNG-ACK is sent with broadcast STID |
| Ranging status | Used to indicated whether UL messages are received within acceptable limits by the ABS 0b01: continue 0b11: abort 0b00: success | When the AAI_RNG-ACK is sent with broadcast STID, this parameter Sshall be included for each received ranging code if Number of received codes > 0. When the AAI_RNG-ACK is sent with unicast STID, the parameter is included for the specific user. |
| Adjustment parameters indication (API) | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | When the AAI_RNG-ACK is sent with broadcast STID, this parameter Sshall be included for each received ranging code when Ranging status is 0b01. When the AAI_RNG-ACK is sent with unicast STID, the parameter is included for the specific user when Ranging status is 0b01, and in this case Bit#2 is always set to 0. |
| Timing offset adjustment | 10 bits | Shall be included when Time offset adjustment indication bit is 1 |
| Power level adjustment | 3 bits | Shall be included when Power level adjustment indication bit is 1 |
| Frequency offset adjustment | 8 bits | Shall be included when Frequency offset adjustment indication bit is 1 |

------------------------------------------------------  Text End  ---------------------------------------------------------------