| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for AAI-E-MBS_CFG Transmission (16.9.3) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) | loa@iii.org.tw |
| Re: | In response to IEEE 802.16 Working Group Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes text for AAI-E-MBS_CFG Transmission |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for AAI-E-MBS_CFG Transmission (16.9.3)

Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai
Institute for Information Industry (III)

Introduction

The information regarding E-MBS configuration is transmitted periodically to the AMSs interested in E-MBS using a MAC management message called the AAI-E-MBS_CFG message. However, idle AMSs have to interpret each A-A-MAP to receive AAI_E-MBS-CFG or they may miss the E-MBS configure information. In order to facilitate E-MBS reception, AAI_E-MBS-CFG shall be advertised at a pre-determined location. The AMS can obtain frame offset as follows:

Frame_offset = 12 bits LSB of 48 bit ABS MAC ID modulo 4

The Transmission interval of AAI_E-MBS-CFG messages is a global value known by ABS and AMS.

Proposed text for the P80216m/D2:

-----------------------------------------------Start of the Text Proposal # 1---------------------------------------------

[Make the following changes to Section 16.9.3.1 in Page 643 beginning Line 36 in P802.16m/D3.]

#### 16.9.3.1 E-MBS Configuration Indicators

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode and MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI_E-MBS-CFG shall be advertised at a pre-determined frame offset in a superframe. The AMS can obtain the frame offset, i.e., Frame_offset, as follows:

Frame_offset = 12 bits LSB of 48 bit ABS MAC ID modulo 4

-----------------------------------------------End of the Text Proposal # 1---------------------------------------------

-----------------------------------------------Start of the Text Proposal # 2---------------------------------------------

[Add Section 16.x as follows]

#### 16.x Parameters and Constants

#### 16.x.1 Global Values

The ABS and AMS shall meet the requirements contained in Table xxx.

Table xxx— Global Values

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| ABS, AMS | AAI_E-MBS-CFG Interval | Transmission time between AAI_E-MBS-CFG messages | — | 10 s | 10 s |

-----------------------------------------------End of the Text Proposal # 2---------------------------------------------

[1] IEEE P80216m/D3, “P802.16m/D3: Advanced Air Interface”