| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CSGID transmission for snooping (16.4.8.1.2) |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Jin Lee, Kiseon Ryu, Ronny Kim and Jin Sam Kwak LG Electronics | {jins978, ksryu, ronnykim, samji }@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D3 for IEEE 802.16 WG Letter Ballot Recirc #30b |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D3 regarding CSGID transmission for snooping |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CSGID transmission for snooping (16.4.8.1.2)

Jin Lee, Kiseon Ryu, Ronny Kim and Jin Sam Kwak

LG Electronics

- Introduction

The contribution proposes the text changes to D3 [1] to describe how CSGID(s) sent by AMS can be used. Currently, CSGID(s) is sent for CSG membership detection while AMS performs network entry to Femto ABS. However, the CSGID(s) information can be also used for macro ABS to recommend CSG Femto ABS (belonging to the CSGID(s)) to detect its CSG AMS(s) by observing UL signaling.

Therefore, this contribution proposes CSGID(s) transmission during NE regardless of cell type.

- References

[1] IEEE P802.16m/D3

- Proposed Text Changes

-------------------------------------------------- Begin Proposed Text Change 1----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line7, page 41. New texts are underlined in blue font.>

16.2.3.1 AAI_RNG-REQ

CSGID(s) shall be included in AAI_RNG-REQ in order for macro ABS to recommend CSG Femto ABS to monitor UL signal from AMS served by the macro ABS. When CSGID(s) is sent to Femto ABS in AAI_RNG-REQ, the CSGID(s) is used for CSG membership detection. Further procedures are described in 16.4.7.5 and 16.4.8.1.2

-------------------------------------------------- End Proposed Text Change 1------------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 2----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 33, page 46. Proposed deletion of text has been shown with red font that has been struck through. New texts are underlined in blue font.>

Table 674 – Parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| …… | …… | …… |
| CSGID(s) | One or more identifiers for one or more CSG(s) | It shall be included when an AMS has one or more CSGID(s) Sent by AMS to aid quick CSG membership detection by Femto ABS as well to aid ABS reselection or to recommend CSG Femto ABS to monitor UL signal of the AMS by macro ABS |

-------------------------------------------------- End Proposed Text Change 2------------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 3----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 36, page 601. Proposed deletion of text has been shown with red font that has been struck through. New texts are underlined in blue font.>

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

When the AMS performs network entry with macro ABS, it shall include CSGID(s) in AAI_RNG-REQ, if one or more CSGID(s) is provisioned in the AMS. If the overlapped macro ABS receives the CSGID(s) from the AMS, Alternatively, it overlapped Macro ABS may recommend CSG Femto ABS to monitor UL signaling of accessible AMS which is served by the Macro ABS.

-------------------------------------------------- End Proposed Text Change 3------------------------------------------------