| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes to inter-RAT HO (16.2.3.37) |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Jin Lee, Ronny Kim, Kiseon Ryu and Jin Sam Kwak LG Electronics | {jins978, ronnykim, ksryu, samji }@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D3 for IEEE 802.16 WG Letter Ballot Recirc #30b |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D3 for inter-RAT HO (16.2.3.37) |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes to inter-RAT HO (16.2.3.37)

Jin Lee, Ronny Kim, Kiseon Ryu and Jin Sam Kwak

LG Electronics

- Introduction

The contribution proposes the text changes to D3 [1] in order to support consistency between section 16.2.6.5.2.1.2.1 (Active other RAT discovery with MIH support) and section 16.2.3.37 (MAC Message).

- References

[1] IEEE P802.16m/D3

- Proposed Text Changes

-------------------------------------------------- Begin Proposed Text Change 1----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 28, page 103. Proposed deletion of text has been shown with red font that has been struck through. New texts are in blue font.>

Table 709-PKM v3 message types

| Code | PKM message type | MAC control message name |
| --- | --- | --- |
| ….. | ……………… | …………………. |
| 9-16 | reservedMIH Initial Request | AAI_PKM-REQ |
| 10 | MIH Acknowledge | AAI_PKM-RSP |
| 11 | MIH Comeback Response | AAI_PKM-RSP |
| 12-16 | reserved |  |

-------------------------------------------------- End Proposed Text Change 1-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 2----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 22, page 109. New texts are in blue font.>

16.2.3.37.9 MIH Initial Request message

Code 9

Attributes are shown in Table xxx

Table xxx – MIH Initial Request attributes

| Attribute | Contents |
| --- | --- |
| MIH frame type | Indicates the type of MIHF frame |
| Delivery Method and Status Code | Indicates the delivery of query response |
| MIHF frame | The encapsulated MIHF query |

-------------------------------------------------- End Proposed Text Change 2-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 3----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 22, page 109. New texts are in blue font.>

16.2.3.37.10 MIH Acknowledge message

Code 10

Attributes are shown in Table xxx

Table xxx- MIH Acknowledge attributes

| Attribute | Contents |
| --- | --- |
| Cycle | Indicates frame number of the first frame where the MIH response is expected to be ready for transmittal to the AMS |
| Query ID | Used to map query and query response |
| Delivery Method and Status Code | Indicates the delivery method and status code |

-------------------------------------------------- End Proposed Text Change 3-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 4---------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 22, page 109. New texts are in blue font.>

16.2.3.37.11 MIH Comeback Response message

Code 11

Attributes are shown in Table xxx

Table xxx – MIH Comeback Response attributes

| Attribute | Contents |
| --- | --- |
| MIHF frame type | Indicates the type of the included MIHF frame. Only included when an MIHF frame is present |
| Query ID | Used to map query and query response |
| Delivery Method and Status Code | Indicate the delivery method and status code |
| MIHF frame | The encapsulated MIH response |

-------------------------------------------------- End Proposed Text Change 4-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 5----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D3, starting on line 15, page 117. New texts are in blue font.>

16.2.3.46 AAI_SII-ADV

Table 723 – Parameter for AAI_SII-ADV message

| Name | Value |
| --- | --- |
| …………. | ………….. |
| Query ID | Used to correlate the response encapsulated in the MIHF frame carried in the MIHF frame with a query previously sent by an AMS within the broadcast area of the ABS. It may be ignored by all other AMSs. |
| MIH Frame Type | The MIHF frame type indicates the service type of the MIHF frame |
| MIH Frame | The encapsulated MIH response |

-------------------------------------------------- End Proposed Text Change 5-----------------------------------------------