| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to AAI_HO-CMD Message for E-MBS Zone Transition (16.9.2.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Chun-Yuan Chiu, Chun-Yen Wang, Yan-Xiu Zheng, Frank Ren, ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | E-mail: ccy@itri.org.tw whsheen@itri.org.tw |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot#30b”, Target topic: “IEEE P802.16m/D3, section 16.9.2.2”. |  |
| Abstract | The contribution proposes changes to the AAI_HO-CMD message in P802.16m/D3 |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to AAI_HO-CMD Message for

E-MBS Zone Transition (16.9.2.2)

Chun-Yuan Chiu, Chun-Yen Wang, Yan-Xiu Zheng, Frank Ren

ITRI

Wern Ho Sheen

Chaoyang University of Technology / ITRI

- Introduction

Current P802.16m/D3 [1] specifies (in Section 16.9.2.2) when an AMS moves across E-MBS zone boundaries in Active Mode or Sleep Mode, the MSTID and FID information regarding the new E-MBS Zone may be updated through the AAI_HO-CMD message via the serving ABS. However, the corresponding table (Table 683) of the AAI_HO-CMD message has no according specification. This contribution proposes changes to AAI_HO-CMD message to be included in P802.16m/D3.

- References

- IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access systems; Advanced Air Interface,” December 2009

- Proposed Text

---------------------------------------------------- Start of Proposed Text----------------------------------------------------

[Adopt the following rows in blue into Table 683, section 16.2.3.11 on page 61-63:]

15.2.3.10 AAI_HO-CMD

The S-ABS shall send AAI_HO-CMD to initiate the HO procedure, or to acknowledge the AAI_HO-REQ sent by the AMS. The AAI_HO-CMD message shall include the following parameters:

Table—680 Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| Mode | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. |  |
| … | … | … |
| Target ABS/YBS ID |  | Shall be included for each target ABS when Mode=0b00 |
| E-MBS Update Indication | 1: E-MBS information regarding the new E-MBS Zone to which the target ABS belongs is included in this message 0: no E-MBS information | Shall be included for each target ABS when Mode = 0b00 |
| N_E-MBS_Flow | Number of E-MBS Flows which have update information | Shall be included when E-MBS Update Indication = 1 |
| E-MBS Update Information | E-MBS update information of each E-MBS flow, which includes: Original E-MBS Zone Identifier; Original Multicast STID; Original Flow ID; New E-MBS Zone Identifier; New Multicast STID; New Flow ID; Physical Carrier Index of the target ABS where the E-MBS flow is provided; | Shall be included when N_E-MBS_Flow > 0 |
| … | … | … |

---------------------------------------------------- End of Proposed Text ------------------------------------------------