| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Design of the broadcast assignment A-MAP IE for Updating Additional Broadcast Message (16.3.6) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Kiseon Ryu Jinsoo Choi and Yongho Kim LG Electronics Intel Corporation | heejeong.cho@lge.com, youngsoo.yuk@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b |  |
| Abstract | The contribution proposes text to specify the Broadcast A-MAP IE |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Design of the broadcast assignment A-MAP IE for Updating Additional Broadcast Message (16.3.6)

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Jinsoo Choi and Yongho Kim

LG Electronics

- Introduction

This contribution includes the proposed text for broadcast A-MAP IE in the IEEE 802.16m D3[6]. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2], the IEEE 802.16m AWD [5] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

- Modifications and key-descriptions

- The main concepts from “16.3.6.5.2.4.15 broadcast assignment A-MAP IE” in the IEEE P802.16m D3 are maintained in this text.

- We propose designs of the broadcast assignment A-MAP IE for updating additional broadcast message.

  - The resource & MCS indication method is changed.

    - The unicast A-MAP uses Isizeoffset and resource allocation index to indicate MCS and HARQ burst size.

    - However, Isizeoffset-based method cannot support lower MCS than QPSK 1/12. The easy way that we can use is to provide the HARQ burst size and the resource allocation size.

    - In our proposal, Broadcast A-MAP uses 6-bit HARQ Burst size & Resource index to indicate transmission format.

  - It includes the indicators for idle mode, sleep mode and handover. (It helps to reduce the decoding overhead for sleep, idle and handover users by avoiding the decoding of unnecessary burst)

  - It includes version information for SCD and NBR-ADV message.

  - By receiving version information, AMS can know in advance whether any values of SCD and NBR-ADV message are changed or not.

- [ Recommended AWD Text Proposal  (16.3.6.5.2.4.13, page 443, line 16) ]

# 16.3.6.5.2.4.13 broadcast assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast burst. All broadcast MAC control messages to be transmitted in a subframe shall be included into a broadcast burst. Table 829 describes the fields in a broadcast assignment A-MAP IE.

Table 829 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast-A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast A-MAP IE |
| HARQ Burst Size MCS (TBD) | 6 | HARQ Burst Size for this burst |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI | 1 |  |
| Message inclusion indicator for Idle mode | 1 | This field is set when this burst include the broadcast messages on idle mode support |
| Message inclusion indicator for sleep mode | 1 | This field is set when this burst include the broadcast messages on sleep mode support |
| Message inclusion indicator for mobility management | 1 | This field is set when this burst include the broadcast messages on handover support |
| Configuration Change Counts for AAI_SCD | 4 | This Configuration Change Count is incremented by one (modulo 16) by the ABS whenever any of the values of the AAI_SCD message change. |
| Configuration Change Counts for AAI_NBR-ADV | 4 | This Configuration Change Count is incremented by one (modulo 16) by the ABS whenever any of the values of the AAI_NBR-ADV message change. |
| Reserved | TBD7 | Reserved bits |
| } |  |  |

*A 16 bit CRC is generated based on the contents of the Broadcast A-MAP IE.

- The broadcast message signaled by the Broadcast A-MAP IE is always transmitted using SFBC as the MIMO encoder format.

-------------------------------------- End of Text Proposal -------------------------------------------