| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the AAI_LBS-ADV Message Format (16.8.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D3,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement, IEEE 802.16-09/0073 |  |
| Abstract | AAI_LBS-ADV Message Format on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the AAI_LBS-ADV Message Format (16.8.2)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

This contribution provides the clarification of AAI_LBS-ADV message format for the LBS in IEEE 802.16m Amendment Draft Standard [1].

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

## [Modify the Section 16.8.2.4.1 of 802.16m/D3 as follows.]

16.8.2.4.1 AAI_LBS-ADV Message (Ron: section header to be deleted)

15.2.3.41 AAI_LBS-ADV Message

An ABS that supports LBS shall use the AAI_LBS-ADV message to broadcast the LBS related configuration information. The message may be broadcast periodically without solicitation by the AMS.

Table 943—AAI_LBS-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| Number of BS | 8 | Number of neighbor BSs included in this message that are identified using the BSID |
| BSID | 16 | 16-bit LSB of unique 48-bit identifier of the Base Station ID parameter of the Serving BS or Neighbor BS. |
| Number of BS Index | 8 | Number of neighbor ABSs included in this message that are identified using an index to their position in the NBR-ADV message |
| Configuration change count for AAI_NBR-ADV | 8 | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID |
| Neighbor BS Index | 8 | Index that corresponds to the position of the ABS in the NBR-ADV message. |
| Absolute Position (Long Format) | 120 | The transmitting ABS's coordinates. This Absolute Position (Long Format) shall only be used for the transmitting ABS and if the Absolute Position (Short Format) is not used. Including parameter (Longitude, Latitude, Altitude) |
| Absolute Position (Short Format) | 64 | Shall be included when absolute position indication is set to 0b1. The transmitting ABS's coordinates. This Absolute Position (Short Format) shall only be used for the transmitting ABS and if the Absolute Position (Long Format) is not used. Including parameter (Longitude, Latitude, Altitude) |
| Relative Position | 48 | (note from editor: something is wrong in this sentence that should be fixed) This Relative Position is used to provide the absolute position of an ABS. A Neighbor ABS's coordinates. When this Relative Position is included it provides the position of a neighbor ABS relatively to the transmitting ABS. Including parameter (Longitude, Latitude, Altitude) |
| GPS Time | 40 | Information about GPS time and time accuracy. Including parameter(GPS time, GPS frame transmission offset, GPS time accuracy) |
| Frequency Accuracy | 8 | Information about the frequency accuracy |

| Name |  | Value | Usage |
| --- | --- | --- | --- |
| Control Message Type |  | AAI_LBS-ADV |  |
| Serving ABS ID |  | 16-bit LSB of unique 48-bit identifier of the Base Station ID parameter of the Serving. |  |
| Position Indication |  | Represents absolute position whether using long format or short format. The value indicates: 0b0: long format is used 0b1: short format is used |  |
| Absolute Position (Long Format) | Longitude | Bit 0-5: longitude resolution 1-34 - # of valid bits in fixed-point value of longitude value 35 – LBS not supported Other – Reserved Bit 6-14: longitude integer Bit 15-39: longitude fraction | Shall be included when Position Indication is set to 0b0. The transmitting ABS's coordinates. This Absolute Position (Long Format) shall only be used for the transmitting ABS and includes Longitude, Latitude, and Altitude. |
|  | Latitude | Bit 0-5: latitude resolution 1-34 - # of valid bits in fixed-point value of latitude value 35 – LBS not supported Other – Reserved Bit 6-14: latitude integer Bit 15-39: latitude fraction |  |
|  | Altitude | Bit 0-3: altitude type - meter - floors Other – Reserved Bit 4-9: altitude resolution 1-30 - # of valid bits in fixed-point value of altitude value 31 – LBS not supported Other – Reserved Bit 10-31: altitude integer Bit 32-39: altitude fraction |  |
| Absolute Position (Short Format) | Longitude | Longitude expressed in 2-15 parts of a degree | Shall be included when Position Indication is set to 0b1. The transmitting ABS's coordinates. This Absolute Position (Short Format) shall only be used for the transmitting ABS and includes Longitude, Latitude, and Altitude. |
|  | Latitude | Latitude expressed in 2-16 parts of a degree |  |
|  | Altitude | Bit 0-15: altitude in meters above sea level |  |
| GPS Time | GPS time in units of frame duration | 22bits | Information about GPS time and time accuracy. |
|  | GPS frame transmission time offset | 10bits |  |
|  | GPS time accuracy | 5bits |  |
| Frequency Accuracy |  | 8bits | Information about the frequency accuracy |
| Number of BS |  | 8bits | Number of neighbor BSs included in this message that are identified using the BSID |
| BSID |  | 16bits | Shall be included when Number of BS > 0 16-bit LSB of unique 48-bit identifier of the Neighbor BS for number of BS. |
| Number of BS Indices |  | 8bits | Number of neighbor BSs included in this message that are identified using an index to their position in the AAI_NBR-ADV message |
| Configuration change count for AAI_NBR_ADV |  | 8bits | Shall be included when Number of BS Indices > 0 |
| Neighbor BS Index |  | 8bits | Shall be included when Number of BS Indices > 0 Index that corresponds to the position of the BS in the NBR-ADV message |
| Relative Position | Longitude | Distance east of reference point in meters | Shall be included when Number of BS > 0 or when Number of BS Indices>0 This Relative Position is used to provide the absolute position of an ABS. When this Relative Position is included, it provides the position of a neighbor ABS relatively to the transmitting ABS. |
|  | Latitude | Distance north of reference point in meters |  |
|  | Altitude | Distance above of reference point in meters |  |

## [Add the following row to the Table 673 in the Section 16.2.3 of 802.16m/D3.]

| AAI_LBS-ADV | LBS Advertisement | Null | Broadcast |
| --- | --- | --- | --- |

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” December 2009