| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS Configuration Indicators (16.9.3) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI Kaushik Josiam, Taeyoung Kim, Hai Wang, Jaejeong Shim Samsung Electronics Kaiying Lv, Nan Li, Bo Sun ZTE Corporation | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr brian.shim@samsung.com lv.kaiying@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D3,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement, IEEE 802.16-09/0073 |  |
| Abstract | E-MBS Configuration Indicators on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS Configuration Indicators (16.9.3)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

Kaushik Josiam, Taeyoung Kim, Hai Wang, Jaejeong Shim

Samsung Electronics

Kaiying Lv, Nan Li, Bo Sun

ZTE Corporation

# Instructions

This contribution provides configuration indicators supporting E-MBS in IEEE 802.16m Amendment Draft Standard [1]. The configuration indicator is transmitted to AMS as a form of AAI_E-MBS-CFG MAC Control Message. For the efficient E-MBS service, it carries the E-MBS related information of neighbor Zones as the form of list mapping of current MSTID + FID and new MSTID + FID. It could be helpful information to update service flow during MBS HO. In addition to E-MBS-CFG message, to update the service flow during HO or MBS location update, AAI_RNG-RSP message includes the Service Flow Update Bitmap and E-MBS Service Flow Encoding corresponding to the bitmap.

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

## [Add the following row to the Table 673 in the Section 16.2.3 of 802.16m/D3.]

| AAI_E-MBS-CFG | E-MBS Configuration | Null | Broadcast |
| --- | --- | --- | --- |

[Adopt the following rows into Table 675-parameters for AAI_RNG-RSP in 16.2.3.2 of 802.16m/D3]

| Name |  | Value | Usage |
| --- | --- | --- | --- |
| Service Flow Update Bitmap |  | MBS Service Flow update bitmap where the MSTID+FID is sorted with increasing order between ABS and AMS. | It may be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and network re-entry for HO procedure if it needs to update. |
| E-MBS Service Flow Encodings | E-MBS Zone ID | New mapping(s) of E-MBS Zone ID, MSTID + FID, and Physical Carrier Index which are used in E-MBS Zone supported by the target ABS and corresponding to the Service Flow Update Bitmap. |  |
|  | MSTID + FID |  |  |
|  | Physical Carrier Index |  |  |

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” December 2009