| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS Control Information Elements (16.3.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D3,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement, IEEE 802.16-09/0073 |  |
| Abstract | E-MBS Control Information Elements on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS Control Information Elements (16.3.6)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

This contribution provides Control Information Elements supporting E-MBS in IEEE 802.16m Amendment Draft Standard [1]. The E-MBS Control Information Elements describe the resource mapping along with the mapping multicast STID and FID per E-MBS Zone. It should describe burst information clearly including time offset, resource index, allocation period, and etc.

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

## [Modify the 16.3.6.5.3 of 802.16m/D3 as follows.]

16.3.6.5.3 E-MBS Control Information Elements

The resource mapping for E-MBS allows are defined in E-MBS-DATA_IE's as defined in Table 830.

Table 830—E-MBS DATA Information Elements

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS_DATA_IE () { |  |  |
| No. of Multicast STIDs+FIDs | 8 | Total number of E-MBS streams in the IE |
| For (i=0; i< No. of Multicast STID +FID’s, i++){ |  |  |
| MSTID+FID | 16 | Multicast STID + Flow ID of an E-MBS stream |
| Isize-offset | 5 | Depends on supported modes, 32modes assumed as baseline |
| MEF | 1 | E-MBS MIMO Encoding Format 0b0: SFBC 0b1: Vertical Encoding |
| If (MEF == 0b1){ |  | Parameters of Vertical Encoding |
| Mt | 1 | Number of streams for transmission for Nt =2, Nt =4, and Nt =8 (Mt <= Nt) 0b0: 1 stream 0b1: 2 streams |
| } |  |  |
| E-MBS Frame/AAI subframe offset | [TBD] | Includes the location of the frame/subframe where the E-MBS data burst begins |
| E-MBS superframe offset | m | m LSB of Superframe number, where m is the value of MSI Length(m). According to the MSI length , m = 1 if MSI Length is 2 superframes m = 2 if MSI Length is 4 superframes m = 3 if MSI Length is 8 superframes m = 4 if MSI Length is 16 superframes |
| E-MBS frame index | 2 | Frame number in the superframe 0b00 : 1st frame 0b01: 2nd frame 0b10: 3rd frame 0b11: 4th frame |
| E-MBS subframe index | 3 | Subframe offset in the frame 0b000: 1st subframe 0b001: 2nd subframe … 0b111: 8th subframe |
| # of subframe of E-MBS | 3 | # of subframes, where E-MBS data burst may span one or more subframes |
| E-MBS Resource Indexing | [TBD]11 | Includes the location of the subcarrier index where the E-MBS data burst begins |
| Allocation Period | [TBD]m+2 | Inter-arrival intervals of E-MBS data bursts for an E-MBS stream (in number of frames), where E-MBS subframe index and # of subframe of E-MBS are the same as the first information of E-MBS data burst |
| } | - | - |
| } |  |  |

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” December 2009