| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS MAP Structure (16.3.6) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI Kaushik Josiam, Taeyoung Kim, Hai Wang, Jaejeong Shim Samsung Electronics Kaiying Lv, Nan Li, Bo Sun ZTE Corporation Chun-Yuan Chiu, Chun-Yen Wang, Yan-Xiu Zheng, Frank Ren ITRI | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr kjosiam@sta.samsung.com lv.kaiying@zte.com.cn ccy@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D3,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement, IEEE 802.16-09/0073 |  |
| Abstract | E-MBS MAP Structure on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS MAP Structure (16.3.6)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

Kaushik Josiam, Taeyoung Kim, Hai Wang, Jaejeong Shim

Samsung Electronics

- Kaiying Lv，Nan Li，Bo Sun

ZTE Corporation

Chun-Yuan Chiu, Chun-Yen Wang, Yan-Xiu Zheng, Frank Ren

ITRI

# Instructions

This contribution provides clarification on the E-MBS MAP in IEEE 802.16m Amendment Draft Standard [1]. The E-MBS MAP is pointed by the AAI_E-MBS-CFG MAC Control Message. The E-MBS MAP can help AMS skip to decode AAI_E-MBS-CFG MAC Control Message according to the AAI_E-MBS-CFG Change indication.

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

## [Modify the 16.3.6.2.3 of 802.16m/D3 as follows.]

16.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries some configuration information for enhanced multicast broadcast service for one EMBS Zone. The E-MBS MAP is pointed by the AAI_E-MBS-CFG MAC Control Message described in 15.9.3.1and is transmitted in the first several resources units of the E-MBS region in the beginning of the MSI. The parameters of the E-MBS region and the burst size of the E-MBS MAP is transmitted in the AAI_E-MBS-CFG MAC control message described in 16.9.3.1. AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance.

Table 798—: E-MBS MAP Structure

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { |  |  |
| AAI_E-MBS-CFG Change Indication | 1 | AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance. If it is set (AAI_E-MBS-CFG Change Indication = 1), parameters in the AAI_E-MBS-CFG message will change and AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS resets the Allocation Lifetime as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission. |
| E-MBS_DATA_IE() |  | Refer to Table 830 |
| } | - | - |
| Padding | Variable | Padding to reach byte boundary |
| } |  |  |

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” December 2009