| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS Service Flow Update in IEEE P802.16m (16.2.3) |  |
| Date Submitted | 2010-01-12 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA Chun-Yuan Chiu, Chun-Yen Wang, Yan-Xiu Zheng, Frank Ren, ITRI | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr ccy@itri.org.tw |
| Re: | “P802.16m/D3,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement, IEEE 802.16-09/0073 |  |
| Abstract | Service flow update to support E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# E-MBS Service Flow Update in IEEE 802.16m (16.2.3)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

Chun-Yuan Chiu, Chun-Yen Wang, Yan-Xiu Zheng, Frank Ren

ITRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

The AMS may continue to receive E-MBS transmissions from any ABS that is part of the E-MBS Zone, regardless of the AMS operating state—Connected State, Idle State.

When an AMS moves across the E-MBS zone boundaries and an ABS in another E-MBS zone is transmitting the same E-MBS flows, it can continue to receive E-MBS data from the ABS in Connected State or Idle State.

According to the IEEE 802.16m Amendment Draft Standard [1], when an AMS moves across the E-MBS boundaries, it can continue to receive E-MBS data from the ABS regardless of its operating state. In Connected State, the AMS performs handover procedure for E-MBS. In Idle State, the AMS may transit to Connected State to perform handover or it may initiate E-MBS location update process for the purpose of E-MBS zone transition unless the AMS already has the MSTID and FID mappings in the target E-MBS zone.

However, the current HO procedure and location update procedure are not sufficient for seamless transition from one E-MBS zone to another without any interruption of E-MBS data service and operation.

Therefore, in this contribution, some MAC control messages (i.e., AAI_HO-CMD and AAI_RNG-RSP) are modified to support the E-MBS HO and E-MBS location update.

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Add the following rows into Table 683-parameters for AAI_HO-CMD message in 16.2.3.11 of 802.16m/D3]

| Name | Value | Usage |
| --- | --- | --- |
| Service Flow Update Bitmap | A bitmap to indicator which E-MBS service flow has a new E-MBS Service Flow Management Encoding included in this message, where service flows are sorted according to the MSTID+FID with increasing order. | It shall be included only if the Mode = 0b00 for each target ABS. |
| E-MBS Service Flow Management Encodings | New MSTID and FID, Packet Classification Rule parameter(s), and E-MBS Zone Identifier Assignment parameter which are used in E-MBS Zone supported by the target ABS. | Shall be included when the corresponding bit in Service Flow Update Bitmap = 1 |
| Physical Carrier Index | Physical Carrier Index on which the E-MBS flow is carried. |  |

[Adopt the following rows into Table 675-parameters for AAI_RNG-RSP in 16.2.3.2 of 802.16m/D3]

| Name | Value | Usage |
| --- | --- | --- |
| E-MBS Zone Identifier | The new E-MBS Zone Identifier which the AMS shall maintain in any state on being served MBS. | It may be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and network re-entry for HO procedure if it needs to update. |
| Service Flow Update Bitmap | MBS Service Flow update bitmap where the MSTID+FID is sorted with increasing order between ABS and AMS. |  |
| E-MBS Service Flow Encodings | New mapping(s) of MSTID + FID which are used in E-MBS Zone supported by the target ABS and corresponding to the Service Flow Update Bitmap. |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” December 2009