| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_MC-ADV message format for IEEE 802.16m (section 16.2.8.2.3.1.1) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1856 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #30b |  |
| Abstract | Text proposal for 16.2.8.2.3.1.1 AAI_MC-ADV section |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_MC-ADV message format for IEEE 802.16m (section 16.2.8.2.3.1.1)

Inuk Jung, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to provide message format of AAI_MC-ADV message.

- References

[1] IEEE P802.16m/D3, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

On the 80216m_D2, page 186, line 1, add current text in 16.2.8.2.3.1.1 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.8.2.3.1.1 AAI_MC-ADV Message

Table 735—AAI_MC-ADV MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_MC-ADV Message format(){ |  |  |
| Management Message Type = NN | 8 | — |
| Multi-carrier configuration change count | 4 | A change count associated with a particular configuration for all possible carriers in the network. Incremented by 1 upon each update. |
| MC configuration broadcast flag | 1 | 0: only broadcast serving BS specific info 1: broadcast multi-carrier definition fro all available carriers in the network. |
| If (MC configuration broadcast flag == 1) { |  |  |
| Multicarrier configuration mode (MCM) | 2 | 0b00: Single carrier mode 0b01: One Carrier group 0b10: Multiple Carrier groups 0b11: Reserved *Note: If 0b00, NCG and NC is implicitly set to 1. If 0b01, NCG is implicitly set to 1. |
| If (MCM==10) { |  |  |
| Number of Carrier Groups (NCG) | 4 | Groups of contiguous and aligned carriers of serving ABS |
| } |  |  |
| If (MCM==01 ¦¦ MCM==10) { |  |  |
| Number of carriers (NC) | 6 | Number of carriers of the ABS of serving ABS |
| } |  |  |
| Physical Carrier Index of Current Carrier | 6 | The carrier that ABS broadcast this message |
| Uniformity Flag | 1 | 0: All Carriers have the same Protocol Version, Carrier Type Duplexing Mode and Transmit Power. 1: otherwise |
| For (i=1; i<= Number of Carrier Groups; i++) { |  |  |
| Carrier Configuration Index | 6 | Index associated to Table 648 |
| Start Frequency Assignment Index | 6 | Frequency Assignment Index of the first carrier in carrier group #i |
| For (j=1; j <= Number of Carriers; j++) { |  |  |
| Carrier Bandwidth | 3 | Bandwidth size of the carrier 0b000: 5MHz 0b001: 10MHz 0b010:20MHz 0b011: 7MHz 0b100: 8.75MHz 0b101~0b111: Reserved |
| SA-Preamble power de-boosting level | 6 | Values indicate power back-off levels in 0.1 dB steps starting from 0 dB |
| Physical Carrier SA-PREAMBLE Index | TBD 6 |  |
| If (Uniformity Flag == 1) { |  |  |
| Carrier Type | 1 | “0” for fully configured “1” for partially configured |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Transmit Power | 6 | A-Preamble transmit power level |
| } |  |  |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) 0b10: no SFH included 0b11: reserved For macrocell ABS, the bitmap shall be either 0b00 or 0b01 |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 1111 |
| If( SFH_encoding_format =00) { | — | //encoding format type-1 |
| If( Control_bitmap[0] ==1 ){ | — | — |
| SFH Subpkt 1 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| If( Control_bitmap[1] ==1 ){ | — | — |
| SFH Subpkt 2 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| If( Control_bitmap[2] ==1 ){ | — | — |
| SFH Subpkt 3 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| } | — | — |
| If( SFH_encoding_format =01) { | — | — |
| Delta information | — | Delta encoding, w.r.t. the reference BS |
| } |  |  |
| Serving BS Carrier Number | 6 | //start broadcasting serving BS specific info |
| For (i=0; i<Serving BS Carrier Number-1; i++){ |  |  |
| Physical Carrier Index | 6 |  |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| For (i=0; i<P; i++) { |  |  |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m MAC version. |
| Preamble Index | 6 |  |
| } |  |  |
| } |  |  |

--------------------------------------------------------Text End -----------------------------------------------------